KRW 2005 Annual load estimation

krw05.flo <- read.flo("krw",05)

krw05.lab <- read.lab("krw",05)

krw05.sed <- merge.flo("krw",05)

nonstorms <- drop.storms("krw",05)

dim(nonstorms)

[1] 38455     4

head(nonstorms, 5)

chr     q turb turbcode

1 (09/26/04 11:00:00) 5.472   NA        2

2 (09/26/04 11:10:00) 5.472   NA        2

3 (09/26/04 11:20:00) 5.472   NA        2

4 (09/26/04 11:30:00) 5.459   NA        2

5 (09/26/04 11:40:00) 5.459   NA        2

> table(nonstorms$turbcode)

0     2     5     6

24424  8832  1735  3464

8832 values of bad turbidity out of 38455 intervals to be estimated

trb05.flo <- nonstorms[!is.na(nonstorms$turb), ]

flo05.flo <- nonstorms[is.na(nonstorms$turb), ]

dim(trb05.flo)

[1] 29623     4

dim(flo05.flo)

[1] 8832    4

29623+8832

[1] 38455

# Develop model for good turb data

range(trb05.flo$turb)

[1]   1 439

table(krw05.sed$labcode2)

0

180

# No bad lab codes to worry about

table(krw05.sed$labcode1)

0   7

173   7

# dump 12 bots 12-18 are suspect: ISCO arm was hanging up

# Six of seven were outliers on scatterplots.

# Eliminate bad turbidity and tracking code 7

trb05.sed <- krw05.sed[!is.na(krw05.sed$turb) & krw05.sed$turb < 2000 & krw05.sed$labcode1 != 7, ]

turbsscplot("trb",05)

# Also eliminate the two egregious outliers

trb05.sed[trb05.sed$ssc > 1200 & trb05.sed$turb < 300,]

chr dump bottle  ssc turb turbcode     q labcode1 labcode2

81 (12/29/04 21:00:00)    9      8 1365   62        0 136.7        0        0

98 (01/01/05 04:10:00)   11     10 4067  217        0 507.4        0        0

trb05.sed <- trb05.sed[!(trb05.sed$ssc > 1200 & trb05.sed$turb < 300),]

table(trb05.sed$turbcode)

0   5

267   4

turbsscplot("trb",05,type="power")

turbsscplot("trb",05,type="logxy")

# Try a few different models for comparison

turb1 <- turbsrc("trb",05,040801,0000,050731,2400,type="power",var=F)

turb2 <- turbsrc("trb",05,040801,0000,050731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",05,040801,0000,050731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",05,040801,0000,050731,2400,type="linear",var=F)

start            end       type surr      yhat   n    r2      s cv

turb1 10/18/04 19:40 07/19/05 15:20      power turb 1100237.2 168 0.934 71.523 NA

turb2 10/18/04 19:40 07/19/05 15:20 logxy:mvue turb 1141840.8 168 0.888  0.390 NA

turb3 10/18/04 19:40 07/19/05 15:20      loess turb  684651.9 168 0.934 71.960 NA

turb4 10/18/04 19:40 07/19/05 15:20     linear turb  828056.5 168 0.934 71.921 NA

> sum(turb3$predssc < 0)

[1] 19537

> sum(turb4$predssc < 0)

[1] 20161

# Can't use linear or loess: too many negative predictions

# Tough to choose between the power model or logxy model, but power model has the higher r-square

start            end  type surr    yhat   n    r2      s cv

turb1 10/18/04 19:40 07/19/05 15:20 power turb 1058766 168 0.934 71.523 NA

Total load = 1058766 kg

# A relatively good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo05.flo$q)

[1]   4.599 235.900

> range(krw05.sed$q)

[1]   13.44 1076.00

# Eliminate high flow data that could tweak regression and tracking code 7s

flo05.sed <- krw05.sed[krw05.sed$q < 260 & krw05.sed$labcode1 != 7, ]

qsscplot("flo",05)

# Eliminate outlier bottle 8

min(flo05.sed$q)

[1] 13.44

flo05.sed <- flo05.sed[flo05.sed$q > 13.5, ]

qsscplot("flo",05)

flow1 <- flowsrc("flo",05,040801,0000,050731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",05,040801,0000,050731,2400,type="power",var=F)

flow3 <- flowsrc("flo",05,040801,0000,050731,2400,type="loess",var=F)

flow4 <- flowsrc("flo",05,040801,0000,050731,2400,type="loess",degree=2,var=F)

total(flow1,flow2,flow3,flow4)

start            end       type surr     yhat  n    r2       s cv

flow1 09/26/04 11:00 12/29/04 14:40 logxy:mvue flow 141316.9 93 0.399   0.774 NA

flow2 09/26/04 11:00 12/29/04 14:40      power flow 142987.9 93 0.223 105.346 NA

flow3 09/26/04 11:00 12/29/04 14:40      loess flow 150428.2 93 0.225 105.610 NA

flow4 09/26/04 11:00 12/29/04 14:40      loess flow 127314.4 93 0.236 105.558 NA

> sum(flow3$predssc <0)

[1] 0

> sum(flow4$predssc <0)

[1] 7729

#logxy looks like the best model to me, highest r2 by far

start            end       type surr     yhat  n    r2     s cv

flow1 09/26/04 11:00 12/29/04 14:40 logxy:mvue flow 141316.9 93 0.399 0.774 NA

Total load = 141317 kg

==================================================

# Total non-storm load

1058766 + 141317 = 1200083 kg = 1201 metric tons

# Total storm load = 5923567 kg

# Total annual load

5924 + 1201 = 7125 metric tons total annual load

# Storm load is 5924/7125 = 83.2% of annual load