Event loads were originally computed by Sarah Wilson

Rating equation: (s<0.7865)*(0) + (s>0.7865)*(25.328*s^2 + 18.389*s -30)

That was Clark's rating based on only HY03 data.

Here the loads are recomputed with Jack's HY03-07 rating

(s<1.8365)  Q = 5.013*s^3.770

(s>1.8365)  Q = 6.911*s^2 + 172.61*s - 290.70

NOTE: Events 301 to 304 had previously been estimated without knowledge of the 88 timed samples.  The previous "poor" estimates have now been replaced with better ones.

======================================================================

Storm 301: 021214,0110,021215,1510   359605 kg  Excellent

Storm 302: 021215,1520,021217,0330  1746058 kg  Very good

Storm 303: 021219,0800,021224,1000   264058 kg  Fair

Storm 304: 021227,0400,021230,0610  4662140 kg  Fair

Storm 305: 021230,0620,030103,0000  1169692 kg  Excellent

Storm 306: 030113,1050,030115,1800   118525 kg  Excellent

Storm 307: 030219,0110,030221,1800   800885 kg  Excellent

Storm 308: 030313,0800,030317,1200   132867 kg  Excellent

Storm 309: 030325,1740,030330,1200   388878 kg  Very good

Storm 310: 030412,0100,030416,0140    44791 kg  Good

Storm 311: 030423,1900,030425,0150   102853 kg  Very good

Storm 312: 030425,0200,030427,1640   395766 kg  Very good

Storm 313: 030429,1140,030502,0000   110281 kg  Fair

Total of event loads               10296399 kg

======================================================================

ftr03.flo <- read.flo("ftr",03)

ftr03.lab <- read.lab("ftr",03)

mismatches("ftr",03)

# Bottle 5:1-2, 15:5-6, 16:2-3 and 28:1 are missing

ftr03.sed <- merge.flo("ftr",03)

# Storm 301: 021214,0110,021215,1510

----------------------------------------------

#dd7-8 excellent coverage timed samples every 2 hrs

#turb data quality excellent

four()

turbsscplot("ftr",03,021214,0110,021215,1510)

turbsscplot("ftr",03,021214,0110,021215,1510,type="logxy")

turbsscplot("ftr",03,021214,0110,021215,1510,type="sqrt")

turbsscplot("ftr",03,021214,0110,021215,1510,type="power")

----------------------------------------------

turb1 <- turbsrc("ftr",03,021214,0110,021215,1510,type="linear")

turb2 <- turbsrc("ftr",03,021214,0110,021215,1510,type="power")

turb3 <- turbsrc("ftr",03,021214,0110,021215,1510,type="sqrt")

turb4 <- turbsrc("ftr",03,021214,0110,021215,1510,type="logxy")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 12/14/02 01:10 12/15/02 15:10     linear turb 361970.0 19 0.992 23.729 1.617

turb2 12/14/02 01:10 12/15/02 15:10      power turb 358724.6 19 0.995 18.309 1.271

turb3 12/14/02 01:10 12/15/02 15:10  sqrt:duan turb 359605.0 19 0.994  0.543    NA

turb4 12/14/02 01:10 12/15/02 15:10 logxy:mvue turb 380467.1 19 0.984  0.115 4.738

# The best 3 estimates agree within 1%

# The power fit overestimates the recession very slightly but nails the peak

# The sqrt model looks even better

ttsplot("ftr",03,turb2)

ttsplot("ftr",03,turb3)

ttsplot("ftr",03,turb3)

total(turb3)

start            end      type surr   yhat  n    r2     s cv

turb3 12/14/02 01:10 12/15/02 15:10 sqrt:duan turb 359605 19 0.994 0.543 NA

Total load = 359605 kg  Excellent

# Storm 302: 021215,1520,021217,0330

----------------------------------------------

#dump 8 Excellent sample coverage with timed samples every 2 hrs

#turb and stage data quality very good

----------------------------------------------

four()

turbsscplot("ftr",03,021215,1520,021217,0330)

turbsscplot("ftr",03,021215,1520,021217,0330,type="power")

turbsscplot("ftr",03,021215,1520,021217,0330,type="logxy")

turbsscplot("ftr",03,021215,1520,021217,0330,type="sqrt")

turb1 <- turbsrc("ftr",03,021215,1520,021217,0330)

turb2 <- turbsrc("ftr",03,021215,1520,021217,0330,type="power")

turb3 <- turbsrc("ftr",03,021215,1520,021217,0330,type="logxy")

turb4 <- turbsrc("ftr",03,021215,1520,021217,0330,type="sqrt")

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2       s    cv

turb1 12/15/02 15:20 12/17/02 03:30     linear turb 1774728 18 0.944 150.687 4.125

turb2 12/15/02 15:20 12/17/02 03:30      power turb 1746058 18 0.989  69.754 1.947

turb3 12/15/02 15:20 12/17/02 03:30 logxy:mvue turb 1718545 18 0.996   0.060 2.007

turb4 12/15/02 15:20 12/17/02 03:30  sqrt:duan turb 1722977 18 0.978   1.616    NA

# All estimates agree withing 3%, Best models are power and logxy

ttsplot("ftr",03,turb2)

start            end  type surr    yhat  n    r2      s    cv

turb2 12/15/02 15:20 12/17/02 03:30 power turb 1746058 18 0.989 69.754 1.947

Total load = 1746058 kg,  CV = 1.95 %  Very good to excellent

# Would be excellent if not for sample 6

# Storm 303: 021219,0800,021224,1000

------------------------------------

# Dumps 9 - 11

# turb data and flow data missing for significant period of time

# Hydrograph was reconstructed from SFM in 3 pieces to match the 4 ORs

# ISCO collected timed samples for most of this period

# Sample coverage is good enough to use lineartime

qsscplot("ftr",03,021219,0800,021224,1000)

time1 <- lineartime("ftr",03,021219,0800,021224,1000,ssc1=40, ssc2=16.75)

ttsplot("ftr",03,time1)

# This slightly overestimates the period of falling stage right after bottle 7

# The major uncertainty is the reconstruction of nearly the entire hydrograph

total(time1)

start            end   type surr     yhat  n r2  s cv

time1 12/19/02 08:00 12/24/02 10:00 linear time 264057.5 26 NA NA NA

Total load = 264058 kg   fair to good

Storm 304: 021227,0400,021230,0610

----------------------------------------------

#dumps 11-13

#Double-peaked event with no samples on first peak

#Need to include samples from dump 8 to cover appropriate turbidity range

#Second peak was on timed sampling until TTS started sampling late on recession

#turb data missing on recession of first peak: 021227,2130 to 021228, 0930

#stage data look fine

four()

turbsscplot("ftr",03,021227,0400,021230,0610)

turbsscplot("ftr",03,021227,0400,021230,0610,dump=8:13)

turbsscplot("ftr",03,021227,0400,021230,0610,dump=8:13,type="power")

turbsscplot("ftr",03,021227,0400,021230,0610,dump=8:13,type="loess",degree=2)

# Also need a flow model for the period of missing turbidity

# And this will have to extrapolated quite a bit at the upper end

four()

qsscplot("ftr",03,021227,0400,021230,0610,dump=13)

qsscplot("ftr",03,021227,0400,021230,0610,dump=13,type="linear")

qsscplot("ftr",03,021227,0400,021230,0610,dump=13,type="loess")

qsscplot("ftr",03,dump=13,bot=14:19)

----------------------------------------------

turb1 <- turbsrc("ftr",03,021227,0400,021227,2130,dump=8:13,type="loess",degree=2)

# The missing turbidity data is on falling limb, so use falling flow samples

flow1 <- flowsrc("ftr",03,021227,2140,021228,0920,dump=13,bot=14:19)

turb2 <- turbsrc("ftr",03,021228,0930,021230,0610,dump=8:13,type="loess",degree=2)

total(turb1,flow1,turb2)

start            end       type surr    yhat  n    r2      s    cv

turb1 12/27/02 04:00 12/27/02 21:30      loess turb 2100435 65 0.986 58.634    NA

flow1 12/27/02 21:40 12/28/02 09:20 logxy:mvue flow 1737118  6 0.997  0.046 4.117

turb2 12/28/02 09:30 12/30/02 06:10      loess turb 1201749 65 0.986 58.634    NA

Total load = 5039301 kg

ttsplot("ftr",03,turb1,flow1,turb2)

# See how sensitive this is to the tweak at the top for dump 8 bottle 7

turb1a <- turbsrc("ftr",03,021227,0400,021227,2130,dump=8:13,type="linear")

# The missing turbidity data is on falling limb, so use falling flow samples

flow1 <- flowsrc("ftr",03,021227,2140,021228,0920,dump=13,bot=14:19)

turb2a <- turbsrc("ftr",03,021228,0930,021230,0610,dump=8:13,type="linear")

total(turb1a,flow1,turb2a)

start            end       type surr    yhat  n    r2       s    cv

turb1a 12/27/02 04:00 12/27/02 21:30     linear turb 1690773 65 0.944 113.671 2.340

flow1  12/27/02 21:40 12/28/02 09:20 logxy:mvue flow 1737118  6 0.997   0.046 4.117

turb2a 12/28/02 09:30 12/30/02 06:10     linear turb 1249100 65 0.944 113.671 2.246

Total load = 4676991 kg,  CV = 1.85 %

# This is only 7.7% lower so we can't call this a very good fit

# We really don't know how high the peak concentration was

ttsplot("ftr",03,turb1a,flow1,turb2a)

# Can't use that last model though because it its negative at start of rise

# Because dump 8 bottle 7 was something of an outlier, let's discount it here

turb1b <- turbsrc("ftr",03,021227,0400,021227,2130,dump=13,type="power")

ttsplot("ftr",03,turb1b,flow1,turb2a)

total(turb1b,flow1,turb2a)

start            end       type surr    yhat  n    r2       s    cv

turb1b 12/27/02 04:00 12/27/02 21:30      power turb 1675923 19 0.979  56.743 4.398

flow1  12/27/02 21:40 12/28/02 09:20 logxy:mvue flow 1737118  6 0.997   0.046 4.117

turb2a 12/28/02 09:30 12/30/02 06:10     linear turb 1249100 65 0.944 113.671 2.246

Total load = 4662140 kg,  CV = 2.28 %  fair to good

# This one is 8% lower than the one that respects sample 7 from dump 8

----------------------------------------------

# Storm 305: 021230,0620,030103,0000

#grade excellent: good bottle coverage, turb data not adjusted, linear  turb:ssc relationship with cv 3.02% and r2 0.974

----------------------------------------------

#13:4-19

#turb data quality fine

four()

turbsscplot("ftr",03,021230,0620,030103,0000)

turbsscplot("ftr",03,021230,0620,030103,0000,type="logxy")

turbsscplot("ftr",03,021230,0620,030103,0000,type="sqrt")

turbsscplot("ftr",03,021230,0620,030103,0000,type="power")

----------------------------------------------

#final object

turb305 <- turbsrc("ftr",03,021230,0620,030103,0000)

ttsplot("ftr",03,turb305)

total(turb305)

start     end   type surr    yhat  n    r2      s    cv

turb305 12/30/02 06:20 1/03/03 linear turb 1169692 16 0.974 59.192 3.023

Total load = 1169692 kg,  CV = 3.02 %  Excellent

----------------------------------------------

#coefficients

turb305$coef

[,1]

[1,] -70.282158

[2,]   2.321101

# Storm 306: 030113,1050,030115,1800

#grade excellent/very good

----------------------------------------------

#dump 14 bots 7-14

#turb data quality fine

four()

turbsscplot("ftr",03,030113,1050,030115,1800)

turbsscplot("ftr",03,030113,1050,030115,1800,type="logxy")

turbsscplot("ftr",03,030113,1050,030115,1800,type="sqrt")

turbsscplot("ftr",03,030113,1050,030115,1800,type="power")

----------------------------------------------

#candidate 1

turb306 <- turbsrc("ftr",03,030113,1050,030115,1800,type="logxy")

ttsplot("ftr",03,turb306)

total(turb306)

start            end       type surr     yhat n    r2     s    cv

turb306 01/13/03 10:50 01/15/03 18:00 logxy:mvue turb 118524.6 8 0.996 0.051 1.819

Total load = 118525 kg,  CV = 1.82 %  Excellent

----------------------------------------------

#coefficients

turb306$coef

[,1]

-1.100770

xsample  1.325545

# Storm 307: 030219,0110,030221,1800

#grade excellent/very good

#only details not excellent are small adjustment in turb data and use of power rather than linear model

----------------------------------------------

#19:14-19, 20:1-9

#turb data quality good, adjusted small amount on falling limb between bots 6 and 7

four()

turbsscplot("ftr",03,030219,0110,030221,1800)

turbsscplot("ftr",03,030219,0110,030221,1800,type="logxy")

turbsscplot("ftr",03,030219,0110,030221,1800,type="sqrt")

turbsscplot("ftr",03,030219,0110,030221,1800,type="power")

----------------------------------------------

#candidate 1

turb1 <- turbsrc("ftr",03,030219,0110,030221,1800)

turb2 <- turbsrc("ftr",03,030219,0110,030221,1800,type="power")

turb3 <- turbsrc("ftr",03,030219,0110,030221,1800,type="logxy")

turb4 <- turbsrc("ftr",03,030219,0110,030221,1800,type="sqrt")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 02/19/03 01:10 02/21/03 18:00     linear turb 794202.9 15 0.989 47.995 2.117

turb2 02/19/03 01:10 02/21/03 18:00      power turb 810216.8 15 0.990 46.373 1.771

turb3 02/19/03 01:10 02/21/03 18:00 logxy:mvue turb 851047.8 15 0.991  0.125 4.336

turb4 02/19/03 01:10 02/21/03 18:00  sqrt:duan turb 800885.3 15 0.991  1.011    NA

# Note the linear model has a few negative predictions and CV points to power model

ttsplot("ftr",03,turb2)

ttsplot("ftr",03,turb4)

# The sqrt model fits the samples best

----------------------------------------------

#FINAL OBJECT

start            end      type surr     yhat  n    r2     s cv

turb4 02/19/03 01:10 02/21/03 18:00 sqrt:duan turb 800885.3 15 0.991 1.011 NA

Total load = 800885 kg  Excellent

----------------------------------------------

#coefficients

turb307$coef

(Intercept)  sqrt(turb)

-4.081662    1.523969

----------------------------------------------

# Storm 308: 030313,0800,030317,1200

#excellent/verygood

----------------------------------------------

#21:1-15

#turb data quality good, small adjustment falling limb between bots 11-12

four()

turbsscplot("ftr",03,030313,0800,030317,1200)

turbsscplot("ftr",03,030313,0800,030317,1200,type="logxy")

turbsscplot("ftr",03,030313,0800,030317,1200,type="sqrt")

turbsscplot("ftr",03,030313,0800,030317,1200,type="power")

----------------------------------------------

#FINAL OBJECT

turb308 <- turbsrc("ftr",03,030313,0800,030317,1200,type="power")

ttsplot("ftr",03,turb308)

total(turb308)

start            end  type surr     yhat  n    r2      s   cv

turb308 03/13/03 08:00 03/17/03 12:00 power turb 132866.8 15 0.992 14.328 2.35

Total load = 132867 kg,  CV = 2.35 %  Excellent

----------------------------------------------

#coefficients

turb308$coef

constant     power

0.1773364 1.3782528

# Storm 309: 030325,1740,030330,1200

#grade very good

----------------------------------------------

#22:11-20, 23:1-4

#turb data quality fine small adjustment falling limb bet dds 22-23

four()

turbsscplot("ftr",03,030325,1740,030330,1200)

turbsscplot("ftr",03,030325,1740,030330,1200,type="logxy")

turbsscplot("ftr",03,030325,1740,030330,1200,type="sqrt")

turbsscplot("ftr",03,030325,1740,030330,1200,type="power")

#big difference in turb:ssc of bots 14 and 15 (14 much higher ssc at similar turbidity)

turb1 <- turbsrc("ftr",03,030325,1740,030330,1200)

turb2 <- turbsrc("ftr",03,030325,1740,030330,1200,type="logxy",bias="qmle")

turb3 <- turbsrc("ftr",03,030325,1740,030330,1200,type="sqrt")

turb4 <- turbsrc("ftr",03,030325,1740,030330,1200,type="power")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 03/25/03 17:40 03/30/03 12:00     linear turb 373022.4 14 0.962 85.581 8.149

turb2 03/25/03 17:40 03/30/03 12:00 logxy:qmle turb 388878.3 14 0.985  0.162 5.296

turb3 03/25/03 17:40 03/30/03 12:00  sqrt:duan turb 379427.8 14 0.977  1.610    NA

turb4 03/25/03 17:40 03/30/03 12:00      power turb 389096.9 14 0.963 85.391 5.109

----------------------------------------------

#FINAL OBJECT

turb309 <- turbsrc("ftr",03,030325,1740,030330,1200,type="logxy",bias="qmle")

ttsplot("ftr",03,turb309)

total(turb309)

start            end       type surr     yhat  n    r2     s    cv

turb309 03/25/03 17:40 03/30/03 12:00 logxy:qmle turb 388878.3 14 0.985 0.162 5.296

Total load = 388878 kg,  CV = 5.3 %  Very good

----------------------------------------------

#coefficients

turb309$coef

[,1]

-1.278024

xsample  1.287965

# Storm 310: 030412,0100,030416,0140

#grade is good, would be very good/excellent if not for uncertainty regarding turb data adjustment at peak of storm

----------------------------------------------

#24:4-13, 25:1

#turb data adjusted top half of falling limb, probably right after peak turbidity but can't know 100%

four()

turbsscplot("ftr",03,030412,0100,030416,0140)

turbsscplot("ftr",03,030412,0100,030416,0140,type="logxy")

turbsscplot("ftr",03,030412,0100,030416,0140,type="sqrt")

turbsscplot("ftr",03,030412,0100,030416,0140,type="power")

----------------------------------------------

#candidate 1

turb310 <- turbsrc("ftr",03,030412,0100,030416,0140,type="logxy",bias="qmle")

ttsplot("ftr",03,turb310)

total(turb310)

start            end       type surr     yhat  n    r2    s   cv

turb310 04/12/03 01:00 04/16/03 01:40 logxy:qmle turb 47482.16 11 0.947 0.21 6.71

Total load = 47482 kg,  CV = 6.71 %

#estimated ssc at bot 10 much lower than lab ssc

----------------------------------------------

#FINAL OBJECT

turb310 <- turbsrc("ftr",03,030412,0100,030416,0140,type="power")

ttsplot("ftr",03,turb310)

total(turb310)

start            end  type surr     yhat  n    r2     s    cv

turb310 04/12/03 01:00 04/16/03 01:40 power turb 44790.88 11 0.993 6.334 3.442

Total load = 44791 kg,  CV = 3.44 % Good to very good

----------------------------------------------

#coefficients

turb310$coef

constant     power

0.1126097 1.4454039

# Storm 311: 030423,1900,030425,0150

#very good, only issue is small adjustment to turb data between bots 17 18

----------------------------------------------

#dump 25: 12-20

#turb data adjusted between bots 17-18

four()

turbsscplot("ftr",03,030423,1900,030425,0150)

turbsscplot("ftr",03,030423,1900,030425,0150,type="logxy")

turbsscplot("ftr",03,030423,1900,030425,0150,type="sqrt")

turbsscplot("ftr",03,030423,1900,030425,0150,type="power")

#log, power best fit

----------------------------------------------

#candidate 1

turb1 <- turbsrc("ftr",03,030423,1900,030425,0150)

turb2 <- turbsrc("ftr",03,030423,1900,030425,0150,type="power")

turb3 <- turbsrc("ftr",03,030423,1900,030425,0150,type="sqrt")

turb4 <- turbsrc("ftr",03,030423,1900,030425,0150,type="logxy")

total(turb1,turb2,turb3,turb4)

start            end       type surr      yhat n    r2      s    cv

turb1 04/23/03 19:00 04/25/03 01:50     linear turb  96712.37 9 0.991 19.536 4.710

turb2 04/23/03 19:00 04/25/03 01:50      power turb 102852.97 9 0.997 11.106 1.950

turb3 04/23/03 19:00 04/25/03 01:50  sqrt:duan turb 101120.31 9 0.992  0.560    NA

turb4 04/23/03 19:00 04/25/03 01:50 logxy:mvue turb 102438.54 9 0.990  0.092 3.306

----------------------------------------------

#FINAL OBJECT

turb311 <- turbsrc("ftr",03,030423,1900,030425,0150,type="power")

ttsplot("ftr",03,turb311)

total(turb311)

start            end  type surr     yhat n    r2      s   cv

turb311 04/23/03 19:00 04/25/03 01:50 power turb 102853.0 9 0.997 11.106 1.95

Total load = 102853 kg,  CV = 1.95 %  Very good to excellent

----------------------------------------------

#coefficients

turb311$coef

constant     power

0.1419746 1.4290790

# Storm 312: 030425,0200,030427,1640

#very good, small turb adjustments very minor, cv probably would be (?)5%

----------------------------------------------

#26:1-18

#turb data small adjustments at bots 2,3 and between 12 13

four()

turbsscplot("ftr",03,030425,0200,030427,1640)

turbsscplot("ftr",03,030425,0200,030427,1640,type="logxy")

turbsscplot("ftr",03,030425,0200,030427,1640,type="sqrt")

turbsscplot("ftr",03,030425,0200,030427,1640,type="power")

----------------------------------------------

#candidate 1

turb1 <- turbsrc("ftr",03,030425,0200,030427,1640)

turb2 <- turbsrc("ftr",03,030425,0200,030427,1640,type="logxy",bias="qmle")

turb3 <- turbsrc("ftr",03,030425,0200,030427,1640,type="sqrt")

turb4 <- turbsrc("ftr",03,030425,0200,030427,1640,type="power")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 04/25/03 02:00 04/27/03 16:40     linear turb 400891.5 18 0.982 54.969 5.413

turb2 04/25/03 02:00 04/27/03 16:40 logxy:qmle turb 396209.6 18 0.984  0.148 3.812

turb3 04/25/03 02:00 04/27/03 16:40  sqrt:duan turb 395765.6 18 0.987  1.139    NA

turb4 04/25/03 02:00 04/27/03 16:40      power turb 419490.0 18 0.980 59.166 4.876

ttsplot("ftr",03,turb2)

ttsplot("ftr",03,turb3)

----------------------------------------------

#final object

turb312 <- turbsrc("ftr",03,030425,0200,030427,1640,type="sqrt")

ttsplot("ftr",03,turb312)

total(turb312)

start            end      type surr     yhat  n    r2     s cv

turb312 04/25/03 02:00 04/27/03 16:40 sqrt:duan turb 395765.6 18 0.987 1.139 NA

Total load = 395766 kg  Very good

----------------------------------------------

#coefficients

turb312$coef

(Intercept)  sqrt(turb)

-3.478410    1.431569

# Storm 313: 030429,1140,030502,0000

#estimate only fair due to adjustment of turb data and only 4 bots

----------------------------------------------

#26:23-4, 27:1-2

#turb data adjusted between bots 24-5 and bots 25-7 never processed (don't exist) to validate turb post adjustment

#use dds 25-7 since so few bots during period?

two()

turbsscplot("ftr",03,030429,1140,030502,0000)

qsscplot("ftr",03,030429,1140,030502,0000)

#turb relationship not great, could be different relationship rising v falling limbs or could just be few bots without great relationship

#same with flow relationship

four()

turbsscplot("ftr",03,dump=25:27,txt="dump")

turbsscplot("ftr",03,dump=25:27,type="sqrt")

turbsscplot("ftr",03,dump=25:27,type="logxy")

turbsscplot("ftr",03,dump=25:27,type="power")

turb1 <- turbsrc("ftr",03,030429,1140,030502,0000)

turb2 <- turbsrc("ftr",03,030429,1140,030502,0000,dump=25:27)

turb3 <- turbsrc("ftr",03,030429,1140,030502,0000,dump=25:27,type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",03,030429,1140,030502,0000,dump=25:27,type="power")

turb5 <- turbsrc("ftr",03,030429,1140,030502,0000,dump=25:27,type="sqrt")

total(turb1,turb2,turb3,turb4,turb5)

start     end       type surr      yhat  n    r2      s     cv

turb1 04/29/03 11:40 5/02/03     linear turb 110280.82  4 0.928 30.615 15.637

turb2 04/29/03 11:40 5/02/03     linear turb 103615.48 52 0.985 37.547  6.148

turb3 04/29/03 11:40 5/02/03 logxy:qmle turb  98674.28 52 0.970  0.217  3.051

turb4 04/29/03 11:40 5/02/03      power turb 115108.03 52 0.983 41.043  5.015

turb5 04/29/03 11:40 5/02/03  sqrt:duan turb 103438.16 52 0.984  1.073     NA

# Range of estimates varies by 17% (or 12% if power model excluded)

ttsplot("ftr",03,turb1)

ttsplot("ftr",03,turb3)

----------------------------------------------

#Sarah's FINAL OBJECT best cv, conservative estimate

turb313 <- turbsrc("ftr",03,030429,1140,030502,0000,dump=25:27,type="logxy")

ttsplot("ftr",03,turb313)

total(turb313)

start     end       type surr     yhat  n   r2     s   cv

turb1 04/29/03 11:40 5/02/03 logxy:mvue turb 89804.19 52 0.97 0.217 3.05

Total load = 89804 kg,  CV = 3.05 %

#Jack's choice (stays closest to samples)

total(turb1)

start     end   type surr     yhat n    r2      s     cv

turb1 04/29/03 11:40 5/02/03 linear turb 110280.8 4 0.928 30.615 15.637

Total load = 110281 kg,  CV = 15.64 %  Fair

----------------------------------------------

#coefficients

turb1$coef

[,1]

[1,] -0.06968509

[2,]  1.20773133