KRW 2008 Annual SAND and FINES load estimation

krw08.flo <- read.flo("krw",08)

krw08.lab <- read.sand("krw",08)

krw08.sed <- merge.flo.sand("krw",08)

# Now create the ssc column: it will represent only FINES

krw08.sed$ssc <- krw08.sed$totssc - krw08.sed$sand

dim(krw08.flo)

[1] 44704     9

dim(krw08.sed)

[1] 71 11

table(krw08.flo$turbcode)

0     2     5

18069 15831 10804

15831 values of bad turbidity out of 44704 intervals to be estimated

trb08.flo <- krw08.flo[!is.na(krw08.flo$turb), ]

flo08.flo <- krw08.flo[is.na(krw08.flo$turb), ]

dim(trb08.flo)

[1] 23795     9

dim(flo08.flo)

[1] 20909     9

23795+20909

[1] 44704

# Develop model for good turb data

range(trb08.flo$turb)

[1]  -5 880

# The negative turbidity could be a problem with some model types

# Add 6 to eliminate 0 and negative values

trb08.flo$turb <- 6 + trb08.flo$turb

table(krw08.sed$labcode2)

0  1  6

56 13  2

# Code 6 means > 15% organic debris or foreign material

krw08.sed[krw08.sed$labcode2==6, ]

chr dump bottle   sand totssc turb turbcode     q labcode1 labcode2    ssc

5  (10/19/07 20:20:00)    2      8  78.79 294.39  215        0 134.6        0        6 215.60

58 (02/03/08 05:30:00)    8     23 123.72 499.15  219        0 716.8        0        6 375.43

range(trb08.flo$turb)

[1]   1 886

range(krw08.sed$turb)

[1]  30 748

sum(trb08.flo$turb < 30)/dim(trb08.flo)[1]

[1] 0.7817188

# 78% of the predicting will be below the range of the sample turbidities

par(bg="gray",mfrow=c(1,2))

turbsscplot("krw",08,txt="dump")

turbsscplot("krw",08,type="logxy")

# Eliminate the egregious and influential outliers: 8(5), 8(12), 3(3) and 4(6)

trb08.sed <- krw08.sed

trb08.sed <- trb08.sed[trb08.sed$dump!=8 | trb08.sed$bottle!=5,]

trb08.sed <- trb08.sed[trb08.sed$dump!=8 | trb08.sed$bottle!=12,]

trb08.sed <- trb08.sed[trb08.sed$dump!=3 | trb08.sed$bottle!=3,]

trb08.sed <- trb08.sed[trb08.sed$dump!=4 | trb08.sed$bottle!=5,] # outlier in log-log space

table(trb08.sed$turbcode)

0  5

66  1

four()

turbsscplot("trb",08)

turbsscplot("trb",08,type="loess")

turbsscplot("trb",08,type="power")

turbsscplot("trb",08,type="logxy")

# Try a few different models for comparison

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="linear",var=F)

turb4 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2      s cv

turb1 10/11/07 18:00 03/25/08 17:50      power turb 4900686 67 0.969 54.559 NA

turb2 10/11/07 18:00 03/25/08 17:50 logxy:mvue turb 5080456 67 0.933  0.185 NA

turb3 10/11/07 18:00 03/25/08 17:50     linear turb 4575025 67 0.966 57.480 NA

turb4 10/11/07 18:00 03/25/08 17:50      loess turb 4974813 67 0.970 54.003 NA

> sum(turb3$predssc < 0)

[1] 14391

> sum(turb4$predssc < 0)

[1] 1740

# Avoid linear and loess models because of the negative predictions

# The power model has the higher r-square of the remaining 2 models

total(turb1)

start            end  type surr    yhat  n    r2      s cv

turb1 10/11/07 18:00 03/25/08 17:50 power turb 4900686 67 0.969 54.559 NA

Total fines load by turbidity = 4900686 kg

# A relatively good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo08.flo$q)

[1]   7.614 143.100

> range(krw08.sed$q)

[1]   57.27 1172.00

# Prediction is needed below 143 cfs = 4.05 cms

sum(flo08.flo$q < 57.27)/dim(flo08.flo)[1]

[1] 0.9327084

# 93% of the predictions will be below the range of the regression data

# Eliminate high flow data that could tweak regression

par(mfrow=c(1,2),bg="gray")

qsscplot("krw",08,txt="dump")

qsscplot("krw",08)

# Eliminate high flows above ~8 cms = 280cfs

flo08.sed <- krw08.sed[krw08.sed$q < 280, ]

# Eliminate outlier 4(5) and sample 8(5) which has SSC = 0

flo08.sed <- flo08.sed[flo08.sed$dump!=8 | flo08.sed$bottle!=5,]

flo08.sed <- flo08.sed[flo08.sed$dump!=4 | flo08.sed$bottle!=5,]

par(mfrow=c(2,2),bg="white")

qsscplot("flo",08)

qsscplot("flo",08,type="power")

qsscplot("flo",08,type="loess")

qsscplot("flo",08,type="linear")

flow1 <- flowsrc("flo",08,040801,0000,080731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",08,040801,0000,080731,2400,type="power",var=F)

flow3 <- flowsrc("flo",08,040801,0000,080731,2400,type="loess",var=F)

flow4 <- flowsrc("flo",08,040801,0000,080731,2400,type="linear",var=F)

total(flow1,flow2,flow3,flow4)

start            end       type surr      yhat  n    r2      s cv

flow1 08/19/07 06:20 06/24/08 16:50 logxy:mvue flow  839138.9 24 0.103  0.373 NA

flow2 08/19/07 06:20 06/24/08 16:50      power flow  901248.4 24 0.086 67.340 NA

flow3 08/19/07 06:20 06/24/08 16:50      loess flow 1124183.6 24 0.082 68.757 NA

flow4 08/19/07 06:20 06/24/08 16:50     linear flow 1096612.7 24 0.082 67.501 NA

> sum(flow3$predssc <0)

[1] 0

> sum(flow4$predssc <0)

[1] 0

#Agreement among models only fair they vary by 34%

#logxy looks like the best model to me, highest r2 anyway and favors low values

#By far most of the prediction is beyond the lower end of the regression

start            end       type surr     yhat  n    r2     s cv

flow1 08/19/07 06:20 06/24/08 16:50 logxy:mvue flow 839138.9 24 0.103 0.373 NA

Total fines load by discharge = 839139 kg

Total fines load by turbidity = 4900686 kg

Total fines load = 839139 + 4900686 = 5739825 kg

==================================================

# Now compute total load with the same samples (i.e. just those with sand fractions)

trb08.sed$ssc <- trb08.sed$totssc

flo08.sed$ssc <- flo08.sed$totssc

four()

turbsscplot("trb",08,type="power",txt="dump")

turbsscplot("trb",08,type="logxy",txt="dump")

turbsscplot("trb",08,type="linear",txt="dump")

turbsscplot("trb",08,type="loess",txt="dump")

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="linear",var=F)

turb4 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2      s cv

turb1 10/11/07 18:00 03/25/08 17:50      power turb 5313180 67 0.966 59.571 NA

turb2 10/11/07 18:00 03/25/08 17:50 logxy:mvue turb 5456725 67 0.934  0.184 NA

turb3 10/11/07 18:00 03/25/08 17:50     linear turb 5072274 67 0.964 61.103 NA

turb4 10/11/07 18:00 03/25/08 17:50      loess turb 5480662 67 0.967 58.566 NA

> sum(turb3$predssc < 0)

[1] 10159

> sum(turb4$predssc < 0)

[1] 16

# Too many negative predictios by linear model, but loess is OK

# Loess and power models differ by only 3%

# Use the power model just for consistency with the fines model

total(turb1)

start            end  type surr    yhat  n    r2      s cv

turb1 10/11/07 18:00 03/25/08 17:50 power turb 5313180 67 0.966 59.571 NA

Total load by turbidity from reduced sample = 5313180 kg

----------------------------------------------------------

qsscplot("flo",08)

qsscplot("flo",08,type="power")

qsscplot("flo",08,type="loess")

qsscplot("flo",08,type="linear")

flow1 <- flowsrc("flo",08,040801,0000,080731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",08,040801,0000,080731,2400,type="power",var=F)

flow3 <- flowsrc("flo",08,040801,0000,080731,2400,type="loess",var=F)

flow4 <- flowsrc("flo",08,040801,0000,080731,2400,type="linear",var=F)

total(flow1,flow2,flow3,flow4)

start            end       type surr    yhat  n    r2      s cv

flow1 08/19/07 06:20 06/24/08 16:50 logxy:mvue flow 1187631 24 0.019  0.450 NA

flow2 08/19/07 06:20 06/24/08 16:50      power flow 1467839 24 0.005 92.395 NA

flow3 08/19/07 06:20 06/24/08 16:50      loess flow 1650438 24 0.022 93.988 NA

flow4 08/19/07 06:20 06/24/08 16:50     linear flow 1593529 24 0.002 92.525 NA

> sum(flow3$predssc < 0)

[1] 0

> sum(flow4$predssc < 0)

[1] 0

# Use the logxy model for consistency with fines estimate

total(flow1)

start            end       type surr    yhat  n    r2    s cv

flow1 08/19/07 06:20 06/24/08 16:50 logxy:mvue flow 1187631 24 0.019 0.45 NA

Total load by discharge from reduced sample = 1187631 kg

Total load by turbidity from reduced sample = 5313180 kg

Total load from reduced sample = 1187631 + 5313180 = 6500811

This is 22% higher than the 5342431 kg based on sum of storms and inter-storm loads

That's not surprising since so much extrapolation was required for the reduced sample

==========================================

Total load 6500811

Total fines 5739825

Total sand  760986

Percent sand = 11.7%

Percent fines= 88.3%

Applying these percentages to the more reliable total load (5342431) gives

Total fines = 4717045

Total sand = 625386