Surface Water Ambient Monitoring Program (SWAMP)

# Suspended Sediment and Turbidity Monitoring

# Freshwater Creek and Elk River

STANDARD OPERATING PROCEDURE 03

DISHCARGE MEASUREMENTS

Prepared for:

Salmon Forever

Arcata, CA

Prepared by:

Rand Eads

Surface Water Specialist

RiverMetrics LLC

PO Box 507

Lafayette, OR 97127

Version 1.0

February 2008

1. Procedures

a. Scope

This standard operating procedure describes the method for collecting instantaneous discharge measurements.  Discharge measurements are used to develop a discharge rating curve for each gage site.  Nearly continuous electronic stage measurements, in conjunction with the rating curve, can be used estimate water discharge for any time scale.

b. Method Summary

Two methods are used to measure water discharge: the 0.6 method and the 0.2 and 0.8 method.  Measurements are collected by wading and from bridges.  The wading method is used when the flow is low enough to safely wade in the channel.  Bridge sampling is used during high flows.  In both cases, the sampling method is identical, but different equipment is used.  A velocity meter is lowered to the measurement depth pre-selected verticals.  The product of velocity and the cross-sectional area of each vertical give the instantaneous discharge for that column of water.  All the instantaneous discharges from the verticals are summed to produce a discharge for that particular stage.  Generally, it takes between 15 and 20 discharge measurements, over the range or expected stages, to produce a reliable rating curve.  High discharges are the most difficult to measure because they occur infrequently and they are more technically demanding.  If the cross-sectional geometry (channel control) is stable, only occasional repeat measurements are necessary to verify the relationship once the rating is established.  If channel geometry is not stable, multiple ratings are required for various periods, creating more work fieldwork and reduced accuracy.

c. Definitions

Price AA – a velocity meter used at depths above 0.5 feet, and within the velocity range of 0.1 to 25 ft/sec.

Pigmy – a velocity meter used for shallow and low velocity flows, and within the velocity range of 0.1 to 4.9 ft/sec

Columbus sounding weight: weight used with a crane

Topsetting rod: used for wading measurements to position the meter

Crane: used from a bridge to lower the sounding weight and meter

d. Health and Safety

Wading measurements require extreme caution.  The general rule of thumb for safe wading, established by the USGS, is that the product of velocity x depth should be less than 10.  The individual entering the water must be aware of other unsafe conditions such as depressions in the streambed, submerged debris, and areas of higher velocity.  A belt should always be worn with chest waders, and a life jacket is required.  A two-person crew is mandatory when collecting wading measurements for safety reasons.

Collecting samples from a bridge with a crane and reel also requires two persons for safety.  Extreme caution must be used when measuring the rising limb at flood stage when organic transport is the highest.  The crewmember not operating the crane should be looking up stream for floating debris.  Keep fingers away from the cable and reel.  Use two persons when lifting counter weights, samplers, the crane, or reels to prevent back injuries.  Use proper lifting techniques: back is straight, knees bent, and do not twist.

e. Cautions

Do not allow the cups on the velocity meter to contact obstacles that could damage them.  If the pivot pin is damaged or worn, the meter will not pass the spin test.  It is safest to measure discharge on the falling limb of the hydrograph when less debris is in transport and the stage is changing more slowly.  Catching the sounding weight on debris can damage the equipment, possibly pulling the crane, and reel into the stream.

f. Interferences

Improper selection of the verticals, inaccurate measurements, or erroneous computations will compromise the data quality.  Taking too long to complete the measurement reduces the accuracy when the stage is changing rapidly.

g. Personnel Qualifications

Personnel must be trained in the proper sampling techniques and use of equipment.  The Project Manager will train the field crew.  Field personnel are required to read the publications listed in the reference section.

h. Equipment and Supplies

Four-wheel crane cart with counter weights or vehicle mounted crane

Type A or B sampling reel

Columbus 50 and 75 lb sounding weights

Field notebook or discharge forms

Waders and wading belt

Life jacket

Fiberglass survey tape

Price AA current meter

Price Pigmy meter (low flows)

Topsetting rod (low flows)

i. Procedures

General Considerations

- Determine if the measurement will be conducted by wading or from a bridge based on prior measurements and the knowledge of safe wading conditions.

- Decide whether to use the 0.2 and 0.8 method or 0.6 method.  If the flow is stable and the depth is greater than 2.5 feet, the 0.2 and 0.8 method provides grater accuracy.  The 0.6 method is more commonly used when the stage is changing more rapidly.

- For flood conditions, the measurement must be completed as quickly as possible to reduce the errors from large changes in stage.  In these conditions make a mid-measurement stage reading:

  - Use the 0.6 method

  - Reduce the measurement time from 40 to 20 seconds

  - Reduce the number of verticals measured to 12 or 15

- For higher velocities use the 5-click connection and for lower velocities use the 1-click connection.

- Use the correct sounding weight to reduce the angle measurement error (water depth) caused by the flow pulling the weight downstream.

- Depth setting procedure:

  - Topsetting rod, 0.6 method

    - Place the topsetting rod in the center of the vertical to be measured.

    - Read the depth on the topsetting rod.

    - Set the corresponding depth on the scale in the handle.

  - Topsetting rod, 0.2 and 0.8 depth setting

    - When the depth of the water is divided by 2 this the 0.2 setting.

    - When the depth is multiplied by 2 this becomes 0.8 setting

  - Crane

    - Use a table to determine the depth settings for each method

- For the two verticals on the edge of the channel an estimate, based on the adjacent vertical, is used if part of the edge vertical has zero depth or flow.

- Use between 20 and 25 verticals depending on the stream width.

Wading Measurements

- Assemble the current meter and mound on the topsetting rod.

- Adjust the knurled cylinder so the cups mechanism is setting freely on the pivot pin.

- Do a spin test and record the number of seconds elapsing before the cups come to a stop, 90 seconds is considered OK, if less than 60 seconds the meter should not be used.

- The position the fins so the weight is facing down.

- Determine the width of each measurement vertical and place the topsetting rod in the middle of the vertical.

- Read the stream depth on the rod.

- Set the depth in the handle.

- Stand away from the topsetting rod at a 45 degree angle and hold the rod vertically.

- Start the stop watch on a click, but do not count this click

- Count the number on clicks and seconds past 40 seconds (12 clicks at 43 seconds).

- The second crewmember records the data.

- Continue across the channel.

- No vertical should ideally contain more than 5% of the flow, and if any verticals contain more than 10% of the flow increase the number of verticals and re-measure.

Bridge Measurements

- The rule of thumb for selecting the sounding weight is that the weight, in pounds, should be greater than the product of the velocity x depth.

- Connect the weight to the hanging bar and place the meter on the hanging bar in the correct hole location (different size weights have hanging bars with meter attachment hole in specific locations).

- Connect the hanging bar to the cable.

- Lower the assemblies until the horizontal fins are on the water surface.

- Set the dial to zero.

- Lower the weight to the bottom and record the depth.

- Read the correct measurement depth from the table and raise the assembly to that depth.

- After the measurement raise the weight above the water surface before moving to the next vertical.

j. Data Records and Management

Data sheets and records are kept at the Salmon Forever office.  Computations are done in an Excel spreadsheet and paper and electronic copies are kept on file.

2. Quality Control and Assurances

Resample 10% of the verticals with repeat measurements for quality control purposes.  Repeat measurements should immediately follow the vertical to be re-sampled.  Double check all data entries in the field and ensure that the field procedures are follow correctly.

3. References and Attachments

http://pubs.usgs.gov/twri/twri3-A6/

http://pubs.usgs.gov/twri/twri3a8/

http://pubs.usgs.gov/twri/twri3-a10/

http://pubs.usgs.gov/twri/twri3-a13/