FTR 2008 Annual load estimation

ftr08.flo <- read.flo("ftr",08)

ftr08.lab <- read.lab("ftr",08)

ftr08.sed <- merge.flo("ftr",08)

nonstorms <- drop.storms("ftr",08)

dim(nonstorms)

[1] 30146     4

head(nonstorms, 5)

chr       q turb turbcode

1 (09/18/07 15:40:00) 0.09389   NA        2

2 (09/18/07 15:50:00) 0.09389   NA        2

3 (09/18/07 16:00:00) 0.09297   NA        2

4 (09/18/07 16:10:00) 0.09205   NA        2

5 (09/18/07 16:20:00) 0.09297   NA        2

table(nonstorms$turbcode)

0     1     2     5

25643    11  4273   219

4273 values of bad turbidity (and 11 questionable) out of 41891 intervals to be estimated

trb08.flo <- nonstorms[!is.na(nonstorms$turb), ]

flo08.flo <- nonstorms[is.na(nonstorms$turb), ]

dim(trb08.flo)

[1] 25866	4

dim(flo08.flo)

[1] 4280	4

25866 + 4280

[1] 30146

# Develop model for good turb data

range(trb08.flo$turb)

[1]  0 355

table(ftr08.sed$labcode2)

0   1   4   6   9

172  12   1   2   1

# Code 4 means spilled < 15% of sediment mass during or after filtering

# Code 6 means > 15% organic debris or foreign material

# Code 9 means spillage during or after filtering as in code 4,

but > 15% of sediment mass

ftr08.sed[ftr08.sed$labcode2 > 1, ]

chr dump bottle     ssc turb turbcode     q labcode1 labcode2

6  (10/19/07 10:00:00)    1      6  312.42  236        0 200.4        0        9

13 (10/20/07 06:10:00)    1     13   22.43   57        0 138.3        0        6

28 (12/04/07 00:40:00)    3      7  112.26  147        0 332.4        0        6

67 (01/04/08 08:00:00)    6     11 1120.21  610        0 520.5        0        4

turbsscplot("ftr",08,txt="dump")

attach(ftr08.sed[ftr08.sed$labcode2 > 1, ])

points(turb,ssc,pch=16,cex=1.2,col=2)

# None seem to be particular outliers; leave them in

# Eliminate bad turbidity and un-needed upper range

trb08.sed <- ftr08.sed[!is.na(ftr08.sed$turb) & ftr08.sed$turb < 375, ]

turbsscplot("trb",08,txt="dump")

# Eliminate the egregious outliers from dump 8 (bots 6 and 7)

trb08.sed <- trb08.sed[!(trb08.sed$turb > 150 & trb08.sed$ssc < 50),]

table(trb08.sed$turbcode)

0   1   5

160   2   4

turbsscplot("trb",08)

turbsscplot("trb",08,type="power")

turbsscplot("trb",08,type="logxy")

# Try a few different models for comparison

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="linear",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

total(turb1,turb2,turb3)

start            end       type surr     yhat   n    r2      s cv

turb1 10/18/07 07:50 05/04/08 12:20     linear turb 466018.5 166 0.807 45.437 NA

turb2 10/18/07 07:50 05/04/08 12:20      power turb 505505.9 166 0.824 43.412 NA

turb3 10/18/07 07:50 05/04/08 12:20 logxy:mvue turb 513755.0 166 0.834  0.492 NA

> sum(turb1$predssc < 0)

[1] 20534

# Can't use linear because of all the negative predictions

# The other two models vary by less than 2%

# The logxy model is safer than the power model because of the emphasis on low turb

# 95% of the turb data to be predicted is below 50 NTU

total(turb3)

start            end       type surr   yhat   n    r2     s cv

turb3 10/18/07 07:50 05/04/08 12:20 logxy:mvue turb 513755 166 0.834 0.492 NA

Total load = 513755 kg  Good

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo08.flo$q)

[1]  0.06277 18.79000

> range(ftr08.sed$q)

[1]   2.715 962.200

# Eliminate high flow data that could tweak regression and tracking code 7s

flo08.sed <- ftr08.sed[ftr08.sed$q < 35, ]

qsscplot("flo",08)

There is no trend whatsoever, just use the mean SSC for discharge < 19 cfs

flo08.sed <- ftr08.sed[ftr08.sed$q < 19, ]

qsscplot("flo",08)

mean(ftr08.sed$ssc[ftr08.sed$q < 35])

[1] 26.04283

mean(ftr08.sed$ssc[ftr08.sed$q < 19])

[1] 27.10872

# Use 26 mg/L for all

0.6 * 10 * 26 * sum(flo08.flo$q)/35.315

[1] 17008.80

==================================================

# Total non-storm load

513755 + 17008 = 530763 kg = 531 metric tons

# Total storm load = 2081984 kg = 2082 metric tons

# Total annual load

2082 + 531 = 2613 metric tons total annual load

# Storm load is 2081/2613 = 79.6% of annual load