Surface Water Ambient Monitoring Program (SWAMP)

# Suspended Sediment and Turbidity Monitoring

# Freshwater Creek and Elk River

STANDARD OPERATING PROCEDURE 02

DEPTH-INTEGRATED SAMPLING

Prepared for:

Salmon Forever

Arcata, CA

Prepared by:

Rand Eads

Surface Water Specialist

RiverMetrics LLC

PO Box 507

Lafayette, OR 97127

Version 1.0

February 2008

1. Procedures

a. Scope

This standard operating procedure describes the method for collecting depth-integrated samples.  These samples represent an accurate cross-section flow weighted suspended sediment concentration.  The data are used to correct, when present, sampling bias from automatic pumping sampler point samples.

b. Method Summary

Two methods are used to collect a depth-integrated sample.  The wading method is used when the flow is low enough to safely wade in the channel.  Bridge sampling is used during high flows.  In both cases, the sampling method is identical, but different equipment is used.  A sampler is lowered and raised on pre-selected verticals in the channel.  The sampler collects an isokenetic sample that represents the average SSC in the vertical.  The SSC from each sample bottle (that may represent several verticals) are used to compute an average SSC.

c. Definitions

Isokenetic -- water and sediment entering the sample bottle through the nozzle is representative of the ambient conditions.

SSC – suspended sediment concentration

EWI – equal width intervals (the sampled vertical are evenly spaced in the cross-section.

DH-48 and DH-75 – hand held depth-integrating samplers

D-74 – depth integrated sampler lowered by crane from overhead

d. Health and Safety

Wading measurements require extreme caution.  The general rule of thumb for safe wading, established by the USGS, is that the product of velocity x depth should be less than 10.  The individual entering the water must be aware of other unsafe conditions such as depressions in the streambed, submerged debris, and areas of higher velocity.  A belt should always be worn with chest waders, and a life jacket is required.  A two-person crew is mandatory when collecting wading measurements.

Collecting samples from a bridge with a crane and reel requires two persons.  Extreme caution must be used when measuring the rising limb at flood stage when organic transport is the highest.  The crewmember not operating the crane should be looking up stream for floating debris.  Keep fingers away from the cable and reel.  Use two persons when lifting counter weights, samplers, the crane, or reels to prevent back injuries.  Use proper lifting techniques: back is straight, knees bent, and do not twist.

e. Cautions

Sample bottles that are over-filled must be discarded and recollected.  Dragging the D-74 along the bed as it is being lifted will contaminate the sample with bed sediments.  Catching large floating debris on the D-74 can cause the cable to be broken, or in the worse case, the crane and reel can be pulled into the stream.  Inconsistent rate of lowering and raising (transit speed) the sampler will bias the sample when using EWI method.  Depths over 15 feet cannot be sampled with these devices.  Incorrect nozzle size for the velocity and depth can cause the sample bottle to be filled too fast or too slowly, or to sample inappropriately.

f. Interferences

Improper labeling of the sample bottles, or incorrect sampling technique can compromise the data quality.

g. Personnel Qualifications

Personnel must be trained in the proper sampling techniques and use of equipment.  The Project Manager will train the individual.  Field personnel are required to read the publications listed in the reference section.  A training video on sampling techniques is available from the USGS (can be download from the link in the Reference section).

h. Equipment and Supplies

Four wheel crane cart or vehicle mounted crane

Type A or B sampling reel

1-pint glass sample bottles with labels

D-74 sampler

DH-48 or DH-75 sampler, with handle

Field notebook

Waders and wading belt

Life jacket

Cloth tape or pre-marked verticals on bridge rail

i. Procedures

The measurement cross-sections have already been established for the four locations.  Wading measurements use established rebar pins and bridge measurements have established attachment points.  The purpose of collecting depth-integrated measurements is to determine if a bias exists between the point samples (ISCO sampler) and the cross-sectional SSC average.  Simultaneous pumped and depth-integrated samples are required for comparison.  Each sampling location should have a tally sheet showing the turbidity level at which samples have been collected.  Using the Turbidity Threshold Sampling program’s threshold list is a good starting point.  Use the following rules:

- Collect samples from new thresholds before repeating already sampled thresholds

- Always collect a sample if the current turbidity is higher than has been previously measured

- Repeat threshold samples more often if the sediment sources upstream have changed (new roads or landslides).

- Collect at least 15 to 20 samples pairs, but make the decision based on the relationship (more samples if more outliers).

- Use fewer verticals in a rapidly changing flow.

Sampling procedures:

Sampling by wading or from a bridge uses the same rules

- Based on the current streams conditions establish an equal spacing of verticals from the stream edges.  At least 10 verticals are required.

- Get the sampling equipment ready.  Use the nozzle that was shipped with the DH-48/DH-75.  Use the chart in USGS publication to determine the correct nozzle size for the D-74 based on the stream characteristics.  In general, the 3/16” diameter nozzle is appropriate.

- Plan the depth-integrated sampling so it will be half completed when the ISCO samples (at the 10-minute interval).  Alternatively, collect an ISCO sample at the start and end of depth-integrated sampling if the sediment load is changing rapidly.

- Label the sample bottle

- Start at the first vertical and use the SAME TRANSIT RATE (established by the fastest and deepest vertical) both going up and down, and across verticals.  As soon at the sampler touches the streambed immediately reverse its direction.

- Move to the next vertical and repeat the process.

- Change the sample bottle before the volume reaches the tapering bottle’s neck or at the upper portion of the etched glass volume mark.

- If the sample bottle is over-filled, discard the sample and re-sample the verticals represented by the discarded bottle.

- The minimum sample volume is the bottom of the etched glass, or approximately at the bottom of the label.  The last bottle of multiple bottles can be less than the minimum volume.

- Label the bottles with River, Date, Time, Observer, Stage, and bottle sequence (e.g. 1/3, 2/3, and 3/3).

- Write the same information in the station or field notebook.

j. Data Records and Management

Data sheets and records are kept in the laboratory.  (See the Laboratory SOP for details).

2. Quality Control and Assurances

Resample 10% of the verticals with a separate bottle for quality control.  Double check all labels in the field and ensure that the field notes indicate the matching DI sample collected by the TTS program.  Prepare and examine the chain of custody forms to make sure they are complete.

3. References and Attachments

http://pubs.usgs.gov/twri/twri3-c2/

http://pubs.usgs.gov/sir/2005/5077/