KRW 2004 Annual load estimation

krw04.flo <- read.flo("krw",04)

krw04.lab <- read.lab("krw",04)

krw04.sed <- merge.flo("krw",04)

nonstorms <- drop.storms("krw",04)

dim(nonstorms)

[1] 43139     3

head(nonstorms, 5)

chr     q turb turbcode

1 (08/17/03 06:00:00) 4.919   NA        2

2 (08/17/03 06:10:00) 4.919   NA        2

3 (08/17/03 06:20:00) 4.919   NA        2

4 (08/17/03 06:30:00) 4.919   NA        2

5 (08/17/03 06:40:00) 4.919   NA        2

table(nonstorms$turbcode)

0     1     2     4     5     6     7

5900   391 35042   271  1237   282    16

35042 values of bad turbidity out of 43139 intervals to be estimated

trb04.flo <- nonstorms[!is.na(nonstorms$turb), ]

flo04.flo <- nonstorms[is.na(nonstorms$turb), ]

dim(trb04.flo)

[1] 8097     3

dim(flo04.flo)

[1] 35042     3

8097+35042

[1] 43139

# Develop model for good turb data

range(trb04.flo$turb)

[1]   3 399

table(krw04.sed$labcode2)

0   1

164   2

# No bad lab codes to worry about

# Eliminate bad turbidity and turbidity > 450

trb04.sed <- krw04.sed[!is.na(krw04.sed$turb) & krw04.sed$turb < 450, ]

table(trb04.sed$turbcode)

0  1  4  5  6

96  6  3 12  1

turbsscplot("trb",04)

turbsscplot("trb",04,type="logxy",txt="dump")

Dump 4 bottle 1 has too much influence and the turbidity is questionable.

krw04.sed <- krw04.sed[-1,]

trb04.sed <- krw04.sed[!is.na(krw04.sed$turb) & krw04.sed$turb < 450, ]

turbsscplot("trb",04,type="logxy")

turb1 <- turbsrc("trb",04,030801,0000,040731,2400,type="logxy",var=F)

start            end       type surr     yhat   n    r2     s cv

turb1 11/12/03 20:30 03/27/04 12:00 logxy:mvue turb 539254.4 117 0.936 0.324 NA

Total load = 539254 kg

# A relatively good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo04.flo$q)

[1]   3.048 146.600

> range(krw04.sed$q)

[1]   6.783 972.400

# Eliminate high discharges to avoid influence on regression

flo04.sed <- krw04.sed[krw04.sed$q <= 165, ]

qsscplot("flo",04)

The scatter is huge. We just have to pray there isn't much sediment

to be estimated here.

flow1 <- flowsrc("flo",04,030801,0000,040731,2400,var=F)

total(flow1)

start            end       type surr     yhat  n    r2    s cv

flow1 08/17/03 06:00 07/01/04 08:40 logxy:mvue flow 346019.1 96 0.385 1.09 NA

Total load = 346019 kg

# Compare with power model and loess models

two()

qsscplot("flo",04,type="power")

qsscplot("flo",04,type="loess")

flow2 <- flowsrc("flo",04,030801,0000,040731,2400,type="power",var=F)

flow3 <- flowsrc("flo",04,030801,0000,040731,2400,type="loess",var=F)

total(flow1,flow2,flow3)

start            end       type surr     yhat  n    r2       s cv

flow1 08/17/03 06:00 07/01/04 08:40 logxy:mvue flow 398381.3 96 0.373   1.101 NA

flow2 08/17/03 06:00 07/01/04 08:40      power flow 532738.2 96 0.114  99.607 NA

flow3 08/17/03 06:00 07/01/04 08:40      loess flow 504496.0 96 0.114 100.012 NA

# All estimates are poor. Luckily this part of the load is quite small

# Total non-storm load 539254 + 346019

539254 + 346019 = 885273 kg = 885 metric tons

# Total storm load = 5396535 kg

5397 + 885 = 6282 metric tons

# Storm load is 5397/6282 = 85.9% of annual load