Data corrections FTR04 - Jack Lewis

These corrections are in addition to Clark’s earlier work.

Stage corrections - Jack Lewis

All the early data thru October is very strange and highly questionable.  Diurnal fluctuations with dips at 7-9am each day. After transducer was replaced on Oct. 12, we see different diurnal fluctuations of amplitude 0.06, with afternoon peaks.  These went away on Oct 25 after re-wiring data logger.  I did not try to interpolate to remove all these.

There was a sudden jump in stage from 0.885 to 0.92 at 19Sep03:1520

I removed this jump by shifting upwards all the data prior to that time.

The section from 01Nov2003:0630 to 04Nov2003:0010 was displaced.  I shifted this up to fit the rest of the record.

I applied variable shifts to match Ors thru 12Nov.  Had to ignore OR of 10Nov to get a reasonable curve.

Reconstructed 12Nov2003:1400 to 05Dec2003:1340 from SFM using linear method.  This section was so screwed up there was no alternative.  After reconstructing I made a series of shifts to get better agreement with the Ors.

After Dec05 only relatively minor shifts were required to match ORs.  Removed a stage drop at 27Dec03:0240.

Reconstructed the recession 23Apr to 17May from SFM using linear method.  This removed the diurnal transducer fluctuations that started up again.

Reconstructed 22May to 27May from SFM using linear method.  Again to remove diurnal fluctuations.

Reconstructed 31May to 30Jun from SFM using linear method. Did this in two pieces to match all Ors. Again to remove diurnal fluctuations.

Turbidity corrections - Jack Lewis

Removed a few short spikes that Clark missed in dumps 1-10

There were lots and lots of diurnal sun-spikes.  I left most of the small ones that Clark did not remove.

Coded most of storm on 15Nov as questionable and interpolated the big spikes.  The turbidity pattern is very unusual and the lab data don’t corroborate the spikes.

Unusual spikes on 19Feb and 20Feb maxed out probe.  Samples that were collected had significant amount of sediment.  Both disturbances were from 0940 to 1530 hours, within

10 minutes.  This seems like construction or a similar point-source disturbance.  I interpolated to remove both spikes, and will regard the samples as invalid for load estimation.

Exceptions: interpolated sun-disturbances on Mar 27, Apr 23, May 19.

No explanation of big turbidity pulse on Apr 15. Shape looks real. Not sampled because min stage had been raised to avoid sampling sunspikes.

Reconstructed 04Apr2004:0650 to 1450 from SFM

Interpolated noise on May 28.