KRW05 Event Loads

stn,storm,sdate,stime,edate,etime

KRW,501,041206,0940,041207,1940   75376 kg  Poor quality

KRW,502,041207,1950,041211,0000  722873 kg  Fair

KRW,503,041230,1930,050104,0000  535531 kg  Very good

KRW,504,050107,0400,050108,0850  241696 kg  Very good

KRW,505,050108,0900,050111,0000  505551 kg  Very good

KRW,506,050319,0640,050323,0030  423069 kg  Fair

KRW,507,050327,1230,050331,1200 1021153 kg  Very good

KRW,508,050406,2350,050408,0820  235399 kg  Good

KRW,509,050408,0830,050412,0600 1868466 kg  Excellent

KRW,510,050508,0010,050511,1200  294453 kg  Good

-----------------------------------------------------

SUM of events:                  5923567 kg

-----------------------------------------------------

krw05.flo <- read.flo("krw",05)

krw05.lab <- read.lab("krw",05)

mismatches("krw",05)

# Many unmatched samples in flo file; none in lab file

krw05.sed <- merge.flo("krw",05)

Storm 501: dump 5: KRW,501,041206,0940,041207,1940

No turbidity data; only 4 samples, all on recession;

No choice but to use discharge sediment rating curve.

qsscplot("krw",05,041206,0940,041207,1940)

flow1 <- flowsrc("krw",05,041206,0940,041207,1940)

ttsplot("krw",05,flow1)

total(flow1)

start            end       type surr     yhat n    r2   s    cv

flow1 12/06/04 09:40 12/07/04 19:40 logxy:mvue flow 75375.89 4 0.919 0.1 5.714

Total load = 75376 kg,  CV = 5.71 %  Poor quality

Storm 502: dumps 5-6 KRW,502,041207,1950,041211,0000

No turbidity or samples on first and largest of the two peaks.

5 samples on recession of second peak, followed by 3 others.

qsscplot("krw",05,dump=rep(5:6,c(5,8)),bot=c(1:5,1:8))

# Not too bad a relationship and its consistent for both events. Don't get too happy though; all samples are from recessions.

turbsscplot("krw",05,dump=6,bot=1:8)

flow1 <- flowsrc("krw",05,041207,1950,041208,1810,dump=rep(5:6,c(5,8)),bot=c(1:5,1:8))

turb1 <- turbsrc("krw",05,041208,1820,041211,0000,dump=6,bot=1:8)

ttsplot("krw",05,flow1,turb1)

total(flow1,turb1)

start            end       type surr     yhat  n    r2      s    cv

flow1 12/07/04 19:50 12/08/04 18:10 logxy:mvue flow 489179.5 12 0.977  0.178 9.198

turb1 12/08/04 18:20 12/11/04 00:00     linear turb 235580.0  8 0.980 20.070 4.044

Total load = 724760 kg,  CV = 6.35 %

# Needs a refinement to remove the discontinuity

flow2 <- flowsrc("krw",05,041207,1950,041208,1500,dump=rep(5:6,c(5,8)),bot=c(1:5,1:8))

turb1 <- turbsrc("krw",05,041208,1820,041211,0000,dump=6,bot=1:8)

> endpoints(flow2)

start       end

48.74144 333.61950

> endpoints(turb1)

start       end

299.16993  29.63991

time1 <- lineartime("krw",05,041208,1510,041208,1810,ssc1=333,ssc2=299)

ttsplot("krw",05,flow2,time1,turb1)

total(flow2,time1,turb1)

start            end       type surr      yhat  n    r2      s    cv

flow2 12/07/04 19:50 12/08/04 15:00 logxy:mvue flow 430619.61 12 0.977  0.178 9.351

time1 12/08/04 15:10 12/08/04 18:10     linear time  56673.91  0    NA     NA    NA

turb1 12/08/04 18:20 12/11/04 00:00     linear turb 235579.97  8 0.980 20.070 4.044

Total load = 722873 kg  Quality fair to poor

Storm 503: dump 7: KRW,503,041230,1930,050104,0000

Very good turbidity data and sample coverage

Can include bottles immmediately before (3) and after (16) event.

turbsscplot("krw",05,041230,1930,050104,0000)

turbsscplot("krw",05,dump=7,bot=3:16)

# Each peak has a hysteresis loop, but they are narrow enough to ignore.

turb1 <- turbsrc("krw",05,041230,1930,050104,0000,dump=7,bot=3:16)

ttsplot("krw",05,turb1)

total(turb1)

start     end   type surr   yhat  n    r2      s    cv

turb1 12/30/04 19:30 1/04/05 linear turb 535531 14 0.965 25.902 4.305

Total load = 535531 kg,  CV = 4.31 %  Quality very good

Storm 504: dump 8: KRW,504,050107,0400,050108,0850

Very good turbidity data with good sample coverage

four()

turbsscplot("krw",05,050107,0400,050108,0850)

turbsscplot("krw",05,050107,0400,050108,0850,type="logxy")

turbsscplot("krw",05,dump=8,bot=1:3)

turbsscplot("krw",05,dump=8,bot=4:8)

Can try this with a single regression or two for hysteresis.

turb1 <- turbsrc("krw",05,050107,0400,050108,0850)

endpoints(turb1)

start       end

-8.190829 97.327534

# Goes negative for first few hours

turb2 <- turbsrc("krw",05,050107,0400,050108,0850,type="logxy")

ttsplot("krw",05,turb2)

start            end       type surr     yhat n    r2      s    cv

turb1 01/07/05 04:00 01/08/05 08:50     linear turb 241268.0 8 0.953 32.362 4.252

turb2 01/07/05 04:00 01/08/05 08:50 logxy:mvue turb 241696.2 8 0.980  0.097 4.261

turb3 <- turbsrc("krw",05,050107,0400,050107,1800,type="logxy")

turb4 <- turbsrc("krw",05,050107,1810,050108,0850,type="logxy")

total(turb3,turb4)

start            end       type surr      yhat n    r2     s    cv

turb3 01/07/05 04:00 01/07/05 18:00 logxy:mvue turb  69603.86 3 0.999 0.043 3.591

turb4 01/07/05 18:10 01/08/05 08:50 logxy:mvue turb 169899.31 5 0.989 0.065 3.357

Total load = 239503 kg,  CV = 2.6 %

# Using turb3 and turb4 creates a discontinuity in SSC that is hard to fix.

# Stay with turb2.

total(turb2)

start            end       type surr     yhat n   r2     s    cv

turb2 01/07/05 04:00 01/08/05 08:50 logxy:mvue turb 241696.2 8 0.98 0.097 4.261

Total load = 241696 kg,  CV = 4.26 %  Quality very good

Storm 505: dump 7: KRW,505,050108,0900,050111,0000

Excellent turbidity and good sample coverage

Include bottle 8 from previous event

four()

turbsscplot("krw",05,dump=8,bot=8:15)

turbsscplot("krw",05,dump=8,bot=8:15,type="power")

turbsscplot("krw",05,dump=8,bot=8:15,type="loess",degree=2)

turbsscplot("krw",05,dump=8,bot=8:15,type="pairs")

Looks like a very similar hysteresis situation to storm 504.

turb1 <- turbsrc("krw",05,050108,0900,050111,0000,dump=8,bot=8:15)

turb2 <- turbsrc("krw",05,050108,0900,050111,0000,dump=8,bot=8:15,type="power")

turb3 <- turbsrc("krw",05,050108,0900,050111,0000,dump=8,bot=8:15,type="power")

turb4 <- turbsrc("krw",05,050108,0900,050111,0000,dump=8,bot=8:15,type="loess",degree=2)

turb5 <- turbsrc("krw",05,050108,0900,050111,0000,type="pairs")

total(turb1,turb2,turb3,turb4,turb5)

start     end   type surr     yhat n    r2      s     cv

turb1 01/08/05 09:00 1/11/05 linear turb 475379.5 8 0.869 92.822 15.494

turb2 01/08/05 09:00 1/11/05  power turb 458716.4 8 0.931 69.252  9.919

turb3 01/08/05 09:00 1/11/05  power turb 458716.4 8 0.931 69.252  9.919

turb4 01/08/05 09:00 1/11/05  loess turb 505551.4 8 0.962 61.629     NA

turb5 01/08/05 09:00 1/11/05  pairs turb 472392.7 7    NA     NA     NA

# Loess is higher because of the curve is steeper at the top, with the extrapolation

# But it's the only reasonable fit near the peak

ttsplot("krw",05,turb4)

total(turb4)

start     end  type surr     yhat n    r2      s cv

turb4 01/08/05 09:00 1/11/05 loess turb 505551.4 8 0.962 61.629 NA

Total load = 505551 kg  Very good quality

Storm 506: dumps 12-13: KRW,506,050319,0640,050323,0030

Excellent turbidity and very good sample coverage.

But bottles 12-18 in second peak had an ISCO problem.

four()

turbsscplot("krw",05,050319,0640,050323,0030)

qsscplot("krw",05,050319,0640,050323,0030)

turbsscplot("krw",05,dump=rep(12:13,c(9,3)),bot=c(4:11,19,1:3))

turbsscplot("krw",05,dump=rep(12:13,c(8,3)),bot=c(4,6:11,19,1:3))

Something seems terribly wrong with samples 12-14, and a couple others;  Lab notes to the rescue, say "Dump 12 ISCO arm problems - arm hanging up - disqualify D12 B12-18".

These should have been coded as problematic.  No documentation of a problem with sample 5.

turb1 <- turbsrc("krw",05,050319,0640,050323,0030,dump=rep(12:13,c(8,3)),bot=c(4,6:11,19,1:3))

ttsplot("krw",05,turb1)

start            end   type surr     yhat  n    r2     s   cv

start            end   type surr     yhat  n    r2     s    cv

turb1 03/19/05 06:40 03/23/05 00:30 linear turb 423069.1 11 0.987 9.853 1.767

Total load = 423069 kg,  CV = 1.77 %  Quality fair

Storm 507: dump 13-14 : KRW,507,050327,1230,050331,1200

Two peaks: excellent turbidity and sample coverage

four()

turbsscplot("krw", 05, 050327,1230,050331,1200)

turbsscplot("krw",05, dump=c(rep(13,4),14),bot=c(7,10:12,1))

turbsscplot("krw",05, dump=14, bot=1:13, type="power")

turbsscplot("krw",05, dump=14, bot=1:13, type="pairs")

From lab notes, "filter mix up, leave out [dump 13, bottle 9]".  From the looks of the scatterplot, bottle 8 was involved in the mix-up too. I'll omit both.

turb1 <- turbsrc("krw",05,050327,1230,050329,0000,dump=c(rep(13,4),14),bot=c(7,10:12,1))

turb2 <- turbsrc("krw",05,050329,0010,050331,1200,dump=14,bot=1:13,type="power")

turb3 <- turbsrc("krw",05,050329,0010,050331,1200,type="pairs")

total(turb1,turb2)

start            end   type surr     yhat  n    r2      s    cv

turb1 03/27/05 12:30 03/29/05 00:00 linear turb 171844.1  5 0.955 24.028 5.695

turb2 03/29/05 00:10 03/31/05 12:00  power turb 836457.2 13 0.972 54.667 3.213

Total load = 1008301 kg,  CV = 2.84 %

ttsplot("krw",05,turb1,turb3)

total(turb1,turb3)

start            end   type surr     yhat  n    r2      s    cv

turb1 03/27/05 12:30 03/29/05 00:00 linear turb 171844.1  5 0.955 24.028 5.695

turb3 03/29/05 00:10 03/31/05 12:00  pairs turb 849308.7 12    NA     NA    NA

Total load = 1021153 kg  Very good

Storm 508: dump 15: KRW,508,050406,2350,050408,0820

Small event, good turbidity, sample coverage OK

four()

turbsscplot("krw",05,050406,2350,050408,0820)

turbsscplot("krw",05,dump=15)

qsscplot("krw",05,050406,2350,050408,0820)

qsscplot("krw",05,050406,2350,050408,0820,type="pairs")

turb1 <- turbsrc("krw",05,050406,2350,050408,0820)

flow1 <- flowsrc("krw",05,050406,2350,050408,0820)

flow2 <- flowsrc("krw",05,050406,2350,050408,0820,type="pairs")

total(turb1,flow1,flow2)

start            end       type surr     yhat n    r2      s     cv

turb1 04/06/05 23:50 04/08/05 08:20     linear turb 227872.7 5 0.814 58.657 10.526

flow1 04/06/05 23:50 04/08/05 08:20 logxy:mvue flow 238699.9 5 0.604  0.402 22.989

flow2 04/06/05 23:50 04/08/05 08:20      pairs flow 225572.2 5    NA     NA     NA

ttsplot("krw",05,flow2)

# Could not extrapolate to the higher discharges between samples 3 and 4.

# Use lineartime until bottle 4 then turbsrc with linear model

# Set ssc1 and ssc2 from samples 1 and 4

time1 <- lineartime("krw",05,050406,2350,050407,1540,ssc1=46, ssc2=373)

turb2 <- turbsrc("krw",05,050407,1550,050408,0820)

ttsplot("krw",05,time1,turb2)

total(time1,turb2)

start            end   type surr     yhat n r2     s   cv

time1 04/06/05 23:50 04/07/05 15:40 linear time 116116.5 2 NA    NA   NA

turb2 04/07/05 15:50 04/08/05 08:20 linear turb 119282.6 3  1 1.027 0.25

Total load = 235399 kg   Quality good

Storm 509: dump 15-16: KRW,509,050408,0830,050412,0600

Excellent turbidity and sample coverage;

# include one extra bottle at each end

four()

turbsscplot("krw",05,dump=rep(15:16,c(17,4)),bot=c(6:22,1:4))

turbsscplot("krw",05,dump=15,bot=6:14,type="power")

turbsscplot("krw",05,dump=15,bot=6:14,type="loess",degree=2)

turbsscplot("krw",05,dump=rep(15:16,c(9,4)),bot=c(14:22,1:4),type="loess",degree=2,span=0.75)

turb1 <- turbsrc("krw",05,050408,0830,050412,0600)

start            end   type surr    yhat  n    r2       s    cv

turb1 04/08/05 08:30 04/12/05 06:00 linear turb 1858795 19 0.914 130.743 5.601

Total load = 1858795 kg,  CV = 5.6 %

turb2 <- turbsrc("krw",05,050408,0830,050408,2250,dump=15,bot=6:14,type="loess",degree=2)

turb3 <- turbsrc("krw",05,050408,2300,050412,0600,dump=rep(15:16,c(9,4)),bot=c(14:22,1:4),type="loess",degree=2,span=0.75)

ttsplot("krw",05,turb2,turb3)

total(turb2,turb3)

start            end  type surr      yhat  n    r2      s cv

turb2 04/08/05 08:30 04/08/05 22:50 loess turb  756451.3  9 0.988 68.823 NA

turb3 04/08/05 23:00 04/12/05 06:00 loess turb 1112014.9 13 0.994 45.538 NA

Total load = 1868466 kg  Excellent

Storm 510: dump 19: KRW,510,050508,0010,050511,1200

Triple peak; excellent turbidity, good sample coverage

except no samples on rising limb of first peak

# Include sample 12 at 050511:1910

four()

turbsscplot("krw",05,050508,0010,050511,1910)

turbsscplot("krw",05,dump=19,bot=5:12)

turbsscplot("krw",05,dump=15:19)

turbsscplot("krw",05,dump=rep(c(15,19),c(13,3)),bot=c(10:22,1:3))

Supplement samples with dump 15 for first peak

turb1 <- turbsrc("krw",05,050508,0010,050508,1810,dump=rep(c(15,19),c(13,3)),bot=c(10:22,1:3))

start       end

6.708263 99.014818

turb2 <- turbsrc("krw",05,050509,0540,050511,1200,dump=19,bot=8:12)

start       end

294.15749  44.37974

time1 <- lineartime("krw",05,050508,1820,050509,0530,ssc1=99,ssc2=294)

ttsplot("krw",05,turb1,time1,turb2)

total(turb1,time1,turb2)

start            end   type surr      yhat  n    r2       s     cv

turb1 05/08/05 00:10 05/08/05 18:10 linear turb  58608.11 16 0.919 129.209 31.028

time1 05/08/05 18:20 05/09/05 05:30 linear time  70867.18  1    NA      NA     NA

turb2 05/09/05 05:40 05/11/05 12:00 linear turb 164977.60  5 0.974  13.998  4.979

Total load = 294453 kg   Quality good

# Most reliable part of estimate is the last half, which accounts for most of the sediment.