There were discharge measurements on Dec 20, 24, and 28 (on the recession of storms 603, 604, and 605) with lower than expected velocities and discharges. Clark fit two rating curves, one before the Jan 1 slide and one after. A discharge measurement at 10.87 ft on Feb 2 (storm 614) indicates clearly that by this time the rating equation had reverted back to the former regime.

The rating equation could have been affected by sediment deposited from the slide or by debris that was observed at the site between Feb 28 and Mar 24.  Both these might be expected to raise the stage for a given flow.  However, the stage was raised only for the 3 measurements taken prior to the Jan 1 slide, and there is no record of debris at the measurement site during those measurements.  In a low-gradient channel, however, it is quite possible that backwater effects (from jams, the bridge itself or tributary channel inflows) create a situation where discharge is not a single-valued function of stage.  The water-surface elevation can then be higher on the falling limb of the hydrograph than on the rising limb at the same discharge producing a "looped" rating curve.  See http://www.weather.gov/oh/hrl//papers/ams/ams98-5.htm.

Below is a plot of peak discharges based on the HY05 rating equations, and corrected stages at both SFM and KRW. Although discharge is being plotted you can think of it as a transformation of stage, since just one equation was used at each station.  The plotting symbols are the storm numbers in each year.

# R commands:

sf05 <- abs(getpeaks("05","sfm")$peak)

sf06 <- abs(getpeaks("06","sfm")$peak)

nf05 <- abs(getpeaks("05","krw")$peak)

nf06 <- abs(getpeaks("06","krw")$peak)

elkpeaks <- data.frame(sf=c(sf05,sf06),nf=c(nf05,nf06))

row.names(elkpeaks) <- c(501:510,601:620)

attach(elkpeaks)

plot(nf,sf,type="n",xlab="KRW peak",ylab="SFM peak")

text(nf[1:10],sf[1:10],1:10,cex=0.6)

text(nf[11:30],sf[11:30],1:20,cex=0.6,col=2)

abline(lm(sf ~ nf))

legend(locator(1),c("hy05","hy06"),col=1:2,pch=1)

There is no indication of elevated stages at storms 3-5 in HY06, when the anomalous discharge measurements were taken. Note: the scatter in HY05 is equally as great as that in HY06, when the hypothesized changes occurred.

Let’s took a look at the pattern of residuals with time.

plot(resid(fit0),type="n")

text(row.names(elkpeaks),resid(fit0))

text(1:10,resid(fit0)[1:10],1:10,cex=0.6)

text(11:30,resid(fit0)[11:30],1:20,col=1,cex=0.6)

“Index” is the storm sequence. The pattern looks random. We can test for a statistical difference between pre and post-slide (HY06 storms 1-6 and 7-20).

# R commands (see below for plot)

plot(nf,sf,type="n",xlab="KRW peak",ylab="SFM peak")

text(nf[11:16],sf[11:16],1:6,cex=0.6,col=1)

text(nf[17:30],sf[17:30],7:20,cex=0.6,col=2)

abline(lm(sf ~ nf, data=elkpeaks, subset=11:16))

abline(lm(sf ~ nf, data=elkpeaks, subset=17:30),col=2)

elkpeaks$T <- factor(rep(0:2,c(10,6,14)))

fit1 <- lm(sf ~ nf, data=elkpeaks, subset=11:30)

fit2 <- lm(sf ~ nf + T, data=elkpeaks, subset=11:30)

anova(fit1,fit2)

Analysis of Variance Table

Model 1: sf ~ nf

Model 2: sf ~ nf + T

Res.Df        RSS Df  Sum of Sq      F  Pr(>F)

1     18 2.0451e-06

2     17 1.6278e-06  1 4.1725e-07 4.3576 0.05221 .

Almost significant, but the change is in the wrong direction.  If the stage had been elevated, as indicated by the discharge measurements, the pre-slide data (points 1-6) would show higher than expected peaks at SFM (since only one rating equation was used).  Also storm 4 is more like the post-slide data and storms 9,10,12, 13, and 15 are more like the pre-slide data.  (All data in the plot are from HY2006: pre-slide in black and post-slide in red).

Let’s see what this looks like if we apply the Clark’s pre-slide and post-slide equations to HY2007:

(Ignore the regression line.) Applying the pre-slide rating equation is clearly a mistake. As expected it moved the pre-slide data points (HY06 storms 1-6) away from the other points.  There is no reason these peaks should not fit the overall relation between the KRW and SFM peaks. The equation has a HUGE effect. It’s not hard to see that applying the “pre-slide” equation to any of the HY06 storms would make them outliers.  The post-slide equation seems to be consistent with the HY05 rating equation.  The post-slide discharge measurements should be considered for addition to the HY05 equation and the pre-slide rating equation now seems very suspicious. I haven’t found any problems with the discharge measurements themselves.  I am guessing that a backwater condition existed and that there just was not a simple relationship between stage and discharge under the conditions measured. So either those points should be omitted or some sort of hysteresis model should be applied. The post-slide points were also measured during storm recessions, so if there was a backwater condition, it appears to have been temporary. In the absence of a hydraulic model, my recommendation for storm 3-5 sediment loads would be to use a compromise rating equation based on all the HY06 measurements. Such a rating equation will underestimate the peak flows, so for any peak flow analysis use a rating equation that omits the Dec 20-28 measurements.

The following shows a rating equation built from HY03 to HY07 excluding the Dec 20-28, 2006 measurements.  Intercept was artificially set to -32 in Excel to get q=0 at about 1.40.  Use this for HY06 storms 6-20.

How to calculate discharge for storms 602-605 (Dec 18-30)?

The peak flows estimated using the HY05 rating equation are very close to those predicted from KRW, esp for storms 4 and 5 (see second graph above). Therefore the HY05 rating equation (or, equivalently, the HY03-07 curve which is nearly identical) seems fine for the rising limbs.  That means when the stage started falling, the discharge was probably also falling.  (When there is hysteresis, there can be a period of time when the stage is steady or falling and the discharge is still rising.) If the falling limbs follow the lower curve described by the pre-slide measurements, then we need to determine or guess the duration of the transition period when the relationship is switching to the lower curve. The only information we have in that regard are the times of the discharge measurements.  Each was taken on the falling limb.

Dec 20 from 4 to 6 hrs after the peak

Dec 24 from 22 to 23 hrs after the peak

Dec 28 from 2 to 5.5 hrs after the peak (took 3.5 hrs to complete!)

Since these data are presumably governed by the pre-slide rating curve, let’s guess that the transition period happens in the first 3 hrs after the peak.  Then we just need to reconstruct those 3 hrs from KRW (in TTS Adjuster) and we’re done!  The plan:

(1) use the HY03-07 rating for rising limb

(2) use pre-slide rating for falling limb

(3) reconstruct the first 3 hours after the peak using KRW

Pre-slide rating curve (to use for the recessions) is:

Q = 1.9237*s^2 + 3.8039*s - 5.4047

The peak times are

602 (12/19/05 16:10:00)

603 (12/20/05 12:50:00)

604 (12/22/05 15:30:00)

605 (12/28/05 10:20:00)

The difficulty:  Applying the pre-slide rating simply makes the SFM flows too low relative to KRW.  SFM flows are generally 70-100% of the KRW flows.  Applying the pre-slide rating to the recessions drops it to less than half.  What if we just ignore those Q measurements and use a single rating equation (based on HY03-07) for all of HY2006?  If storms 602-605 fit the general relationship between KRW and SFM storm flow volumes, then that is what we should do.

# R commands

# Rating equation for SFM06 is q = 2.5828*s^2+19.266*s-32

sf05 <- abs(getflows("05","sfm")$flow)

sf06 <- abs(getflows("06","sfm")$flow)

nf05 <- abs(getflows("05","krw")$flow)

nf06 <- abs(getflows("06","krw")$flow)

elkflows <- data.frame(sf=c(sf05,sf06),nf=c(nf05,nf06))

row.names(elkflows) <- c(501:510,601:620)

attach(elkflows)

plot(nf,sf,type="n",xlab="KRW flow",ylab="SFM flow")

text(nf[1:10],sf[1:10],1:10,cex=0.6)

text(nf[11:30],sf[11:30],1:20,cex=0.6,col=2)

abline(lm(sf ~ nf))

legend(locator(1),c("hy05","hy06"),col=1:2,pch=1)

Storms 603, 604, and 605 fit very well.  Storm 602 appears to have too small a flow at South Fork.  Applying the pre-slide rating equation will lower the SFM discharge further, moving 602 the wrong way.  (Note: storms 503 and 606 are also unexplained outliers. 606 is the storm during which the slide occurred.)  This analysis suggests that the pre-slide discharge measurements should be left out of the rating curve computations.