FTR rating equation update: Lewis 01/09/2010

This analysis includes 4 new measurements taken in HY2008.  To account for staff plate replacement (Feb 2006), I've added 0.045 ft to all stage measurements thru calendar year 2005

ftrq <- read.csv("ftr/Q and rating curves/Qmeas0308.csv")

hy         id       date   stg       q qual

1  2003  11/9/2002  11/9/2002 0.735    5.35 good

2  2003 12/14/2002 12/14/2002 2.455  166.72 poor

3  2003 12/15/2002 12/15/2002 2.085  110.89 good

4  2003 12-28-02 a 12/28/2002 5.285  814.52 fair

5  2003 12-28-02 b 12/28/2002 6.435 1089.22 fair

6  2003   1/5/2003   1/5/2003 1.645   36.81 good

7  2003  4/13/2003  4/13/2003 2.255  119.17 good

8  2004 10/12/2003 10/12/2003 1.015    0.29 fair

9  2004   1/4/2004   1/4/2004 2.045   69.67 good

10 2004  2/29/2004  2/29/2004 1.935   63.89 good

11 2005  9/19/2004  9/19/2004 1.015    0.33 good

12 2005  11/6/2004  11/6/2004 1.155    3.81 fair

13 2005   1/3/2005   1/3/2005 2.005   71.93 fair

14 2005   1/4/2005   1/4/2005 1.845   50.59 fair

15 2005   1-5-05 a   1/5/2005 1.685   29.36 good

16 2005   1-5-05 b   1/5/2005 1.685   34.26 fair

17 2005   1/6/2005   1/6/2005 1.615   24.06 fair

18 2005  3/21/2005  3/21/2005 2.295  125.47 fair

19 2005  3/22/2005  3/22/2005 2.115   93.99 good

20 2005  3/29/2005  3/29/2005 3.765  194.04 fair

21 2006  9/18/2005  9/18/2005 0.715    0.58 fair

22 2006 12/11/2005 12/11/2005 1.205   15.31 good

23 2006  3/12/2006  3/12/2006 1.830   50.26 good

24 2007  9/10/2006  9/10/2006 0.540    0.16 fair

25 2007 11/12/2006 11/12/2006 0.790    2.35 fair

26 2007 11/18/2006 11/18/2006 1.140    6.65 good

27 2007 12/29/2006 12/29/2006 1.820   40.67 good

28 2007  2/25/2007  2/25/2007 3.490  389.37 good

29 2008  1/10/2008  1/10/2008 2.170  108.66

30 2008  1/31/2008  1/31/2008 3.680  375.37

31 2008   2/1/2008   2/1/2008 2.710  179.13

32 2008   2/5/2008   2/5/2008 1.920   61.25

ftrq$sym <- as.character(ftrq$hy %% 1000)

attach(ftrq)

plot(stg, q, type="p", pch=sym, cex=0.7)

AS BEFORE, I'm going to assume that the 3/29/05 measurement is an error.

detach(ftrq)

ftrq <- ftrq[-20, ]

attach(ftrq)

scatter.smooth(stg,q, type="n", pch=sym, span=0.55)

points(stg, q, pch=sym, col=2, cex=0.7)

# Plot HY03-HY07 model and add HY08 data in red

plot(stg,q,main="FTR HY03-08",xlab="stage (ft)",ylab="discharge (cfs)")

points(stg[sym==8],q[sym==8],col=2,pch=16)

x1 <- seq(0,1.85,0.05)

y1 <- 5.013*x1^3.770

lines(x1,y1,col=1)

x2 <- seq(1.85,6.5,0.05)

y2 <- -290.7 + 172.61*x2 + 6.911*x2^2

lines(x2,y2,col=1)

legend(1.25,950,c("HY03-07","HY08"),col=1:2,pch=c(1,16),lty=c(1,-1))

I don't think two points is strong enough evidence that the rating curve has

shifted.  Instead my strategy will be to fit a single curve to all the data.

A two-part curve may be needed as before.

# Try a power function and a quadratic

fit1 <- nls(q ~ a*stg^b, data=ftrq, start=list(a=4,b=4))

fit2 <- lm(q ~ poly(stg,2), data=ftrq)

plot(stg,q,main="FTR HY03-08",xlab="stage (ft)",ylab="discharge (cfs)")

x0 <- seq(0,7,0.25)

y1 <- predict(fit1,newdata=data.frame(stg=x0))

y2 <- predict(fit2,newdata=data.frame(stg=x0))

lines(x0,y1)

lines(x0,y2,col=2)

legend(locator(1),c("power","quadratic"),lty=1,col=1:2)

# Both are poor fits. Go to 2-part fit as was done for HY03-07

# I arrived at the breakpoints below by trial and error

fit1 <- nls(q ~ a*stg^b, data=ftrq, subset=stg < 2.00, start=list(a=4,b=4))

fit2 <- lm(q ~ stg + I(stg^2), data=ftrq, subset=stg > 2.00)

y1 <- predict(fit1,newdata=data.frame(stg=x0))

y2 <- predict(fit2,newdata=data.frame(stg=x0))

plot(stg,q,main="FTR HY03-08",xlab="stage (ft)",ylab="discharge (cfs)")

lines(x0,y1)

lines(x0,y2,col=2)

# Zoom in

plot(stg,q,main="FTR HY03-08",xlab="stage (ft)",ylab="discharge (cfs)",xlim=c(1.5,2.5),ylim=c(0,200))

lines(x0,y1)

lines(x0,y2,col=2)

Find the intersection of the two models

> fit1

Nonlinear regression model

model:  q ~ a * stg^b

data:  ftrq

a        b

3.243694 4.473371

residual sum-of-squares:  237.304

> fit2

Call:

lm(formula = q ~ stg + I(stg^2), data = ftrq, subset = stg >     2)

Coefficients:

(Intercept)          stg     I(stg^2)

-238.66       132.23        11.79

uniroot(

function(x) 3.243694*x^4.473371 - (-238.66+132.23*x+11.79*x^2),

interval=c(1.5,2)

)$root

[1] 1.960877

# Change of equation will be at 1.9609

x1 <- seq(0,1.96,0.02)

x2 <- seq(1.96,6.5,0.02)

y1 <- predict(fit1,newdata=data.frame(stg=x1))

y2 <- predict(fit2,newdata=data.frame(stg=x2))

plot(stg,q,main="FTR HY03-08",xlab="stage (ft)",ylab="discharge (cfs)",xlim=c(1.5,2.5),ylim=c(0,200))

lines(x1,y1)

lines(x2,y2,col=2)

# Zoom in on intersection with split at 1.96

plot(stg,q,main="FTR HY03-08",xlab="stage (ft)",ylab="discharge (cfs)",type="n")

text(stg,q,sym,cex=0.7)

lines(x1,y1)

lines(x2,y2,col=2)

# Zoom back out

# Equation for HY03 to HY08 will be

Stage < 1.9609:  Q = 3.2437s^4.4734

Stage > 1.9609:  Q = -238.66 + 132.23s + 11.79s^2

# Add old rating equations and legend

x3 <- seq(0,1.85,0.05)

y3 <- 5.013*x3^3.770

x4 <- seq(1.85,6.5,0.05)

y4 <- -290.7 + 172.61*x4 + 6.911*x4^2

x5 <- seq(0.7865,6.2,0.02)

y5 <- 25.328*x5^2 + 18.389*x5 -30

lines(x3,y4,col=3)

lines(x4,y4,col=3)

lines(x5,y5,col=4)

legend(locator(1),c("Q = 3.2437s^4.4734","Q = -238.66 + 132.23s + 11.79s^2","Jack's 03-07 rating equation","Clark's 03 rating equation used by Sarah"), col=1:4,lty=1, cex=0.80)

From this it seems clear that the HY03 loads need to be recomputed using one of the new equations.  To be consistent use Jack's 03-07 equation since that was used for 04 and 05.