KRW04 Event Loads

KRW,401,031212,1600,031213,2100  204528 kg   Very good

KRW,402,031213,2110,031217,0000  612933 kg   Excellent

KRW,403,031228,2010,031231,1430  969025 kg   Good

KRW,404,031231,1440,040105,0000  865311 kg   Good

KRW,405,040216,1010,040217,2350 1496332 kg   Very good

KRW,406,040218,0000,040220,1200  658872 kg   Very good

KRW,407,040225,0120,040229,0000  589534 kg   Good

-----------------------------------------------------

SUM of events                   5396535 kg   Very good

-----------------------------------------------------

krw04.flo <- read.flo("krw",04)

krw04.lab <- read.lab("krw",04)

mismatches("krw",04)

# Many unmatched samples in flo file; none in lab file

krw04.sed <- merge.flo("krw",04)

Storm 401: dump 6: KRW,401,031212,1600,031213,2100

Very good turbidity and good sample coverage

four()

turbsscplot("krw",04,031212,1600,031213,2100)

turbsscplot("krw",04,031212,1600,031213,2100,type="power")

turbsscplot("krw",04,031212,1600,031213,2100,type="logxy")

turbsscplot("krw",04,031212,1600,031213,2100,type="sqrt")

turb1 <- turbsrc("krw",04,031212,1600,031213,2100,type="power")

ttsplot("krw",04,turb1)

total(turb1)

start            end  type surr     yhat n    r2      s    cv

turb1 12/12/03 16:00 12/13/03 21:00 power turb 204528.0 9 0.986 18.575 2.581

Total load = 204528 kg,  CV = 2.58 %  Quality very good

Storm 402: dump 6:  KRW,401,031213,2110,031217,0000

Excellent turbidity and sample coverage; should include bot 1 from dump 7 too

four()

turbsscplot("krw",04,031213,1500,031217,1200)

# Note the hysteresis

turbsscplot("krw",04,031213,1500,031217,1200,type="power")

turbsscplot("krw",04,031213,1500,031214,0350,type="linear")

turbsscplot("krw",04,031214,0400,031217,1200,type="logxy")

turb1 <- turbsrc("krw",04,031213,2110,031217,0000,dump=c(rep(6,14),7),bot=c(11:24,1))

endpoints(turb1)

start       end

55.43519 -33.91862

turb2 <- turbsrc("krw",04,031213,2110,031214,0350,type="linear",dump=6,bot=c(11:15))

turb3 <- turbsrc("krw",04,031214,0400,031217,0000,type="logxy",dump=c(rep(6,9),7),bot=c(16:24,1),bias="qmle")

ttsplot("krw",04,turb2,turb3)

total(turb2,turb3)

start            end       type surr     yhat  n    r2      s    cv

turb2 12/13/03 21:10 12/14/03 03:50     linear turb 234173.3  5 0.998 27.016 1.674

turb3 12/14/03 04:00 12/17/03 00:00 logxy:qmle turb 378759.8 10 0.996  0.091 3.550

Total load = 612933 kg,  CV = 2.29 %  Quality excellent

Storm 403: dump 8: KRW,403,031228,2010,031231,1430

Turbidity had some problems on the recession; tried to reconstruct but

result is speculative and not sure whether to keep spikes;

Sample coverage is good

four()

turbsscplot("krw",04,031228,2010,031231,1430,type="power")

turbsscplot("krw",04,031228,2010,031229,0850)

turbsscplot("krw",04,031228,2010,031229,0850,type="power")

turbsscplot("krw",04,031229,0900,031231,1430,type="power")

turb1 <- turbsrc("krw",04,031228,2010,031231,1430,type="power")

ttsplot("krw",04,turb1)

total(turb1)

start            end  type surr   yhat  n    r2      s    cv

turb1 12/28/03 20:10 12/31/03 14:30 power turb 975232 16 0.949 54.206 3.472

Total load = 975232 kg,  CV = 3.47 %

# Alternative: model the hysteresis

# Not continuous: need to connect with a line segment

turb2 <- turbsrc("krw",04,031228,2010,031229,0830,type="power")

turb3 <- turbsrc("krw",04,031229,1100,031231,1430,type="power")

> tail(turb2$predssc,5)

[1] 1003.868 1028.009 1037.700 1047.409 1071.764

> head(turb3$predssc,5)

[1] 795.3789 769.0383 752.9383 713.0599 691.3573

time1 <- lineartime("krw",04,031229,0840,031229,1050,ssc1=1072,ssc2=820)

total(turb2,time1,turb3)

start            end   type surr     yhat  n    r2      s    cv

turb2 12/28/03 20:10 12/29/03 08:30  power turb 292897.0  4 0.993 33.374 3.422

time1 12/29/03 08:40 12/29/03 10:50 linear time 198244.7  0    NA     NA    NA

turb3 12/29/03 11:00 12/31/03 14:30  power turb 477883.4 12 0.979 28.825 2.964

Total load = 969025 kg   Good

Alternatives differ by less than 1%, but remember a lot of turbidity was reconstructed

Storm 404: dump 9: KRW,404,031231,1440,040105,0000

Had to reconstruct a lot of turbidity, but the main peak is very good;

However, no samples were collected on the rising limb.

four()

turbsscplot("krw",04,031231,1440,040105,0000)

turbsscplot("krw",04,031231,1440,040105,0000,type="power")

turbsscplot("krw",04,031231,1440,040105,0000,type="logxy")

turbsscplot("krw",04,031231,1440,040105,0000,type="loess")

turb1 <- turbsrc("krw",04,031231,1440,040105,0000)

turb2 <- turbsrc("krw",04,031231,1440,040105,0000,type="power")

turb3 <- turbsrc("krw",04,031231,1440,040105,0000,type="logxy",bias="qmle")

turb4 <- turbsrc("krw",04,031231,1440,040105,0000,type="loess")

total(turb1,turb2,turb3,turb4)

start     end       type surr     yhat  n    r2      s    cv

turb1 12/31/03 14:40 1/05/04     linear turb 873964.8 11 0.993 24.418 2.620

turb2 12/31/03 14:40 1/05/04      power turb 894945.3 11 0.989 30.720 2.980

turb3 12/31/03 14:40 1/05/04 logxy:qmle turb 908565.4 11 0.981  0.149 5.907

turb4 12/31/03 14:40 1/05/04      loess turb 865310.7 11 0.995 20.376    NA

# The loess model is the best fit

ttsplot("krw",04,turb4)

total(turb4)

start     end  type surr     yhat  n    r2      s cv

turb4 12/31/03 14:40 1/05/04 loess turb 865310.7 11 0.995 20.376 NA

Total load = 865311 kg   Good

Storm 405: dump 13: KRW,405,040216,1010,040217,2350

Very good turbidity and sample coverage

four()

turbsscplot("krw",04,040216,1010,040217,2350)

turbsscplot("krw",04,dump=13,bot=4:7)

turbsscplot("krw",04,dump=13,bot=8:11)

turbsscplot("krw",04,dump=13,bot=11:15)

turb1 <- turbsrc("krw",04,040216,1010,040217,2350)

endpoints(turb1)

start       end

-32.97149 286.32741

time1 <- lineartime("krw",04,040216,1010,040216,1050,ssc1=6,ssc2=10)

turb2 <- turbsrc("krw",04,040216,1100,040217,2350)

ttsplot("krw",04,time1,turb2)

total(time1,turb2)

start            end   type surr        yhat  n    r2      s    cv

time1 02/16/04 10:10 02/16/04 10:50 linear time      24.219  0    NA     NA    NA

turb2 02/16/04 11:00 02/17/04 23:50 linear turb 1496307.862 12 0.936 83.636 3.812

Total load = 1496332 kg   Very good

Alternative

turb3 <- turbsrc("krw",04,040216,1100,040216,1630)

turb4 <- turbsrc("krw",04,040216,1640,040217,0420)

turb5 <- turbsrc("krw",04,040217,0430,040217,2350,dump=13,bot=11:15)

total(time1,turb3,turb4,turb5)

start            end   type surr       yhat n    r2      s    cv

time1 02/16/04 10:10 02/16/04 10:50 linear time     24.219 0    NA     NA    NA

turb3 02/16/04 11:00 02/16/04 16:30 linear turb  12606.654 4 0.998 16.250 2.730

turb4 02/16/04 16:40 02/17/04 04:20 linear turb 521204.954 4 0.968 38.697 2.420

turb5 02/17/04 04:30 02/17/04 23:50 linear turb 994884.612 5 0.998 15.858 1.477

Total load = 1528720 kg  only 2% higher but very discontinuous

Storm 406: dump 13-14: KRW,406,040218,0000,040220,1200

Turbidity was reconstructed during latter part of recession

Sample coverage is only fair (4 samples): need to include neighboring samples

four()

turbsscplot("krw",04,dump=rep(13:14,c(3,3)),bot=c(15:17,1:4))

turbsscplot("krw",04,dump=rep(13:14,c(2,4)),bot=c(16:17,1:4))

turbsscplot("krw",04,dump=rep(13:14,c(2,4)),bot=c(16:17,1:4),type="power")

turbsscplot("krw",04,dump=rep(13:14,c(2,4)),bot=c(16:17,1:4),type="loess")

turb1 <- turbsrc("krw",04,040218,0000,040220,1200,dump=rep(13:14,c(2,4)),bot=c(16:17,1:4),type="power")

ttsplot("krw",04,turb1)

total(turb1)

start            end  type surr     yhat n    r2      s    cv

turb1 2/18/04 02/20/04 12:00 power turb 658872.2 6 0.996 13.767 2.526

Total load = 658872 kg,  CV = 2.53 %  very good

Storm 407: dump 14: KRW,407,040225,0120,040229,0000

Good turbidity and sample coverage; need to include neighboring sample(s)

turbsscplot("krw",04,dump=rep(14:15,c(7,1)),bot=c(6:12,1))

turb1 <- turbsrc("krw",04,040225,0120,040229,0000,dump=rep(14:15,c(7,1)),bot=c(6:12,1))

ttsplot("krw",04,turb1)

total(turb1)

start     end   type surr     yhat n    r2      s    cv

turb1 02/25/04 01:20 2/29/04 linear turb 589534.1 8 0.973 20.428 4.137

Total load = 589534 kg,  CV = 4.14 %  Quality good