Data corrections FTR06 - Jack Lewis

There are field forms going back to 07Sep2005, but the data I received from Clark start at 01Nov2005

and I have no raw data.  Most if not all of the raw data this year was collected on the FTS data logger and I'm

not sure where those files are. It was a trial of the FTS version of TTS. Clark must have manually assembled

the flo file I started with.  I have kept that file as ftr06.flo.orig.

Rating equation

A new rating equation was computed based on the data from HY03 - HY08.

Because the staff plates were moved in Feb 2006, all stages prior to that date

(in the discharge measurement data set) were augmented by 0.045 and the

rating equation recalc'd.

Stage corrections - Jack Lewis

Stage offset was not changed all year. That makes the corrections simpler.  Here's a plot of OR differences versus time.  The biggest changes of 0.06 are not alarming and do not suggest any data recording errors, with the possible exception of:

12-19-2005 18:20 electronic stage was written on FF as 2.953 whil true estage was 2.836 suggesting a time recording error.  OR diff was 0.054 on this one.

There is no trend from Feb to June:

This apparently calls for a stage shift of +0.048 for latter half of year, but it turns out that both staff plates were replaced in February.  The FFs do not mention any transducer issues.

I'm going to assume that the shift is due to the new staff plates being lower.  The rating equation is based on a mix of staff plate readings taken before and after the staff plates were replaced. The 08 stages are indeed a little higher than expected for a given discharge.  The rating equation that will be applied to years 2006 and later should be recomputed after adding a constant to all earlier staff plate readings. Then the ENTIRE HY06 stage record needs to be shifted upwards to be compatible with the new staff plates.

ftr06.or <- read.or("ftr",06)

Read 18 records

[1] ".\\ftr\\ftr06.flo"

> ftr06.or$diff <- ftr06.or$staff - ftr06.or$raw

chr dump staff rawstg corstg   diff

1  (10/12/05 17:11:00)    1  0.70     NA     NA     NA

2  (10/15/05 11:50:00)    1  0.87     NA     NA     NA

3  (10/17/05 15:40:00)    1  0.70     NA     NA     NA

4  (10/19/05 16:50:00)    1  0.65     NA     NA     NA

5  (10/25/05 16:50:00)    2  0.65     NA     NA     NA

6  (10/29/05 13:20:00)    2  0.83     NA     NA     NA

7  (11/02/05 17:00:00)    2  0.71  0.714  0.714 -0.004

8  (11/06/05 14:00:00)    3  1.98  1.958  1.958  0.022

9  (11/08/05 17:00:00)    3  1.48  1.469  1.469  0.011

10 (11/15/05 16:40:00)    3  1.53  1.522  1.522  0.008

11 (11/19/05 17:40:00)    3  0.93  0.936  0.936 -0.006

12 (11/28/05 16:30:00)    4  1.38  1.372  1.372  0.008

13 (11/30/05 17:00:00)    4  1.65  1.638  1.638  0.012

14 (12/02/05 10:10:00)    5  2.52  2.490  2.490  0.030

15 (12/06/05 17:00:00)    5  1.29  1.307  1.307 -0.017

16 (12/08/05 17:20:00)    5  1.65  1.649  1.649  0.001

17 (12/13/05 17:20:00)    5  1.01  1.017  1.017 -0.007

18 (12/17/05 16:40:00)    6  0.85  0.864  0.864 -0.014

19 (12/19/05 16:40:00)    6  3.00  3.003  3.003 -0.003

20 (12/19/05 18:20:00)    6  2.89  2.836  2.836  0.054

21 (12/20/05 17:00:00)    7  2.80  2.814  2.814 -0.014

22 (12/23/05 11:10:00)    8  2.49  2.457  2.457  0.033

23 (12/27/05 16:20:00)    8  2.04  2.048  2.048 -0.008

24 (12/29/05 17:40:00)    9  2.39  2.383  2.383  0.007

25 (02/26/06 13:00:00)   16  0.83  0.797  0.797  0.033

26 (03/01/06 08:40:00)   16  2.33  2.292  2.292  0.038

27 (03/08/06 17:00:00)   17  2.30  2.240  2.240  0.060

28 (03/12/06 14:00:00)   17  1.85  1.792  1.792  0.058

29 (03/18/06 12:00:00)   18  2.01  1.949  1.949  0.061

30 (03/28/06 18:50:00)   18  1.65  1.597  1.597  0.053

31 (03/29/06 16:50:00)   18  1.56  1.518  1.518  0.042

32 (04/08/06 10:20:00)   19  1.60  1.561  1.561  0.039

33 (04/14/06 17:20:00)   20  1.43  1.380  1.380  0.050

34 (04/26/06 16:40:00)   20  1.24  1.179  1.179  0.061

35 (05/10/06 18:00:00)   21  0.83  0.794  0.794  0.036

36 (06/14/06 14:30:00)   21  0.74     NA     NA     NA

# Compare the mean of the two sets of OR differences after removing the suspect 12/19 OR

> mean(ftr06.or$diff[c(7:19,21:24)],na.rm=T)

[1] 0.003470588

> mean(ftr06.or$diff[c(25:35)],na.rm=T)

[1] 0.04827273

0.0482 - 0.0035

[1] 0.0447

The correction will be +0.045 ft, added to entire hydrograph

Turbidity corrections - Jack Lewis

There are 3 early season turbidity pulses at the very starts of rise in the hydrographs.  They have reasonable shapes but in each case, the peaks were higher than that occurring at the peak of the event.  My guess is that they represent a highly concentrated flush in response to the initial rise in stage.  I coded them as questionable (turbcode 1) but will use them in the annual load estimate.

Interpolated to remove spikes from 28Dec05:1620 to 29Dec05:1420

Removed numerous downspikes (1 data point each) throughout record

Interpolated 18Dec05:1340 to 18Dec05:1700, as suggested by scatterplot