SFM 2005 Annual load estimation

sfm05.flo <- read.flo("sfm",05)

sfm05.lab <- read.lab("sfm",05)

sfm05.sed <- merge.flo("sfm",05)

nonstorms <- drop.storms("sfm",05)

dim(nonstorms)

[1] 44510     4

head(nonstorms, 5)

chr     q turb turbcode

1 (08/15/04 04:40:00) 1.508   NA        2

2 (08/15/04 04:50:00) 1.513   NA        2

3 (08/15/04 05:00:00) 1.513   NA        2

4 (08/15/04 05:10:00) 1.513   NA        2

5 (08/15/04 05:20:00) 1.517   NA        2

table(nonstorms$turbcode)

0     2     5

32557 11294   659

11294 values of bad turbidity out of 44510 intervals to be estimated

But these are nearly all prior to Oct. 18 or after May 28.

trb05.flo <- nonstorms[!is.na(nonstorms$turb), ]

flo05.flo <- nonstorms[is.na(nonstorms$turb), ]

dim(trb05.flo)

[1] 33209     4

dim(flo05.flo)

[1] 11301     4

33209+11301

[1] 44510

# Develop model for good turb data

range(trb05.flo$turb)

[1]   0 925

table(sfm05.sed$labcode2)

0   1   2   3   5   7

274   8   1   1   1   1

# Just a few bad lab codes to worry about

# (5=low volume, 7=major weighing error)

# Eliminate bad turbidity and turbidity > 950 and labcodes 5,7

trb05.sed <- sfm05.sed[!is.na(sfm05.sed$turb) & sfm05.sed$turb < 950 & sfm05.sed$labcode2 < 5, ]

turbsscplot("trb",05)

# Also eliminate the two egregious outliers

trb05.sed[trb05.sed$ssc > 1200 & trb05.sed$turb < 300,]

chr dump bottle  ssc turb turbcode     q labcode1 labcode2

81 (12/29/04 21:00:00)    9      8 1365   62        0 136.7        0        0

98 (01/01/05 04:10:00)   11     10 4067  217        0 507.4        0        0

trb05.sed <- trb05.sed[!(trb05.sed$ssc > 1200 & trb05.sed$turb < 300),]

table(trb05.sed$turbcode)

0   5

267   4

turbsscplot("trb",05,type="power")

turbsscplot("trb",05,type="logxy")

# Try a few different models for comparison

turb1 <- turbsrc("trb",05,040801,0000,050731,2400,type="power",var=F)

turb2 <- turbsrc("trb",05,040801,0000,050731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",05,040801,0000,050731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",05,040801,0000,050731,2400,type="linear",var=F)

start            end       type surr      yhat   n    r2      s cv

turb1 10/18/04 12:40 07/19/05 17:40      power turb 1200209.5 271 0.940 95.590 NA

turb2 10/18/04 12:40 07/19/05 17:40 logxy:mvue turb 1179470.9 271 0.889  0.423 NA

turb3 10/18/04 12:40 07/19/05 17:40      loess turb  815311.7 271 0.941 94.423 NA

turb4 10/18/04 12:40 07/19/05 17:40     linear turb  959823.7 271 0.940 94.955 NA

> sum(turb3$predssc < 0)

[1] 25877

> sum(turb4$predssc < 0)

[1] 24908

# Can't use linear or loess: too many negative predictions

# Tough to choose between the power model or logxy model, but power model fits better at higher concentrations where load is probably greatest

start            end  type surr    yhat   n   r2     s cv

turb1 10/18/04 12:40 07/19/05 17:40 power turb 1200210 271 0.94 95.59 NA

Total load = 1200210 kg

# A relatively good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo05.flo$q)

[1]   1.296 207.200

> range(sfm05.sed$q)

[1]   2.095 1021.000

# Eliminate high discharges to avoid influence on regression

flo05.sed <- sfm05.sed[sfm05.sed$q <= 210, ]

qsscplot("flo",05)

qsscplot("flo",05,type="loess")

flow1 <- flowsrc("flo",05,040801,0000,050731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",05,040801,0000,050731,2400,type="power",var=F)

flow3 <- flowsrc("flo",05,040801,0000,050731,2400,type="loess",var=F)

flow4 <- flowsrc("flo",05,040801,0000,050731,2400,type="loess",degree=2,var=F)

total(flow1,flow2,flow3)

start            end       type surr     yhat   n    r2       s

flow1 08/15/04 04:40 06/11/05 07:00 logxy:mvue flow 67292.20 187 0.513   0.768

flow2 08/15/04 04:40 06/11/05 07:00      power flow 73272.34 187 0.319 225.757

flow3 08/15/04 04:40 06/11/05 07:00      loess flow 81138.15 187 0.317 226.108

flow4 08/15/04 04:40 06/11/05 07:00      loess flow 95442.78 187 0.323 225.949

> sum(flow3$predssc <0)

[1] 0

> sum(flow4$predssc <0)

[1] 0

#I trust the loess fits more than logxy, take the middle estimate

start            end  type surr     yhat   n    r2       s cv

flow3 08/15/04 04:40 06/11/05 07:00 loess flow 81138.15 187 0.317 226.108 NA

Total load = 81138 kg

==================================================

# Total non-storm load

1200210 + 81138 = 1281348 kg = 1281 metric tons

# Total storm load = 8269941 kg

# Total annual load

8270 + 1281 = 9551 metric tons total annual load

# Storm load is 8270/9551 = 86.6% of annual load