KRW 2006 Annual load estimation

krw06.flo <- read.flo("krw",06)

krw06.lab <- read.lab("krw",06)

krw06.sed <- merge.flo("krw",06)

nonstorms <- drop.storms("krw",06)

dim(nonstorms)

[1] 22454     4

head(nonstorms, 5)

chr     q turb turbcode

1 (10/16/05 10:00:00) 17.07    2        0

2 (10/16/05 10:10:00) 16.97    2        0

3 (10/16/05 10:20:00) 16.91    2        0

4 (10/16/05 10:30:00) 16.91    2        0

5 (10/16/05 10:40:00) 16.91    2        0

table(nonstorms$turbcode)

0     2     5     7

12167   266 10019     2

Only 266 values of bad turbidity out of 22454 intervals to be estimated

These were all on the recession limb of a small peak Nov 4-6, 2006

trb06.flo <- nonstorms[!is.na(nonstorms$turb), ]

flo06.flo <- nonstorms[is.na(nonstorms$turb), ]

dim(trb06.flo)

[1] 22188     4

dim(flo06.flo)

[1] 266     4

22188 + 266

[1] 22454

# Develop model for good turb data

range(trb06.flo$turb)

[1]   -1 299

table(krw06.sed$labcode2)

0   1   4

297  15   1

# No bad lab codes to worry about

# (4=sediment spilled, but less than 15%)

# Eliminate bad turbidity

trb06.sed <- krw06.sed[!is.na(krw06.sed$turb), ]

table(trb06.sed$turbcode)

0   5   6

247  62   1

# 5=interpolated, 6=reconstructed

turbsscplot("trb",06,type="loess")

# The outliers are from dump 11.   In the spreadsheet, there is no explanation for the high concentrations, but the turbidity was close to zero at the time and was valid, so they are obviously erroneous.

# Eliminate them

trb06.sed <- krw06.sed[!is.na(krw06.sed$turb) & !(krw06.sed$dump==11 & krw06.sed$bottle %in% 7:9), ]

# Add 1 to eliminate negative turbiditiy

trb06.flo$turb <- trb06.flo$turb+1

# Eliminate turbidity above 400 to reduce influence

trb06.sed <- trb06.sed[trb06.sed$turb < 400, ]

turbsscplot("trb",06)              # r2=0.945, s=41.97

turbsscplot("trb",06,type="loess") # r2=0.946, s=41.67

turbsscplot("trb",06,type="power") # r2=0.946, s=41.52

turbsscplot("trb",06,type="logxy")  # r2=0.961

turb1 <- turbsrc("trb",06,050801,0000,060731,2400,type="power",var=F)

turb2 <- turbsrc("trb",06,050801,0000,060731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",06,050801,0000,060731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",06,050801,0000,060731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat   n    r2      s cv

turb1 10/16/05 10:00 05/10/06 16:20      power turb 1849452 252 0.946 41.527 NA

turb2 10/16/05 10:00 05/10/06 16:20 logxy:mvue turb 1837520 252 0.961  0.239 NA

turb3 10/16/05 10:00 05/10/06 16:20      loess turb 1819907 252 0.946 41.672 NA

turb4 10/16/05 10:00 05/10/06 16:20     linear turb 1788356 252 0.945 41.971 NA

3.4% difference between largest and smallest

> sum(turb1$predssc < 0)

[1] 0

> sum(turb2$predssc < 0)

[1] 0

# Any of the models would work well

# go with logxy as it should fit this non-storm low turbidity data best

total(turb2)

start            end       type surr    yhat   n    r2     s cv

turb2 10/16/05 10:00 05/10/06 16:20 logxy:mvue turb 1837520 252 0.961 0.239 NA

Total load = 1837520 kg

# Looks like a very reliable estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo06.flo$q)

[1] 26.13 64.65

> range(krw06.sed$q)

[1] 18.54 1271.00

krw06.sed <- krw06.sed[!(krw06.sed$dump==11 & krw06.sed$bottle %in% 7:9), ]

qsscplot("krw",06)

The data to be estimated are in a narrow range 26-65cfs = 0.7-1.8 cumecs

# Limit the regression data to between 1 and 4 cumecs

flo06.sed <- krw06.sed[krw06.sed$q < 155, ]

qsscplot("flo",06)

# This curve is noticeably lower than the one based on all the data

# We can actually calculate variance since only 266 data points

flow1 <- flowsrc("flo",06,050801,0000,060731,2400,type="linear")

flow2 <- flowsrc("flo",06,050801,0000,060731,2400,type="loess")

flow3 <- flowsrc("flo",06,050801,0000,060731,2400,type="logxy")

flow4 <- flowsrc("flo",06,050801,0000,060731,2400,type="power")

total(flow1,flow2,flow3,flow4)

start            end       type surr     yhat   n    r2      s      cv

flow1 11/04/05 11:00 11/06/05 07:10     linear flow 1990.692 118 0.157 79.971 114.086

flow2 11/04/05 11:00 11/06/05 07:10      loess flow 3253.328 118 0.167 79.850      NA

flow3 11/04/05 11:00 11/06/05 07:10 logxy:mvue flow 3088.652 118 0.378  0.806  14.385

flow4 11/04/05 11:00 11/06/05 07:10      power flow 2326.794 118 0.166 79.593  48.140

> sum(flow1$predssc <0)

[1] 50

sum(flow2$predssc <0)

[1] 0

#The logxy model has by far the best stats

total(flow3)

start            end       type surr     yhat   n    r2     s     cv

flow3 11/04/05 11:00 11/06/05 07:10 logxy:mvue flow 3088.652 118 0.378 0.806 14.385

Total load = 3089 kg,  CV = 14.39 %

# Not much sediment here

==================================================

# Total non-storm load

1837520 + 3089 = 1840609 kg = 1841 metric tons

# Total storm load = 17240345 kg = 17240 metric tons

# Add 800642 for missing event 608 (estimated from event regression with SFM)

17240345 + 800642 = 18040987 = 18041 metric tons

# Total annual load

18041 + 1841 = 19882 metric tons total annual load

# Storm load is 18041/19882 = 90.7% of annual load