FTR 2004 Annual load estimation

ftr04.flo <- read.flo("ftr",04)

ftr04.lab <- read.lab("ftr",04)

ftr04.sed <- merge.flo("ftr",04)

ftr04.non <- drop.storms("ftr",04)

dim(ftr04.non)

[1] 45366     4

head(ftr04.non, 5)

chr     q turb turbcode

1 (08/01/03 00:20:00) 1.484    3        0

2 (08/01/03 00:30:00) 1.484    3        0

3 (08/01/03 00:40:00) 1.484    4        0

4 (08/01/03 00:50:00) 1.491    3        0

5 (08/01/03 01:00:00) 1.491    3        0

table(ftr04.non$turbcode)

0     1     5     6

37411    60  7848    47

There are no missing values of turbidity; 1=questionable, 5=interpolated, 6=reconstructed

A lot of interpolation was done for sunspikes.

trb04.flo <- ftr04.non[!is.na(ftr04.non$turb) & ftr04.non$turbcode != 2, ]

dim(trb04.flo)

[1] 45366     4

# Develop model for good turb data

range(trb04.flo$turb)

[1]  -1 362

# Eliminate questionable samples from dump 20 and

# Add 2 to get all turbidity in positive territory

trb04.sed <- ftr04.sed[ftr04.sed$dump != 20, ]

trb04.sed$turb <- trb04.sed$turb + 2

trb04.flo$turb <- trb04.flo$turb + 2

range(trb04.flo$turb)

[1]   1 364

range(trb04.sed$turb)

[1]   5 740

table(trb04.sed$labcode2)

0   1   4

219   2   7

# Code 4 = spilled < 15% of sediment mass

two()

turbsscplot("trb",04)

turbsscplot("trb",04,txt="dump")

# Eliminate questionable samples from dumps 10 and 20 and turbidity > 500

trb04.sed <- trb04.sed[!(trb04.sed$dump==10 & trb04.sed$bottle %in% c(4:7,10)), ]

trb04.sed <- trb04.sed[trb04.sed$turb < 500, ]

turbsscplot("trb",04)

turbsscplot("trb",04,type="loess")

turbsscplot("trb",04,type="loess",span=0.8,degree=2)

turbsscplot("trb",04,type="power")

turbsscplot("trb",04,type="logxy")

# Linear goes negative and power doesn't fit lower part well, but

# Try all 4 models

turb1 <- turbsrc("trb",04,030801,0000,040731,2400,var=F)

turb2 <- turbsrc("trb",04,030801,0000,040731,2400,type="loess",span=0.8,degree=2,var=F)

turb3 <- turbsrc("trb",04,030801,0000,040731,2400,type="power",var=F)

turb4 <- turbsrc("trb",04,030801,0000,040731,2400,type="logxy",var=F)

total(turb1,turb2,turb3,turb4)               start            end       type surr     yhat   n    r2      s cv

turb1 08/01/03 00:20 06/30/04 19:00     linear turb 285946.9 213 0.974 36.352 NA

turb2 08/01/03 00:20 06/30/04 19:00      loess turb 278190.3 213 0.981 31.639 NA

turb3 08/01/03 00:20 06/30/04 19:00      power turb 355556.0 213 0.979 33.413 NA

turb4 08/01/03 00:20 06/30/04 19:00 logxy:mvue turb 292141.4 213 0.916  0.500 NA

> sum(turb1$predssc < 0)

[1] 39263

> sum(turb2$predssc < 0)

[1] 0

# Loess is the best fit;  next best would be logxy which is 5% higher

start            end  type surr     yhat   n    r2      s cv

turb2 08/01/03 00:20 06/30/04 19:00 loess turb 278190.3 213 0.981 31.639 NA

Total load = 278190 kg

# This is a very good quality estimate

====================================================================

# Total storm load = 3424594 kg = 3425 metric tons

# Total non-storm load = 278190 kg = 278 metric tons

# total annual load = 3424594 + 278190 = 3702784 kg = 3703 metric tons

# Storm load is 3425/3703 = 92.5% of annual load