FTR 2006 Annual SAND and FINES load estimation

ftr06.flo <- read.flo("ftr",06)

ftr06.lab <- read.sand("ftr",06)

ftr06.sed <- merge.flo.sand("ftr",06)

# Now create the ssc column: it will represent only FINES

ftr06.sed$ssc <- ftr06.sed$totssc - ftr06.sed$sand

# Since it's named ssc, we can use existing functions to calc the fines load

head(ftr06.sed,6)

chr dump bottle  sand totssc turb turbcode     q labcode1 labcode2   ssc

1 (12/18/05 16:00:00)    6      8  14.6  130.9   55        5  17.2        0        0 116.3

2 (12/18/05 17:20:00)    6     10  74.9  238.9  118        0 110.6        0        0 164.0

3 (12/18/05 17:50:00)    6     11 214.7  510.5  210        0 206.9        0        0 295.8

4 (12/18/05 18:20:00)    6     12 290.1  777.5  336        0 286.2        0        0 487.4

5 (12/18/05 20:20:00)    6     13 176.2  646.4  368        0 355.7        0        0 470.2

6 (12/18/05 21:10:00)    6     14 113.7  465.9  280        0 318.1        0        0 352.2

summary(ftr06.sed$ssc)

Min. 1st Qu.  Median    Mean 3rd Qu.    Max.

26.5   248.2   352.2   537.2   594.1  2467.0

dim(ftr06.flo)

[1] 30473     9

table(ftr06.flo$turbcode)

0     1     5     7

29470   173   829     1

Only 173 values of questionable turbidity and none bad out of 30473 intervals to be estimated.  The questionable values are 3 early season turbidity pulses at the very starts of rise in the hydrographs.  They have reasonable shapes but in each case, the peaks were higher than that occurring at the peak of the event.  My guess is that they represent a highly concentrated flush in response to the initial rise in stage.

# No need for a flo06.flo; there are no missing turbidity data

# Develop model for good turb data

range(ftr06.flo$turb)

[1]    0 1615

range(ftr06.sed$turb)

[1]    4 1594

# Referring back to "FTR06 Annual load.rtf", there are no bad lab codes to worry about

table(ftr06.sed$turbcode)

0  5

91  6

# 5=interpolated

turbsscplot("ftr",06,type="loess")

# Looks very good, eliminate the outlier

ftr06.sed <- ftr06.sed[ftr06.sed$ssc < 2400, ]

turbsscplot("ftr",06,txt="dump")

four()

turbsscplot("ftr",06,txt="dump")

turbsscplot("ftr",06,txt="dump",type="power")

turbsscplot("ftr",06,txt="dump",type="logxy")

turbsscplot("ftr",06,txt="dump",type="loess")

# Eliminate the logxy outlier

ftr06.sed <- ftr06.sed[ftr06.sed$ssc > 50, ]

turb1 <- turbsrc("ftr",06,050801,0000,060731,2400,type="power",var=F)

turb2 <- turbsrc("ftr",06,050801,0000,060731,2400,type="logxy",var=F)

turb3 <- turbsrc("ftr",06,050801,0000,060731,2400,type="loess",var=F)

turb4 <- turbsrc("ftr",06,050801,0000,060731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2      s cv

turb1 11/01/05 09:10 05/31/06 23:50      power turb 7080635 95 0.976 67.713 NA

turb2 11/01/05 09:10 05/31/06 23:50 logxy:mvue turb 6940019 95 0.962  0.138 NA

turb3 11/01/05 09:10 05/31/06 23:50      loess turb 6626571 95 0.977 66.162 NA

turb4 11/01/05 09:10 05/31/06 23:50     linear turb 6933943 95 0.976 67.740 NA

# 7% difference between largest and smallest

> sum(turb3$predssc < 0)

[1] 11745

> sum(turb4$predssc < 0)

[1] 3952

# Cannot use linear or loess because of negative predictions

# Power model and logxy differ by only 2% but power model is the best

total(turb1)

start            end  type surr    yhat  n    r2      s cv

turb1 11/01/05 09:10 05/31/06 23:50 power turb 7080635 95 0.976 67.713 NA

Total load = 7080635 kg

# A very reliable estimate for fines

==================================================

# Now compute the total annual load based on this smaller sample

# using only 95 bottles that had sand fractions done

ftr06.sed$ssc <- ftr06.sed$totssc

turb1 <- turbsrc("ftr",06,050801,0000,060731,2400,type="power",var=F)

turb2 <- turbsrc("ftr",06,050801,0000,060731,2400,type="logxy",var=F)

turb3 <- turbsrc("ftr",06,050801,0000,060731,2400,type="loess",var=F)

turb4 <- turbsrc("ftr",06,050801,0000,060731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2       s cv

turb1 11/01/05 09:10 05/31/06 23:50      power turb 10392457 95 0.976 143.648 NA

turb2 11/01/05 09:10 05/31/06 23:50 logxy:mvue turb 10497939 95 0.967   0.154 NA

turb3 11/01/05 09:10 05/31/06 23:50      loess turb  7653543 95 0.973 150.479 NA

turb4 11/01/05 09:10 05/31/06 23:50     linear turb  8878337 95 0.967 165.543 NA

> sum(turb3$predssc < 0)

[1] 27375

> sum(turb4$predssc < 0)

[1] 28202

# Can't use the linear or loess models because they go negative

# The power and logxy modesl agree well, with the power model being superior

total(turb1)

start            end  type surr     yhat  n    r2       s cv

turb1 11/01/05 09:10 05/31/06 23:50 power turb 10392457 95 0.976 143.648 NA

Total load = 10392457 kg

This total compares very well with the previous estimates:

10503896 based on sum of storms and inter-storm loads

10686170 based on one annual regression model using all 273 samples

==================================================

==================================================

Total load 10392457

Total fines 7080635

Total sand  3311822

Percent sand = 31.9%

Percent fines= 68.1%