FTR 2006 Annual load estimation

ftr06.flo <- read.flo("ftr",06)

ftr06.lab <- read.lab("ftr",06)

ftr06.sed <- merge.flo("ftr",06)

nonstorms <- drop.storms("ftr",06)

dim(nonstorms)

[1] 23269     4

head(nonstorms, 5)

chr      q turb turbcode

1 (11/01/05 09:10:00) 0.7985    0        0

2 (11/01/05 09:20:00) 0.7888    0        0

3 (11/01/05 09:30:00) 0.7985    0        0

4 (11/01/05 09:40:00) 0.7985    0        0

5 (11/01/05 09:50:00) 0.7985    0        0

table(nonstorms$turbcode)

0     1     5

22424   173   672

Only 173 values of questionable turbidity and none bad out of 23269 intervals to be estimated.  The questionable values are 3 early season turbidity pulses at the very starts of rise in the hydrographs.  They have reasonable shapes but in each case, the peaks were higher than that occurring at the peak of the event.  My guess is that they represent a highly concentrated flush in response to the initial rise in stage.

trb06.flo <- nonstorms[!is.na(nonstorms$turb), ]

dim(trb06.flo)

[1] 23269     4

# No need for a flo06.flo; there are no missing turbidity data

# Develop model for good turb data

range(trb06.flo$turb)

[1]   0 135

range(ftr06.sed$turb)

[1]    4 1594

table(ftr06.sed$labcode2)

0   1   4

261  14   1

# No bad lab codes to worry about

# (4=sediment spilled, but less than 15%)

table(ftr06.sed$turbcode)

0   5

265  11

# 5=interpolated, 6=reconstructed

trb06.sed <- ftr06.sed

turbsscplot("trb",06,type="loess")

# Looks very good, let's work in the < 200 range to eliminate the outlier and we're only predicting up to 135 NTU

trb06.sed <- ftr06.sed[ftr06.sed$turb < 200, ]

turbsscplot("trb",06,txt="dump")

# Eliminate same outliers as we did for event loads

# Dump 8 bottles 11 and 13, dump 9 bottle 16

trb06.sed <- trb06.sed[!(trb06.sed$turb < 40 & trb06.sed$ssc > 150), ]

turbsscplot("trb",06,txt="dump")

turbsscplot("trb",06,txt="dump",type="power")

turbsscplot("trb",06,txt="dump",type="logxy")

turbsscplot("trb",06,txt="dump",type="loess")

turb1 <- turbsrc("trb",06,050801,0000,060731,2400,type="power",var=F)

turb2 <- turbsrc("trb",06,050801,0000,060731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",06,050801,0000,060731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",06,050801,0000,060731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat   n    r2      s cv

turb1 11/01/05 09:10 05/31/06 23:50      power turb 396013.1 204 0.918 25.943 NA

turb2 11/01/05 09:10 05/31/06 23:50 logxy:mvue turb 430869.6 204 0.860  0.419 NA

turb3 11/01/05 09:10 05/31/06 23:50      loess turb 420789.5 204 0.917 26.183 NA

turb4 11/01/05 09:10 05/31/06 23:50     linear turb 390893.6 204 0.903 28.298 NA

# 10.2% difference between largest and smallest

> sum(turb3$predssc < 0)

[1] 10338

> sum(turb4$predssc < 0)

[1] 13076

# Cannot use linear or loess because of negative predictions

# Hard to choose between power and logxy.

# Both seem to fit well throughout the range

# The logxy might be slightly preferred because of its emphasis on the low end

# 93% of the values to be predicted have turb < 40

total(turb2)

start            end       type surr     yhat   n   r2     s cv

turb2 11/01/05 09:10 05/31/06 23:50 logxy:mvue turb 430869.6 204 0.86 0.419 NA

Total load = 430870 kg

# A fairly reliable estimate

==================================================

# Total non-storm load

430870 kg = 431 metric tons

# Total storm load = 10073026 kg = 10073 metric tons

# Total annual load

10073 + 431 = 10504 metric tons total annual load

# Storm load is 10073/10504 = 95.9% of annual load

==================================================

# For comparison, do the whole year with one model

ftr06.sed <- ftr06.sed[!(ftr06.sed$turb < 40 & ftr06.sed$ssc > 150), ]

ftr06.sed <- ftr06.sed[!ftr06.sed$dump==10 & ftr06.sed$bottle==9), ]

turb0 <- turbsrc("ftr",06,050801,0000,060731,2400,type="power",var=F)

total(turb0)

start            end  type surr     yhat   n    r2       s cv

turb0 11/01/05 09:10 05/31/06 23:50 power turb 10686170 273 0.968 123.273 NA

Total load = 10686170 kg

# This is only 1.7% higher than the sum of storm plus interstorm estimate