SFM07 Event Loads

NOTE: Dump 12 bottles 1-21 and Dump 13 bottles 1-6 had no initial filter weight.

This affects storm 704.  Used average filter weight to estimate SSC.

stn,storm,sdate,stime,edate,etime

SFM,701,061213,0130,061214,2100  131180 kg  Excellent

SFM,702,061225,2230,061229,1200  442620 kg  Excellent

SFM,703,070103,0610,070106,1610  641260 kg  Fair

SFM,704,070210,0500,070213,1200  708815 kg  Very good

SFM,705,070220,1430,070224,1430 3415762 kg  Very good

SFM,706,070224,1440,070227,0800  547597 kg  Very good

SFM,707,070227,0810,070303,1200  319713 kg  Excellent

SFM,708,070326,1000,070329,1200  136985 kg  Very good

-----------------------------------------------------

SUM of events                   6343932 kg  Very good

-----------------------------------------------------

sfm07.flo <- read.flo("sfm",07)

sfm07.lab <- read.lab("sfm",07)

mismatches("sfm",07)

sfm07.sed <- merge.flo("sfm",07)

Storm 701: dump 7: SFM,701,061213,0130,061214,2100

Very good turbidity.  Removed one point spike at peak, probably due to the debris that was noted on the FF 30 minutes later.

Very good sample coverage.

turbsscplot("sfm",07,061213,0130,061214,2100)

That's as good as it gets.

turb1 <- turbsrc("sfm",07,061213,0130,061214,2100)

ttsplot("sfm",07,turb1)

start            end   type surr     yhat n    r2      s    cv

turb1 12/13/06 01:30 12/14/06 21:00 linear turb 131179.9 8 0.995 12.998 3.092

Total load = 131180 kg,  CV = 3.09 %   Excellent

Storm 702: dumps 8-9: SFM,702,061225,2230,061229,1200

Two peaks of similar size (9.5-10.5 ft)

Excellent turbidity

Very good sample coverage but missed rise of second peak (out of bottles)

four()

turbsscplot("sfm",07,061225,2230,061229,1200)

turbsscplot("sfm",07,061225,2230,061229,1200,type="power")

turbsscplot("sfm",07,061225,2230,061229,1200,type="logxy")

turbsscplot("sfm",07,061225,2230,061229,1200,type="sqrt")

Excellent fit

turb1 <- turbsrc("sfm",07,061225,2230,061229,1200)

ttsplot("sfm",07,turb1)

total(turb1)

start            end   type surr     yhat  n    r2      s    cv

turb1 12/25/06 22:30 12/29/06 12:00 linear turb 439844.4 20 0.983 44.629 4.621

Total load = 439844 kg,  CV = 4.62 %

# Looks good except for negatives at start

turb2 <- turbsrc("sfm",07,061225,2230,061229,1200,type="power")

turb3 <- turbsrc("sfm",07,061225,2230,061229,1200,type="logxy",bias="qmle")

total(turb2,turb3)

start            end       type surr     yhat  n    r2      s    cv

turb2 12/25/06 22:30 12/29/06 12:00      power turb 447980.7 20 0.983 44.871 3.522

turb3 12/25/06 22:30 12/29/06 12:00 logxy:qmle turb 442619.5 20 0.992  0.096 2.241

ttsplot("sfm",07,turb3)

total(turb3)

start            end       type surr     yhat  n    r2     s    cv

turb3 12/25/06 22:30 12/29/06 12:00 logxy:qmle turb 442619.5 20 0.992 0.096 2.241

Total load = 442620 kg,  CV = 2.24 %  Excellent

Storm 703: dump 9-10: SFM,703,070103,0610,070106,1610

Two peaks, second one much smaller.

Excellent turbidity except last 4 hours missing

Good sample coverage for the first 12 hrs (including biggest turb peak), but no samples for 34 hrs after that, except 1 at tail end.  No samples at highest flows.

four()

turbsscplot("sfm",07,070103,0610,070106,1610)

turbsscplot("sfm",07,dump=8:10)

turbsscplot("sfm",07,dump=7:12)

turbsscplot("sfm",07,dump=7:12,type="logxy")

The plots show that the relationship has been fairly stable during dumps 7-12

(Dec 12 to Feb 10) and very stable for dumps 8-9 (Dec 19 to Jan 3). Sample 1 from dump 10 is an outlier with SSC=144 at turbidity=62.  It is an outlier in the annual data set as well, and it should not be given too much weight.

turb1 <- turbsrc("sfm",07,070103,0610,070106,1210)

# Need a discharge model for the last four hrs

four()

qsscplot("sfm",07,070103,0610,070106,1610)

qsscplot("sfm",07,dump=8)

qsscplot("sfm",07,dump=9)

qsscplot("sfm",07,dump=rep(8:9,c(7,9)),bot=c(1,7:10,23,24,1:8,10))

The last model combines the recession data from dumps 8 and 9

flow1 <- flowsrc("sfm",07,070106,1220,070106,1610,dump=rep(8:9,c(7,9)),bot=c(1,7:10,23,24,1:8,10))

ttsplot("sfm",07,turb1,flow1)

total(turb1,flow1)

flow1 and turb1 aren't continuous at all.  Sample 1 says last 4 hrs should be higher, while the flow models says lower.  The last four hours of turb 1 are about constant.  Let's just assume SSC stays there at 78 mg/L for another 4 hours.

tail(turb1$predssc,24)

[1] 78.44581 77.34431 79.54731 78.44581 78.44581 77.34431 78.44581 77.34431 77.34431

[10] 78.44581 79.54731 78.44581 79.54731 78.44581 76.24282 77.34431 76.24282 76.24282

[19] 77.34431 78.44581 77.34431 78.44581 77.34431 78.44581

time1 <- lineartime("sfm",07,070106,1220,070106,1610,ssc1=78,ssc2=78)

ttsplot("sfm",07,turb1,time1)

total(turb1,time1)

start            end   type surr       yhat  n    r2      s    cv

turb1 01/03/07 06:10 01/06/07 12:10 linear turb 637289.831 12 0.987 49.169 6.257

time1 01/06/07 12:20 01/06/07 16:10 linear time   3970.649  0    NA     NA    NA

Total load = 641260 kg    Quality fair

Storm 704: dump 11-12: SFM,704,070210,0500,070213,1200

Turbidity missing until 070210:1410

After that turbidity and sample coverage is excellent

Missing initial filter weights on most samples.

Used average filter weight of 0.1163 for all samples

# First look at effect of estimating filter weights on variation

plot(turb,ssc,main="SFM HY07")

points(turb[labcode1==1],ssc[labcode1==1],col=2,pch=16)

legend(100,2300,c("filter wt known","filter wt estimated"),pch=c(1,16),col=1:2)

# Look at this with log axes

plot(turb,ssc,main="SFM HY07",xlim=c(10,1000),ylim=c(10,1000),log="xy")

points(turb[labcode1==1],ssc[labcode1==1],col=2,pch=16)

legend(10,1000,c("filter wt known","filter wt estimated"),pch=c(1,16),col=1:2)

# Estimating filter weight looks like it didn't introduce much error

four()

qsscplot("sfm",07,070209,1200,070210,2000)

turbsscplot("sfm",07,070210,0500,070213,1200)

turbsscplot("sfm",07,dump=12,bot=1:7)

turbsscplot("sfm",07,dump=12,bot=7:21)

# Unknown filter weights did not add much to the variance

flow1 <- flowsrc("sfm",07,070210,0500,070210,1400,dump=c(10,11),bot=c(4,1))

turb1 <- turbsrc("sfm",07,070210,1410,070210,2020,dump=12,bot=1:7)

turb2 <- turbsrc("sfm",07,070210,2030,070213,1200)

ttsplot("sfm",07,flow1,turb1,turb2)

total(flow1,turb1,turb2)

start            end       type surr      yhat  n    r2      s    cv

flow1 02/10/07 05:00 02/10/07 14:00 logxy:mvue flow  13568.30  2 1.000  0.000 0.000

turb1 02/10/07 14:10 02/10/07 20:20     linear turb 105529.65  7 0.997 46.504 2.304

turb2 02/10/07 20:30 02/13/07 12:00     linear turb 589716.76 15 0.998 35.110 3.081

Total load = 708815 kg,  CV = 2.59 %  Very good

Storm 705: dump 14-15: SFM,705,070220,1430,070224,1430

One very large peak

Excellent turbidity and sample coverage!

four()

turbsscplot("sfm",07,070220,1430,070224,1430)

turbsscplot("sfm",07,070220,1430,070224,1430,type="power")

turbsscplot("sfm",07,070220,1430,070224,1430,type="logxy")

turbsscplot("sfm",07,070220,1430,070224,1430,type="loess")

Except for the small turbidity peak between samples 17 and 18, we could do an excellent job this time just using lineartime.

time1 <- lineartime("sfm",07,070220,1430,070224,1430)

turb1 <- turbsrc("sfm",07,070220,1430,070224,1430)

turb2 <- turbsrc("sfm",07,070220,1430,070224,1430,type="power")

turb3 <- turbsrc("sfm",07,070220,1430,070224,1430,type="logxy",bias="qmle")

turb4 <- turbsrc("sfm",07,070220,1430,070224,1430,type="loess")

total(time1,turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2       s    cv

time1 02/20/07 14:30 02/24/07 14:30     linear time 3365168 29    NA      NA    NA

turb1 02/20/07 14:30 02/24/07 14:30     linear turb 3575324 29 0.965 141.382 2.993

turb2 02/20/07 14:30 02/24/07 14:30      power turb 3502956 29 0.970 131.560 2.974

turb3 02/20/07 14:30 02/24/07 14:30 logxy:qmle turb 3559585 29 0.968   0.207 4.750

turb4 02/20/07 14:30 02/24/07 14:30      loess turb 3563409 29 0.969 134.486    NA

# Range of estimates is 6%, but lineartime may be too low because of truncation

ttsplot("sfm",07,turb3)

# Handle the peak between bottle 17 and 18 separately

time2 <- lineartime("sfm",07,070220,1430,070221,1240,ssc1=18)

time3 <- lineartime("sfm",07,070221,1650,070224,1430,ssc2=121)

turb5 <- turbsrc("sfm",07,070221,1250,070221,1640,dump=14,bot=15:19)

ttsplot("sfm",07,time2,time3,turb5)

# Did not raise it as much as expected.

# While turb3 overestimated slightly around the main storm peak

# Go with the hybrid linear time approach

total(time2,time3,turb5)

start            end   type surr      yhat  n    r2      s    cv

time2 02/20/07 14:30 02/21/07 12:40 linear time  494688.1 17    NA     NA    NA

turb5 02/21/07 12:50 02/21/07 16:40 linear turb  665085.7  5 0.987 66.094 1.666

time3 02/21/07 16:50 02/24/07 14:30 linear time 2255988.4 12    NA     NA    NA

Total load = 3415762 kg  Excellent

# 4% lower than power estimate

Storm 706: dump 15: SFM,706,070224,1440,070227,0800

Excellent turbidity and sample coverage

four()

# Include bottle 6 before defined start

turbsscplot("sfm",07,dump=15,bot=6:23)

# Bottle 21 had code 4 spillage, should be excluded

turbsscplot("sfm",07,dump=15,bot=c(6:20,22:23))

turbsscplot("sfm",07,dump=15,bot=c(6:20,22:23),type="power")

turbsscplot("sfm",07,dump=15,bot=c(6:20,22:23),type="loess")

turb1 <- turbsrc("sfm",07,070224,1440,070227,0800,dump=15,bot=c(6:20,22:23))

turb2 <- turbsrc("sfm",07,070224,1440,070227,0800,dump=15,bot=c(6:20,22:23),type="power")

time1 <- lineartime("sfm",07,070224,1440,070227,0800,ssc1= 150,ssc2=100)

total(turb1,turb2,time1)

start            end   type surr     yhat  n    r2      s    cv

turb1 02/24/07 14:40 02/27/07 08:00 linear turb 546245.7 17 0.965 40.214 3.749

turb2 02/24/07 14:40 02/27/07 08:00  power turb 547597.3 17 0.970 37.444 3.173

time1 02/24/07 14:40 02/27/07 08:00 linear time 565869.6 17    NA     NA    NA

# Note the latter includes bogus bottle 21, so estimate should be even higher

ttsplot("sfm",07,turb2)

total(turb2)

start            end  type surr     yhat  n   r2      s    cv

turb2 02/24/07 14:40 02/27/07 08:00 power turb 547597.3 17 0.97 37.444 3.173

Total load = 547597 kg,  CV = 3.17 %  Excellent

# 4% less than linear time estimate

Storm 707: dump 15-16: SFM,707,070227,0810,070303,1200

Very good turbidity and very good sample coverage

four()

turbsscplot("sfm",07,070227,0810,070303,1200)

turbsscplot("sfm",07,070227,0810,070303,1200,type="power")

turbsscplot("sfm",07,070227,0810,070303,1200,type="logxy")

turbsscplot("sfm",07,070227,0810,070303,1200,type="loess")

turb1 <- turbsrc("sfm",07,070227,0810,070303,1200)

turb2 <- turbsrc("sfm",07,070227,0810,070303,1200,type="power")

turb3 <- turbsrc("sfm",07,070227,0810,070303,1200,type="logxy",bias="qmle")

turb4 <- turbsrc("sfm",07,070227,0810,070303,1200,type="loess")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat n    r2      s    cv

turb1 02/27/07 08:10 03/03/07 12:00     linear turb 320012.6 8 0.987 11.029 2.756

turb2 02/27/07 08:10 03/03/07 12:00      power turb 321967.7 8 0.984 12.071 2.967

turb3 02/27/07 08:10 03/03/07 12:00 logxy:qmle turb 319713.5 8 0.992  0.053 1.963

turb4 02/27/07 08:10 03/03/07 12:00      loess turb 317986.9 8 0.992  9.389    NA

# Range of estimates is 1.3%

ttsplot("sfm",07,turb3)

total(turb3)

start            end       type surr     yhat n    r2     s    cv

turb3 02/27/07 08:10 03/03/07 12:00 logxy:qmle turb 319713.5 8 0.992 0.053 1.963

Total load = 319713 kg,  CV = 1.96 %  Excellent

Storm 708: dump 17: SFM,708,070326,1000,070329,1200

Excellent turbidity and sample coveage

four()

turbsscplot("sfm",07,070326,1000,070329,1200)

turbsscplot("sfm",07,070326,1000,070329,1200,type="logxy")

turbsscplot("sfm",07,dump=17,bot=6:10,type="loess")

turbsscplot("sfm",07,dump=17,bot=12:16)

turb1 <- turbsrc("sfm",07,070326,1000,070329,1200,type="logxy",bias="qmle")

total(turb1)

start            end       type surr     yhat  n   r2    s    cv

turb1 03/26/07 10:00 03/29/07 12:00 logxy:qmle turb 133933.4 11 0.98 0.15 4.867

Total load = 133933 kg,  CV = 4.87 %

turb2 <- turbsrc("sfm",07,070326,1000,070327,0830,type="loess")

turb3 <- turbsrc("sfm",07,070327,1640,070329,1200,dump=17,bot=12:16)

endpoints(turb2)

start       end

9.09223 246.90028

time1 <- lineartime("sfm",07,070327,0840,070327,1630,ssc1=247)

ttsplot("sfm",07,turb2,turb3,time1)

total(turb2,turb3,time1)

start            end   type surr     yhat n    r2     s    cv

turb2 03/26/07 10:00 03/27/07 08:30  loess turb 14803.10 5 0.999 5.940    NA

time1 03/27/07 08:40 03/27/07 16:30 linear time 47934.60 2    NA    NA    NA

turb3 03/27/07 16:40 03/29/07 12:00 linear turb 74247.40 5 0.996 9.302 4.333

Total load = 136985 kg  Very good

# This is just 2% higher than turb1