Data corrections HHB07 - Jack Lewis

Lab data corrections

Dump 9: filter weights lost, estiamted as average of 276 filters, assigned labcode 10

Dump 6 bottle 16 is obviously erroneous at 1792 mg/L, water weight was only 4 grams

(I've changed the SSC in hhb07.isc to NA)

Dump 11 bottles 1 and 12 changed from 0 mg/L in hhb07.isc to NA

Observer record corrections – Jack Lewis

OR data entered from field forms by JL

Rating equation

Use Jack's 2 part rating based on HY04-08

Stage corrections - Jack Lewis

plot.or("hhb",07)

plot.or("hhb",07,reread=F,xaxis="stage",linear=T)

Although there is a trend with stage it would be dangerous to extrapolate this relation

to the stages of 10 and 11 feet that occurred in HY07.  Stick with time-variable shifts.

04Nov06:1330 to 11Nov06:1320 variable shift 0.00 to +0.01

11Nov06:1320 to 14Dec06:1500 constant shift of +0.01

31Dec06:1040 to 07Jan07:1300 variable shift 0.00 to -0.01

07Jan07:1300 to 19May07:1510 constant shift of -0.01

19May07:1510 to

plot.or("hhb",07,cor=T)

As these last 3 differences are isolated values and all are at stages above 2.5 feet, I'll attribute the differences to difficulty in reading the staff plate accurately. No further

stage adjustments made.

Stage anomalies:

29apr2007:0830 to 30Apr2007:0930 interpolated

Many stage anomalies from May 1 to end of record left alone.

Turbidity corrections - Jack Lewis

Removed 4 spikes from 30Oct2006 to 05Nov2006

10Nov2006:2240 to 11Nov2006:0100 questionable

12Nov2006:1420 to 13Nov2006:1730 interpolated apparently fouled turbidity

10Jan2007:0900-0940 removed spike

10Feb2007:0350-0420 removed spike

22Feb2007:0650-0720 removed spike