# Functions and commands for creating graphs and stats on 10-minute SSC data

read.ssc <-

function(stn,hy,path="./tenminssc")

{

fname <- paste(stn,zfill(hy,2),"ssc.txt",sep="")

pathname <- paste(path,fname,sep="/")

data <- read.table(pathname,header=T,as.is=T)

data$chr <- chron(data$DATE,paste(data$TIME,”00”,sep=”:”))

data

}

sumdays <- function(months,leap) {

days <- c(31,28,31,30,31,30,31,31,30,31,30,31)

if (leap) days[2] <- 29

sum(days[months])

}

ssc.mycdf <- function(x,title,months,log="x",zero=0.1,leap=F,interval=10,grid="days",...) {

# Plot the annual CDF of SSC at a station

# Results are normalized for a year consisting only of specified months

# if log=T plot CDF of log(x), replacing any zero or negative values with

#    the value given by 'replace' argument

if (any(is.na(x))) stop("Missing values not allowed")

if (log=="x") {

if (any(x <= 0)) {

if (any(x < 0))

cat(sum(x < 0),"negative values replaced by",zero,"\n")

if (any(x == 0))

cat(sum(x == 0),"zero values replaced by",zero,"\n")

x[x <= 0] <- zero

}

}

par(mar=c(5,4,4,4) + 0.1, mgp=c(2.5,0.75,0))

y1lab <- "Proportion exceeding given SSC"

xsort <- sort(x)

nx <- length(x)

daysperyear <- sumdays(months,leap)

px <- nx/daysperyear/(1440/interval)

if (px > 1)

stop("Do not run with more than one year of data")

y <- (1-(1:nx)/nx) * px

plot(xsort, y, xlab="SSC (mg/L)", ylab=y1lab, log=log, ...)

title(title)

y2range <- daysperyear * par()$usr[3:4]

y2ticlab <- pretty(y2range)

y2tic <- y2ticlab/daysperyear

axis(4,at=y2tic,label=y2ticlab,las=2)

mtext(side=4,outer=T,at=0.5,text="Days exceeding given SSC",line=-1.75)

if (grid == "days") {

axis(1,tck=1,fg="gray")

axis(4,tck=1,fg="gray",at=y2tic,label=F)

}

else if (grid == "pro") {

axis(1,tck=1,fg="gray")

axis(2,tck=1,fg="gray")

}

box()

}

ssc.mycdfs <- function(xlist,title,months,log="x",zero=0.1,leap=F,interval=10,grid="days",...) {

# Plot the annual CDF of SSC at one or more stations for a given year

# x will be a list if several stations are to be plotted

# names of components will appear in legend

# If less than a year of data, assume anthing not measured has lower SSC

# if log=T plot CDF of log(x), replacing any zero or negative values with

#    the value given by 'replace' argument

if (!is.list(xlist))

ssc.mycdf(xlist,title,log,zero,leap,interval,...)

y1lab <- "Proportion exceeding given SSC"

par(mar=c(5,4,4,4) + 0.1, mgp=c(2.5,0.75,0))

nxx <- sapply(xlist,length)

daysperyear <- sumdays(months,leap)

pxx <- nxx/daysperyear/(1440/interval)

if (any(pxx > 1))

stop("Do not run with more than one year of data")

xmin <- min(sapply(xlist,min))

xmax <- max(sapply(xlist,max))

if (log=="x")

xmin <- max(zero,xmin)

plot(c(xmin,xmax), c(0,max(pxx)), type="n", xlab="SSC (mg/L)",

ylab=y1lab, log=log, ...)

title(title)

y2range <- daysperyear * par()$usr[3:4]

y2ticlab <- pretty(y2range)

y2tic <- y2ticlab/daysperyear

axis(4,at=y2tic,label=y2ticlab,las=2)

mtext(side=4,outer=T,at=0.5,text="Days exceeding given SSC",line=-1.75)

if (grid == "days") {

axis(1,tck=1,fg="gray")

axis(4,tck=1,fg="gray",at=y2tic,label=F)

}

else if (grid == "pro") {

axis(1,tck=1,fg="gray")

axis(2,tck=1,fg="gray")

}

box()

xnames <- names(xlist)

for (i in 1:length(xlist)) {

x <- xlist[[i]]

nx <- nxx[i]

px <- pxx[i]

xname <- xnames[i]

if (log=="x") {

if (any(x <= 0)) {

if (any(x < 0))

cat(sum(x < 0),"negative values of",xname,"replaced by",zero,"\n")

if (any(x == 0))

cat(sum(x == 0),"zero values of",xname,"replaced by",zero,"\n")

x[x <= 0] <- zero

}

}

xsort <- sort(x)

y <- (1-(1:nx)/nx) * px

lines(xsort,y,col=i)

}

legend(locator(1),xnames,lty=1,col=1:length(xnames))

}

# Determine start and end dates

sdate <- edate <- character(22)

i <- 0

for (stn in c(“hhb”,“ftr”,”sfm”,”krw”))

for (hy in 3:8)

if (!(stn==”hhb” & hy %in% 3:4)) {

i <- i + 1

tmp <- read.ssc(stn,hy)$DATE

sdate[i] <- tmp[1]

edate[i] <- last.val(tmp)

names(sdate)[i] <- names(edate)[i] <- paste(stn,zfill(hy,2))

}

dates(sdate)

hhb 08   hhb 06   hhb 07   hhb 08   ftr 03   ftr 04   ftr 08   ftr 06   ftr 07

11/26/04 10/08/08 10/17/06 10/20/07 11/20/02 08/01/03 08/01/04 11/01/08 10/29/06

ftr 08   sfm 03   sfm 04   sfm 08   sfm 06   sfm 07   sfm 08   krw 03   krw 04

09/18/07 11/16/02 08/30/03 08/15/04 11/02/08 09/20/06 08/12/07 11/16/02 08/17/03

krw 08   krw 06   krw 07   krw 08

09/26/04 10/16/08 09/16/06 08/19/07

dates(edate)

hhb 08   hhb 06   hhb 07   hhb 08   ftr 03   ftr 04   ftr 08   ftr 06   ftr 07

07/16/08 08/10/06 06/10/07 08/04/08 07/31/03 06/30/04 07/16/08 08/31/06 06/10/07

ftr 08   sfm 03   sfm 04   sfm 08   sfm 06   sfm 07   sfm 08   krw 03   krw 04

08/04/08 07/31/03 07/01/04 07/19/08 08/10/06 06/09/07 04/22/08 07/31/03 07/01/04

krw 08   krw 06   krw 07   krw 08

07/19/08 08/10/06 04/22/07 06/24/08

Latest start date is Nov 26

Earliest end date is Apr 22, several end in early May

Define water year as Dec. 1 – Apr 30 for this analysis

# Assign 10-min SSC to objects

i <- 0

for (stn in c(“hhb”,“ftr”,”sfm”,”krw”))

for (hy in 3:8) {

sdate <- paste(12,1,hy-1,sep=”/”)

schr <- chron(sdate,”00:00:00”)

edate <- paste(4,30,hy,sep=”/”)

echr <- chron(edate,“23:50:00”)

if (!(stn==”hhb” && hy %in% 3:4)) {

i <- i + 1

tmp1 <- read.ssc(stn,hy)

tmp2 <- tmp1[tmp1$chr >= schr & tmp1$chr <= echr, ]

assign(paste(stn,zfill(hy,2),sep=””),tmp2)

}

}

We now have objects ftr03-ftr08, sfm03-sfm08, krw03-krw08, hhb08-hhb08,

containing only data between Dec. 1 and Apr 30

xlist <- list(sfm=sfm03$SSC, krw=krw03$SSC, ftr=ftr03$SSC)

ssc.mycdfs(xlist,”HY 03”,months=c(12,1:4), xlim=c(1,4000),leap=F)

xlist <- list(sfm=sfm04$SSC, krw=krw04$SSC, ftr=ftr04$SSC)

ssc.mycdfs(xlist,”HY 04”,months=c(12,1:4),xlim=c(1,2000),leap=T)

xlist <- list(sfm=sfm08$SSC, krw=krw08$SSC, ftr=ftr08$SSC, hhb=hhb08$SSC)

ssc.mycdfs(xlist,”HY 08”,months=c(12,1:4),xlim=c(1,2000),leap=F)

xlist <- list(sfm=sfm06$SSC, krw=krw06$SSC, ftr=ftr06$SSC, hhb=hhb06$SSC)

ssc.mycdfs(xlist,”HY 06”,months=c(12,1:4),xlim=c(1,2000),leap=F)

xlist <- list(sfm=sfm07$SSC, krw=krw07$SSC, ftr=ftr07$SSC, hhb=hhb07$SSC)

ssc.mycdfs(xlist,”HY 07”,months=c(12,1:4),xlim=c(1,2000),leap=F)

xlist <- list(sfm=sfm08$SSC, krw=krw08$SSC, ftr=ftr08$SSC, hhb=hhb08$SSC)

ssc.mycdfs(xlist,”HY 08”,months=c(12,1:4),xlim=c(1,2000),leap=T)

# Function to extract exceedence levels and maximum continuous exposures

exceed <- function(x, plevels=c(0.001,0.01,0.02,0.05,0.10),

ssclevels=c(25,50,100,200,500,1000), interval=10, months, leap) {

daysperyear <- sumdays(months,leap)

nx <- length(x)

px <- nx/daysperyear/(1440/interval)

if (any(px > 1))

stop("Period of x exceeds specified number of days per year")

q <- quantile(x,1-plevels/px)

names(q) <- plevels

counts <- sapply(ssclevels, function(ssc, data) sum(x>ssc), data = x)

hrs <- counts*interval/60

names(hrs) <- ssclevels

# Create matrix of logicals for exceeding each SSC level

xlogical <- sapply(ssclevels, function(level, x) x >= level, x=x)

# Determine run lengths for each

runlen <- apply(xlogical, 2, rle)

# Extract maximum run length for TRUE (x >= level) in each component

max.hrs <- sapply(runlen, function(rln) {

true.length <- rln$lengths[rln$values]

if (length(true.length) == 0)

return(0)

else

return(max(rln$lengths[rln$values])*interval/60)

})

names(max.hrs) <- ssclevels

list(ssc.exceeded.at.given.p = q, hours.exceeded.at.given.ssc= hrs,

max.continuous.hours = max.hrs)

}

ftrstats03 <- exceed(ftr03$SSC, months=c(12,1:4), leap=F)

ftrstats04 <- exceed(ftr04$SSC, months=c(12,1:4), leap=T)

ftrstats05 <- exceed(ftr05$SSC, months=c(12,1:4), leap=F)

ftrstats06 <- exceed(ftr06$SSC, months=c(12,1:4), leap=F)

ftrstats07 <- exceed(ftr07$SSC, months=c(12,1:4), leap=F)

ftrstats08 <- exceed(ftr08$SSC, months=c(12,1:4), leap=T)

sfmstats03 <- exceed(sfm03$SSC, months=c(12,1:4), leap=F)

sfmstats04 <- exceed(sfm04$SSC, months=c(12,1:4), leap=T)

sfmstats05 <- exceed(sfm05$SSC, months=c(12,1:4), leap=F)

sfmstats06 <- exceed(sfm06$SSC, months=c(12,1:4), leap=F)

sfmstats07 <- exceed(sfm07$SSC, months=c(12,1:4), leap=F)

sfmstats08 <- exceed(sfm08$SSC, months=c(12,1:4), leap=T)

krwstats03 <- exceed(krw03$SSC, months=c(12,1:4), leap=F)

krwstats04 <- exceed(krw04$SSC, months=c(12,1:4), leap=T)

krwstats05 <- exceed(krw05$SSC, months=c(12,1:4), leap=F)

krwstats06 <- exceed(krw06$SSC, months=c(12,1:4), leap=F)

krwstats07 <- exceed(krw07$SSC, months=c(12,1:4), leap=F)

krwstats08 <- exceed(krw08$SSC, months=c(12,1:4), leap=T)

hhbstats05 <- exceed(hhb05$SSC, months=c(12,1:4), leap=F)

hhbstats06 <- exceed(hhb06$SSC, months=c(12,1:4), leap=F)

hhbstats07 <- exceed(hhb07$SSC, months=c(12,1:4), leap=F)

hhbstats08 <- exceed(hhb08$SSC, months=c(12,1:4), leap=T)

ssc.ex03 <- do.call(“rbind”,list(sfm03=sfmstats03$ssc,krw03=krwstats03$ssc,ftr03=ftrstats03$ssc))

ssc.ex04 <- do.call(“rbind”,list(sfm04=sfmstats04$ssc,krw04=krwstats04$ssc,ftr04=ftrstats04$ssc))

ssc.ex05 <- do.call(“rbind”,list(sfm05=sfmstats05$ssc,krw05=krwstats05$ssc,ftr05=ftrstats05$ssc,hhb05=hhbstats05$ssc))

ssc.ex06 <- do.call(“rbind”,list(sfm06=sfmstats06$ssc,krw06=krwstats06$ssc,ftr06=ftrstats06$ssc,hhb06=hhbstats06$ssc))

ssc.ex07 <-

do.call(“rbind”,list(sfm07=sfmstats07$ssc,krw07=krwstats07$ssc,ftr07=ftrstats07$ssc,hhb07=hhbstats07$ssc))

ssc.ex08 <-

do.call(“rbind”,list(sfm08=sfmstats08$ssc,krw08=krwstats08$ssc,ftr08=ftrstats08$ssc,hhb08=hhbstats08$ssc))

ssc.exall <- do.call(“rbind”,list(ssc.ex03,ssc.ex04,ssc.ex05,ssc.ex06,ssc.ex07,ssc.ex08))

round(ssc.exall)

0.001 0.01 0.02 0.05 0.1

sfm03  3738 2114 1589  688 424

krw03  1861 1238  963  466 242

ftr03  2122 1052  716  246 109

sfm04  1924  796  579  260  98

krw04  1106  585  400  200  88

ftr04  1333  557  331  116  48

sfm05  2172 1061  680  329 194

krw05  1283  523  373  212 123

ftr05  1330  433  276  106  54

hhb05   852  374  253  125  64

sfm06  2709 1812 1350  866 579

krw06  2076 1093  812  337 285

ftr06  3239  897  541  233 118

hhb06  2080  787  498  225 120

sfm07  2059  747  476  242 132

krw07   999  538  369  174  94

ftr07   816  372  195   87  44

hhb07   603  278  183   99  69

sfm08  2203  755  495  264 143

krw08   968  409  275  148  89

ftr08  1356  297  178  100  58

hhb08  1141  246  153   91  59

hours.ex03 <- do.call(“rbind”,list(sfm03=sfmstats03$hours,krw03=krwstats03$hours,ftr03=ftrstats03$hours))

hours.ex04 <- do.call(“rbind”,list(sfm04=sfmstats04$hours,krw04=krwstats04$hours,ftr04=ftrstats04$hours))

hours.ex05 <- do.call(“rbind”,list(sfm05=sfmstats05$hours,krw05=krwstats05$hours,ftr05=ftrstats05$hours,hhb05=hhbstats05$hours))

hours.ex06 <- do.call(“rbind”,list(sfm06=sfmstats06$hours,krw06=krwstats06$hours,ftr06=ftrstats06$hours,hhb06=hhbstats06$hours))

hours.ex07 <-

do.call(“rbind”,list(sfm07=sfmstats07$hours,krw07=krwstats07$hours,ftr07=ftrstats07$hours,hhb07=hhbstats07$hours))

hours.ex08 <-

do.call(“rbind”,list(sfm08=sfmstats08$hours,krw08=krwstats08$hours,ftr08=ftrstats08$hours,hhb08=hhbstats08$hours))

hours.exall <- do.call(“rbind”,list(hours.ex03,hours.ex04,hours.ex05,hours.ex06,hours.ex07,hours.ex08))

round(hours.exall,1)

25     50    100    200   500  1000

sfm03 2966.8 2310.5 1388.5  744.8 278.7 115.8

krw03 2178.8 1249.0  795.2  453.5 168.2  65.7

ftr03 1224.7  717.5  390.0  214.5 102.2  40.5

sfm04 1402.2  756.2  361.3  218.5  91.0  23.0

krw04 1988.7  870.3  320.0  182.3  53.5   7.8

ftr04  595.8  348.5  213.5  120.3  45.3   9.8

sfm05 1623.0 1018.8  647.5  347.0 106.0  42.5

krw05 1527.7  805.0  455.7  199.8  41.8  11.5

ftr05  722.5  391.2  189.5   98.8  26.7   9.0

hhb05  888.8  471.2  226.2   99.7  18.2   2.0

sfm06 2769.2 2310.7 1677.7 1142.5 448.7 138.8

krw06 2437.5 1825.7 1108.7  546.5 171.3  44.5

ftr06 1791.5  925.2  432.2  214.7  80.7  29.8

hhb06 1704.8  911.5  441.7  210.3  72.3  22.2

sfm07 1781.2 1056.0  528.7  218.5  68.2  22.5

krw07 1219.3  658.8  342.0  149.5  44.0   3.7

ftr07  644.0  321.3  153.0   71.5  23.0   1.3

hhb07  845.8  502.3  179.5   62.8   8.7   0.0

sfm08 1978.2 1029.0  535.0  250.5  70.3  23.0

krw08 1984.3  733.2  302.2  122.3  26.5   3.3

ftr08  763.8  412.5  185.5   61.5  14.8   7.2

hhb08  905.8  445.0  150.8   48.8  13.0   5.8

max.ex03 <- do.call(“rbind”,list(sfm03=sfmstats03$max,krw03=krwstats03$max,ftr03=ftrstats03$max))

max.ex04 <- do.call(“rbind”,list(sfm04=sfmstats04$max,krw04=krwstats04$max,ftr04=ftrstats04$max))

max.ex05 <- do.call(“rbind”,list(sfm05=sfmstats05$max,krw05=krwstats05$max,ftr05=ftrstats05$max,hhb05=hhbstats05$max))

max.ex06 <- do.call(“rbind”,list(sfm06=sfmstats06$max,krw06=krwstats06$max,ftr06=ftrstats06$max,hhb06=hhbstats06$max))

max.ex07 <-

do.call(“rbind”,list(sfm07=sfmstats07$max,krw07=krwstats07$max,ftr07=ftrstats07$max,hhb07=hhbstats07$max))

max.ex08 <-

do.call(“rbind”,list(sfm08=sfmstats08$max,krw08=krwstats08$max,ftr08=ftrstats08$max,hhb08=hhbstats08$max))

max.exall <- do.call(“rbind”,list(max.ex03,max.ex04,max.ex05,max.ex06,max.ex07,max.ex08))

round(max.exall,1)

25     50   100   200  500 1000

sfm03 1073.3  312.5 239.0 146.0 55.2 43.0

krw03  524.3  170.8 136.8  63.3 42.3 31.0

ftr03  238.7   65.8  56.2  46.7 36.5 14.0

sfm04  179.8  115.5  89.3  72.7 25.3  5.0

krw04  401.2   93.3  73.3  56.7 16.5  3.3

ftr04  131.8   67.5  56.3  28.2  8.2  3.0

sfm05  520.8  221.7 110.8  48.8 22.2  9.2

krw05  306.7  166.8  78.2  28.8 15.0  9.3

ftr05  147.3   61.2  32.8  24.5 10.3  4.7

hhb05  196.2   51.5  35.3  24.2  6.8  2.0

sfm06 1339.7 1076.3 611.2 323.7 58.8 26.3

krw06 1247.7  341.3 122.3  59.2 18.2 10.0

ftr06  320.8  142.0  57.8  40.7 16.0  9.8

hhb06  318.0  154.7  77.0  43.0 11.7  5.7

sfm07  448.5  266.3 213.3  66.5 34.7 14.5

krw07  337.8  257.3  74.5  38.0 16.5  3.5

ftr07  121.2   66.0  38.3  26.7 12.7  1.5

hhb07  258.2  164.3  38.3  24.3  6.3  0.0

sfm08  472.3  266.5 130.0 106.5 26.7 16.7

krw08  342.7  241.8  51.0  29.7 14.2  3.5

ftr08  144.3   89.5  49.0  19.3  8.7  4.2

hhb08  253.5  105.0  31.5  19.8  7.5  3.0