Stage corrections SFM05 - Jack Lewis

Discharge rating equation selected by Clark was

(2.5211*s^2)+(21.166*s)-30

This is problematic because it predicts negative discharges at stages below 1.24 ft,

even though stages as low as 1.016 were recorded.  Clark set the intercept to -30

in Excel to help alleviate the problem, but didn't solve it.

> sfm030405

stg       q

1   1.21    0.92

2   4.89   93.51

3  12.41  544.72

4  16.85 1064.72

5   6.73  233.23

6   2.80   34.71

7   5.33  145.92

8   2.88   32.21

9   5.61  154.76

10  8.14  396.93

11  6.45  239.79

12 10.07  398.81

13  9.18  425.16

attach(sfm030405)

plot(stg,q)

There is a slope break at 5 ft of stage and 2 curves are needed to fit these data.

fit1 <- lm(q ~ stg, data=sfm030405, subset=stg <= 5)

fit2 <- lm(q ~ stg, data=sfm030405, subset=stg > 5)

plot(stg,q)

abline(fit1)

abline(fit2)

This still gives negative discharges at low stages.  Try

a power function below 5 to ensure positive predictions

fit1p <- nls(q ~ a*stg^b, data=sfm030405, subset=stg <= 5, start=list(a=1,b=1))

plot(stg,q)

abline(fit2)

x0 <- seq(1,5,0.05)

y0 <- predict(fit1p, data.frame(stg=x0))

lines(x0,y0)

OK but a line doesn't fit the upper data real well.

Try piecing together a power function and a quadratic

plot(stg,q)

fit1p <- nls(q ~ a*stg^b, data=sfm030405, subset=stg <= 3, start=list(a=1,b=1))

x1 <- seq(1,3,0.05)

y1 <- predict(fit1p, data.frame(stg=x1))

lines(x1,y1)

fit2q <- lm(q ~ stg + I(stg^2), data=sfm030405, subset=stg > 2)

x2 <- seq(2,17,0.05)

y2 <- predict(fit2q, data.frame(stg=x2))

lines(x2,y2,col=2)

This looks better. Now find the intersection.

> fit1p

Nonlinear regression model

model:  q ~ a * stg^b

data:  sfm030405

a        b

1.232640 3.157236

> fit2q

Call:

lm(formula = q ~ stg + I(stg^2), data = sfm030405, subset = stg > 2)

Coefficients:

(Intercept)          stg     I(stg^2)

-61.386       28.325        2.183

f <- function(x) -61.386 + 28.325*x + 2.183*x^2 - 1.23264*x^3.157236

uniroot(f,c(2,3))

$root: 2.430222

$f.root: -4.41983e-05

$iter: 6

$estim.prec: 6.103516e-05

# Intersection is at stg=2.4302

# Now compare old and new rating equations

plot(stg,q, xlim=c(0,17), ylim=c(0,1100))

x0 <- seq(0,17,0.05)

y0 <- (2.5211*x0^2)+(21.166*x0)-30

lines(x0,y0)

x1 <- seq(0,2.40,0.05)

y1 <- predict(fit1p,data.frame(stg=x1))

lines(x1,y1,col=2)

x2 <- seq(2.45,17,0.05)

y2 <- predict(fit2q,data.frame(stg=x2))

lines(x2,y2,col=2)

So this just amounts to a minor tweak to float the lower end.

New expression in sfm.sdr is:

(s<2.4302)*(1.2326*s^3.1572) + (s>2.4302)* (2.1831*s^2+28.325*s-61.386)

Stage corrections - Jack Lewis

29Dec:2310 to 30Dec:0840 - Got a nice curve by reconstructing on SFM with a lag of -760 min

28May:1350 to 11Jun:0700 - Reconstructed from HHB using time-linear ratio then smoothed by interpolation

Stage discrepancies average about -0.04 and are stongly related to stage

Strategy: make a y-proportional correction for the whole year, then

go back and deal with the problems not solved.  The trendline in the

second graph is 0.0154 - 0.0104*x.  Adjusted the whole year in two

segments from 0 shift at 1.5 ft to -0.10 at 11.1 ft

Only a few remaining discrepancies need to be dealt with:

28Mar05 to 01Apr05 applied additional shift

04Apr05:1610 to 08Apr05:1700 applied additional shifts.

Turbidity corrections - Jack Lewis

09Dec:2310 to 30Dec:0810 - Replaced interpolation of this rising segment with missing

02Jan:0330 to 02Jan:1100 and 03Jan:1300 - Interpolated to remove spikes. Such spikes

are very atypical for SFM05.

The turbidity record is generally very very good for SFM05.