Data corrections FTR08 - Jack Lewis

Samples -

Observer record corrections – Jack Lewis

No errors found

Rating equation

New equation based on HY03 to HY08 data

Stage corrections - Jack Lewis

Stage offset and multiplier were both changed on 11-3-07.

First I adjusted to meet the ORs on 18Oct and 03Nov.

The resulting plot of corrected stage differences:

plot.or("ftr",08,cor=T)

# The difference on Dec. 3 is +0.057 with no explanation

# E-stage was not recorded but stage was rising.

# If time was off by 20 min that would account for +0.052.

# That's my guess, no stage correction made

# The negative diffs could be explained by errant transducer slope as shown next

plot.or("ftr",08,reread=F,xaxis="stage",cor=T)

# These plots suggest both a time trend and stage trend.  First remove the time

trend, then make a stage-proportional adjustments starting in mid-December.

1. Made one long time-variable adjustment from 03Nov2007 to end of record,

from a shift of 0.000 to 0.027

plot.or("ftr",08,071211,0000,080731,2400,cor=T,linear=T,xaxis="stage",reread=F)

> 0.030617 - 0.02589*3

[1] -0.047053

> 0.030617 - 0.02589*1

[1] 0.004727

Starting at 16Dec2007:0000 adjusted from -0.005 at 1.405 ft to -0.047 at 3 ft on 01Feb08:0540 and from there to 0.000 at 1.18 ft on 30Apr2008.

Also made a minor time-variable adjustment from 11Dec2007:1610 to 16Dec2007:0000

to remove the discontinuity introduced on 16Dec2007.

The result:

plot.or("ftr",08,cor=T)

# Not all the stage differences were removed but at least the differences

# look fairly random now.

Turbidity corrections - Jack Lewis

No turbidity data before DTS-12 installed 18Oct2007:0750.

27Nov2007:2120 to 29Nov2007:0830 interpolated to remove fouling

08Jan2008:1010 spike removed

21Jan2008:0540 spike removed

27Jan2008:1340 to 1520 questionable

27Jan2008:1910 spike removed

24Feb2008:0620 to 0920 questionable

22Apr2008:1920 spike removed

22Apr2008:2030 to 2220 spikes removed by interpolation

In general, relationships between SSC and turb aren't as good this year.

There are a lot of very sharp early hydrograph spikes in turbidity yet

the remainder of the turbidigraphs seem normal.

Dump 8 bottles 6 and 7 seem to have to low a SSC, but the turb looks good.