KRW 2003 Annual load estimation

# Read in the data

krw03.flo <- read.flo("krw",03)

krw03.lab <- read.lab("krw",03)

krw03.sed <- merge.flo("krw",03)

# Eliminate the storm event data

# This requires .../work/hy03/storms03.csv

nonstorms <- drop.storms("krw",03)

dim(nonstorms)

[1] 31124     4

head(nonstorms,5)

chr     q turb turbcode

1 (11/18/02 14:30:00) 2.205   NA        7

2 (11/18/02 14:40:00) 2.205   NA        2

3 (11/18/02 14:50:00) 2.205   NA        2

4 (11/18/02 15:00:00) 2.205   NA        2

5 (11/18/02 15:10:00) 2.205   NA        2

>

table(nonstorms$turbcode)

0     1     2     5     7

14630    51  9730  6581   132

# 9730 values of bad turbidity out of 31124 intervals to be estimated

# Extract rows with turbidity present, to be estimated using turb

trb03.flo <- nonstorms[!is.na(nonstorms$turb), ]

# Extract rows with turbidity missing, to be estimated using flow

flo03.flo <- nonstorms[is.na(nonstorms$turb), ]

# Note we are tricking R into thinking “trb” and “flo” are gaging stations

dim(trb03.flo)

[1] 21394     3

dim(flo03.flo)

[1] 9730     3

# Verify that we got all the data in nonstorms

21394+9730

[1] 31124

# Develop model for good turb data

range(trb03.flo$turb)

[1]   2 237

table(krw03.sed$labcode2)

0   1   5   6

205  21  15   1

# Eliminate lab codes 5-8, missing turbidity, and turbidity > 250

trb03.sed <- krw03.sed[krw03.sed$labcode2 < 4 & !is.na(krw03.sed$turb) & krw03.sed$turb < 250, ]

table(trb03.sed$turbcode)

0   5   7

112  53   4

turbsscplot("trb",03)

# Plenty of outliers in both directions

Coefficients:

(Intercept)         turb

-24.541        2.306

# Trouble is this will predict negative SSC for turb < 19

sum(trb03.flo$turb < 19)

[1] 12170

# There are 12170 values of turbidity < 19

turbsscplot("trb",03,type="power")

turbsscplot("trb",03,type="logxy")

# Apply the models.  Have to turn variance off for this much data.

turb1 <- turbsrc("trb",03,020801,0000,030731,2400,type="power",var=F)

turb2 <- turbsrc("trb",03,020801,0000,030731,2400,type="logxy",var=F)

total(turb1,turb2)

start            end       type surr    yhat   n    r2       s cv

turb1 12/12/02 20:40 06/18/03 16:20      power turb 1368035 169 0.623 105.622 NA

turb2 12/12/02 20:40 06/18/03 16:20 logxy:mvue turb 1321787 169 0.816   0.517 NA

# These two agree reasonably well

total(turb2)

Total load = 1321787 kg  Fair (owing to the high variance)

=======================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo03.flo$q)

[1] 1.727 7.080

> range(krw03.sed$q)

[1]    1.919 1205.000

# Eliminate lab codes > 4 and high discharges that could tweak the regression

flo03.sed <- krw03.sed[krw03.sed$labcode2 < 4 & krw03.sed$q <= 10, ]

qsscplot("flo",03)

# Since there are only 3 samples in this range of discharge, just set SSC to the

mean of these 3.

chr dump bottle   ssc turb turbcode     q labcode1 labcode2

1 (11/19/02 11:50:00)    4      1 5.048   NA        2 2.146        0        0

2 (11/20/02 11:50:00)    4      2 6.742   NA        2 2.031        0        0

3 (11/21/02 11:50:00)    4      3 8.671   NA        2 1.919        0        0

> mean(flo03.sed$ssc)

[1] 6.820333

head(flo03.flo)

chr     q turb turbcode

1  (11/18/02 14:30:00) 2.205   NA        7

2  (11/18/02 14:40:00) 2.205   NA        2

3  (11/18/02 14:50:00) 2.205   NA        2

4  (11/18/02 15:00:00) 2.205   NA        2

5  (11/18/02 15:10:00) 2.205   NA        2

flo03.flo$ssc <- 6.82

flo03.flo$flux <- 0.6*flo03.flo$q*flo03.flo$ssc

sum(flo03.flo$flux)

[1] 168995 kg

=======================================================================

# Total non-storm load = 2689822 by turbidity + 168995 by flow

1321787 + 168995 = 1490782 kg = 1491 metric tons

# Total storm load = 14709821 kg

1491 + 14710 = 16201 metric tons

# Storm loads are 14710 / 16201 = 90.8% of annual load