KRW 2007 Annual load estimation

krw07.flo <- read.flo("krw",07)

krw07.lab <- read.lab("krw",07)

krw07.sed <- merge.flo("krw",07)

nonstorms <- drop.storms("krw",07)

dim(nonstorms)

[1] 27667     4

head(nonstorms, 5)

chr     q turb turbcode

1 (09/16/06 12:50:00) 9.420    0        7

2 (09/16/06 13:00:00) 9.420    0        5

3 (09/16/06 13:10:00) 9.420    0        5

4 (09/16/06 13:20:00) 9.403    0        5

5 (09/16/06 13:30:00) 9.386    0        5

table(nonstorms$turbcode)

0     1     2     5     7

7839   119   165 19542     2

There are 165 values of bad turbidity and 119 questionable

out of 27667 intervals to be estimated.  There is a great deal of interpolated

turbidity data starting in late January.  The questionable data occurred on

Dec 22, Apr 12, and Apr 18 and are perhaps best estimated using discharge.

trb07.flo <- nonstorms[nonstorms$turbcode %in% c(0,5,7), ]

flo07.flo <- nonstorms[nonstorms$turbcode %in% c(1,2), ]

dim(trb07.flo)

[1] 27383	4

dim(flo07.flo)

[1] 284   4

27383 + 284

[1] 27667

# Develop model for good turb data

range(trb07.flo$turb)

[1]  -1 252

trb07.flo$turb[trb07.flo$turb < 0] <- 0

table(krw07.sed$labcode2)

0   1   4   9

141  29   2   1

# Just one bad lab code (9) to worry about

# (4=spilled < 15% of sediment, 9=spilled > 15% of sediment)

# Eliminate bad turbidity and labcode 9

trb07.sed <- krw07.sed[!is.na(krw07.sed$turb) & krw07.sed$labcode2 != 9, ]

table(trb07.sed$turbcode)

0   1   5

146   4  22

turbsscplot("trb",07,type="loess")

Eliminate turbidity above 400.  Too variable and exceeds range needed for prediction.

trb07.sed <- krw07.sed[!is.na(krw07.sed$turb) & krw07.sed$labcode2 != 9 & krw07.sed$turb < 400, ]

turbsscplot("trb",07,type="power")

turbsscplot("trb",07,type="logxy")

turb1 <- turbsrc("trb",07,060801,0000,070731,2400,type="power",var=F)

turb2 <- turbsrc("trb",07,060801,0000,070731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",07,060801,0000,070731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",07,060801,0000,070731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat   n    r2      s cv

turb1 09/16/06 12:50 04/22/07 12:00      power turb 557796.5 161 0.948 34.732 NA

turb2 09/16/06 12:50 04/22/07 12:00 logxy:mvue turb 605073.7 161 0.893  0.371 NA

turb3 09/16/06 12:50 04/22/07 12:00      loess turb 537881.9 161 0.947 35.011 NA

turb4 09/16/06 12:50 04/22/07 12:00     linear turb 518742.6 161 0.946 35.189 NA

> sum(turb3$predssc < 0)

[1] 12213

> sum(turb4$predssc < 0)

[1] 13150

# Can't use loess or linear: too many negative predictions

# The power model or logxy model differ by 9%.  Hard to say which is better.

# Power regression looks nicer but most predictions are needed below 20 NTU

# Logxy usually does better in that range but the dispersion is ill-conditioned

total(turb1)

start            end  type surr     yhat   n    r2      s cv

turb1 09/16/06 12:50 04/22/07 12:00 power turb 557796.5 161 0.948 34.732 NA

Total load = 557797 kg

# A fairly good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo07.flo$q)

[1]  37.46 112.10

> range(krw07.sed$q)

[1]  22.72 1078.00

qsscplot("krw",07,type="loess")

The data to be estimated are below 112cfs = 3.17 cumecs.

# Limit the regression data to less than 10 cumecs, and labcodes != 9

flo07.sed <- krw07.sed[krw07.sed$q < 350 & krw07.sed$labcode2 != 5, ]

qsscplot("flo",07)

qsscplot("flo",07,type="power")

flow1 <- flowsrc("flo",07,060801,0000,070731,2400,type="logxy")

flow2 <- flowsrc("flo",07,060801,0000,070731,2400,type="power")

flow3 <- flowsrc("flo",07,060801,0000,070731,2400,type="loess")

flow4 <- flowsrc("flo",07,060801,0000,070731,2400,type="linear")

total(flow1,flow2,flow3,flow4)

start            end       type surr     yhat   n    r2      s     cv

flow1 11/21/06 11:20 04/18/07 17:00 logxy:mvue flow 15857.71 121 0.343  0.758  9.294

flow2 11/21/06 11:20 04/18/07 17:00      power flow 12964.89 121 0.415 68.382 16.150

flow3 11/21/06 11:20 04/18/07 17:00      loess flow 14324.78 121 0.422 68.216     NA

flow4 11/21/06 11:20 04/18/07 17:00     linear flow 13572.97 121 0.407 68.789 19.479

> sum(flow3$predssc <0)

[1] 0

> sum(flow4$predssc <0)

[1] 0

#Based on CV, logxy model is best. Range of estimates is 12-13%

#The logxy model looks better

total(flow1)

start            end       type surr     yhat   n    r2     s    cv

flow1 11/21/06 11:20 04/18/07 17:00 logxy:mvue flow 15857.71 121 0.343 0.758 9.294

Total load = 15858 kg,  CV = 9.29 %

==================================================

# Total non-storm load

557797 + 15858 = 573655 kg = 574 metric tons

# Quality is good

# Total storm load = 5060464 kg = 5060 metric tons

# Total annual load

5060 + 574 = 5634 metric tons total annual load

# Storm load is 5060/5634 = 89.8% of annual load