HHB07 Event Loads

stn,storm,sdate,stime,edate,etime

HHB,701,061213,0000,061214,2110   145197 kg  Fair

HHB,702,061225,2240,061229,1200   257277 kg  Excellent

HHB,703,070103,0540,070106,1040   663351 kg  Excellent

HHB,704,070210,0450,070213,1200   289151 kg  Good

HHB,705,070220,1420,070224,1410  1752431 kg  Very good

HHB,706,070224,1420,070227,0630   235679 kg  Good

HHB,707,070227,0640,070303,1200   260911 kg  Fair

HHB,708,070326,1000,070329,1200   204303 kg  Very good

-----------------------------------------------------

SUM of events:                   3808300 kg  Very good

-----------------------------------------------------

hhb07.flo <- read.flo("hhb",07)

hhb07.lab <- read.lab("hhb",07)

mismatches("hhb",07)

# Unmatched samples in flo file: 5(2,7-9), 6(10-14), 8(1,2,5,10), 10(3,8,10)

hhb07.sed <- merge.flo("hhb",07)

-----------------------------------------------------

Storm 701, dump 6, 061213,0000,061214,2110

# Excellent stage and turbidity data but only 1 sample (dump 6, bot 15)

# Will have to use bottles from storm 702 to define relationships

# Peak turbidity is 243

four()

turbsscplot("hhb",07,dumps=6,type="power")

turbsscplot("hhb",07,dumps=7,type="power")

turbsscplot("hhb",07,dumps=6:7)

turbsscplot("hhb",07,dumps=6:7,type="power")

turb1 <-  turbsrc("hhb",07,061213,0000,061214,2110,dumps=6,type="power")

turb2 <-  turbsrc("hhb",07,061213,0000,061214,2110,dumps=7,type="power")

turb3 <-  turbsrc("hhb",07,061213,0000,061214,2110,dumps=6:7)

turb4 <-  turbsrc("hhb",07,061213,0000,061214,2110,dumps=6:7,type="power")

total(turb1,turb2,turb3,turb4)

start            end   type surr     yhat  n    r2      s     cv

turb1 2/13/06 12/14/06 21:10  power turb 162852.2 10 0.879 13.516 17.503

turb2 2/13/06 12/14/06 21:10  power turb 145487.8 16 0.971 17.786  3.855

turb3 2/13/06 12/14/06 21:10 linear turb 142032.2 26 0.961 16.956  3.160

turb4 2/13/06 12/14/06 21:10  power turb 145196.8 26 0.966 15.804  2.662

# Estimates vary by 13%, but the range of the first model is too limited

ttsplot("hhb",07,turb4)

total(turb4)

start            end  type surr     yhat  n    r2      s    cv

turb4 2/13/06 12/14/06 21:10 power turb 145196.8 26 0.966 15.804 2.662

Total load = 145197 kg,  CV = 2.66 %  Fair

-----------------------------------------------------

Storm 702,061225,2240,061229,1200

# Two-peak affair with excellent stage and turbidity data

# Excellent sample coverage on both peaks

four()

turbsscplot("hhb",07,061225,2240,061229,1200)

turbsscplot("hhb",07,061225,2240,061229,1200,type="power")

turbsscplot("hhb",07,061225,2240,061227,0320,type="sqrt")

turbsscplot("hhb",07,061227,0330,061229,1200,type="sqrt")

turb1 <-  turbsrc("hhb",07,061225,2240,061229,1200,type="power")

turb2 <-  turbsrc("hhb",07,061225,2240,061227,0320,type="power")

turb3 <-  turbsrc("hhb",07,061227,0330,061229,1200,type="power")

turb4 <-  turbsrc("hhb",07,061225,2240,061227,0320,type="sqrt")

turb5 <-  turbsrc("hhb",07,061227,0330,061229,1200,type="sqrt")

total(turb1)

start            end  type surr     yhat  n    r2      s    cv

turb1 12/25/06 22:40 12/29/06 12:00 power turb 262656.6 13 0.964 19.938 4.048

Total load = 262657 kg,  CV = 4.05 %

total(turb2,turb3)

start            end  type surr      yhat n    r2      s    cv

turb2 12/25/06 22:40 12/27/06 03:20 power turb  98772.48 6 0.995  7.321 2.250

turb3 12/27/06 03:30 12/29/06 12:00 power turb 161909.65 7 0.963 23.267 6.371

Total load = 260682 kg,  CV = 4.05 %

total(turb4,turb5)

start            end      type surr      yhat n    r2     s cv

turb4 12/25/06 22:40 12/27/06 03:20 sqrt:duan turb  98444.64 6 0.994 0.423 NA

turb5 12/27/06 03:30 12/29/06 12:00 sqrt:duan turb 158832.72 7 0.979 0.788 NA

Total load = 257277 kg  Excellent

# These 3 estimates vary by 2.1%

# Only the last model fits the recession limb samples (14-16) well

ttsplot("hhb",07,turb5,turb4)

-----------------------------------------------------

Storm 703, dump 8, 070103,0540,070106,1040

# Excellent stage and turbidity data

# Barely adequate sample coverage (6 samples)

# Augment with bottle 16 from previous storm

two()

turbsscplot("hhb",07,070103,0540,070106,1040)

turbsscplot("hhb",07, dump=c(7,rep(8,9)), bot=c(16,1:9))

turb1 <-  turbsrc("hhb",07,070103,0540,070106,1040)

turb2 <-  turbsrc("hhb",07,070103,0540,070106,1040, dump=c(7,rep(8,9)), bot=c(16,1:9))

total(turb1,turb2)

start            end   type surr     yhat n    r2      s    cv

turb1 01/03/07 05:40 01/06/07 10:40 linear turb 663350.5 6 0.994 13.696 2.878

turb2 01/03/07 05:40 01/06/07 10:40 linear turb 651555.8 7 0.995 14.283 2.611

# Estimates differ by 1.8%

# The latter estimate has a better prediction range but make negative predictions

ttsplot("hhb",07,turb2)

ttsplot("hhb",07,turb1)

total(turb1)

start            end   type surr     yhat n    r2      s    cv

turb1 01/03/07 05:40 01/06/07 10:40 linear turb 663350.5 6 0.994 13.696 2.878

Total load = 663351 kg,  CV = 2.88 %  Excellent

-----------------------------------------------------

Storm 704, dumps 9-10, 070210,0450,070213,1200

# Two-peaked storm, second peak didn't have much turbidity response

# Excellent stage and turbidity data

# A couple of quick turbidity spikes removed

# Fair sample coverage (nothing on the peak); peak turbidity 294

# Bottle 3 was empty suggesting a pumping sample problem

# Might help to augment with bottles from next event

four()

turbsscplot("hhb",07,070210,0450,070213,1200)

turbsscplot("hhb",07,070210,0000,070223,1200)

turbsscplot("hhb",07,070210,0000,070223,1200,type="power")

turbsscplot("hhb",07,070210,0000,070223,1200,type="logxy")

turb1 <-  turbsrc("hhb",07,070210,0450,070213,1200)

turb2 <-  turbsrc("hhb",07,070210,0450,070213,1200,070210,0000,070223,1200)

turb3 <-  turbsrc("hhb",07,070210,0450,070213,1200,070210,0000,070223,1200,type="power")

turb4 <-  turbsrc("hhb",07,070210,0450,070213,1200,070210,0000,070223,1200, type="logxy",bias="qmle")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s     cv

turb1 02/10/07 04:50 02/13/07 12:00     linear turb 291319.1  7 0.801 46.131 13.276

turb2 02/10/07 04:50 02/13/07 12:00     linear turb 298681.9 27 0.970 39.592  5.889

turb3 02/10/07 04:50 02/13/07 12:00      power turb 297131.3 27 0.970 39.208  5.791

turb4 02/10/07 04:50 02/13/07 12:00 logxy:qmle turb 289150.5 26 0.947  0.290  6.120

# Estimates vary by 3.3%

ttsplot("hhb",07,turb3)

ttsplot("hhb",07,turb4)

# This fits the recession better

total(turb4)

start            end       type surr     yhat  n    r2    s   cv

turb4 02/10/07 04:50 02/13/07 12:00 logxy:qmle turb 289150.5 26 0.947 0.29 6.12

Total load = 289151 kg,  CV = 6.12 %  Good

-----------------------------------------------------

Storm 705, dump 11, 070220,1420,070224,1410

# Odd dome-shaped hydrograph

# Very good stage and turbidity data

# Staff plate reading didn't match stage on late recession (diff = -0.046)

# Only minor stage shift was applied (about -0.01)

# One quick turbidity spike removed

# Outstanding sample coverage (14 bottles well-distributed)

four()

turbsscplot("hhb",07,070220,1420,070224,1410)

turbsscplot("hhb",07,070220,1420,070224,1410,type="power")

turbsscplot("hhb",07,070220,1420,070224,1410,type="logxy")

turbsscplot("hhb",07,070220,1420,070224,1410,type="loess")

turb1 <-  turbsrc("hhb",07,070220,1420,070224,1410)

turb2 <-  turbsrc("hhb",07,070220,1420,070224,1410,type="power")

turb3 <-  turbsrc("hhb",07,070220,1420,070224,1410,type="logxy",bias="qmle")

turb4 <-  turbsrc("hhb",07,070220,1420,070224,1410,type="loess")

endpoints(turb2)

start      end

16.00455 39.87501

turb5 <- lineartime("hhb",07,070220,1420,070224,1410,ssc1=16,ssc2=20)

total(turb1,turb2,turb3,turb4,turb5)
               start            end       type surr    yhat  n    r2      s    cv

turb1 02/20/07 14:20 02/24/07 14:10     linear turb 1756307 14 0.964 46.907 4.263

turb2 02/20/07 14:20 02/24/07 14:10      power turb 1752431 14 0.965 46.172 4.111

turb3 02/20/07 14:20 02/24/07 14:10 logxy:qmle turb 1788353 14 0.913  0.252 7.749

turb4 02/20/07 14:20 02/24/07 14:10      loess turb 1756467 14 0.967 46.858    NA

turb5 02/20/07 14:20 02/24/07 14:10     linear time 1720168 14    NA     NA    NA

# Estimates vary by 4%

ttsplot("hhb",07,turb5)

ttsplot("hhb",07,turb2)

start            end  type surr    yhat  n    r2      s    cv

turb2 02/20/07 14:20 02/24/07 14:10 power turb 1752431 14 0.965 46.172 4.111

Total load = 1752431 kg,  CV = 4.11 %  Very good to excellent

-----------------------------------------------------

Storm 706, dump 12, 070224,1420,070227,0630

# Excellent stage and turbidity data

# Very good sample coverage except at bottom end (7 bottles)

# Possibly augment with bottle 16 from previous event

four()

turbsscplot("hhb",07,070224,1420,070227,0630)

turbsscplot("hhb",07,070223,0800,070227,0630)

turbsscplot("hhb",07,070223,0800,070227,0630,type="power")

turbsscplot("hhb",07,070223,0800,070227,0630,type="logxy")

turb1 <-  turbsrc("hhb",07,070224,1420,070227,0630)

turb2 <-  turbsrc("hhb",07,070224,1420,070227,0630,070223,0800,070227,0630)

turb3 <-  turbsrc("hhb",07,070224,1420,070227,0630,070223,0800,070227,0630,type="power")

turb4 <-  turbsrc("hhb",07,070224,1420,070227,0630,070223,0800,070227,0630,type="logxy",bias="qmle")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat n    r2      s    cv

turb1 02/24/07 14:20 02/27/07 06:30     linear turb 222147.9 7 0.939 15.666 9.114

turb2 02/24/07 14:20 02/27/07 06:30     linear turb 231252.8 8 0.937 15.346 7.342

turb3 02/24/07 14:20 02/27/07 06:30      power turb 231635.8 8 0.938 15.209 6.132

turb4 02/24/07 14:20 02/27/07 06:30 logxy:qmle turb 235679.4 8 0.921  0.121 4.771

# Estimates vary by 6.1%

ttsplot("hhb",07,turb4)

total(turb4)

start            end       type surr     yhat n    r2     s    cv

turb4 02/24/07 14:20 02/27/07 06:30 logxy:qmle turb 235679.4 8 0.921 0.121 4.771

Total load = 235679 kg,  CV = 4.77 %  Good

-----------------------------------------------------

Storm 707, dump 12, 070227,0640,070303,1200

# Small peak with only 3 samples (bots 8, 9, 10)

# Very good stage and turbidity data

# Poor sample coverage

# Need to augment with more bottles from this dump

four()

turbsscplot("hhb",07,dump=12)

turbsscplot("hhb",07,dump=12, bot=1:10)

turbsscplot("hhb",07,dump=12, bot=4:10)

turbsscplot("hhb",07,dump=12, bot=4:10, type="power")

turb1 <-  turbsrc("hhb",07,070227,0640,070303,1200,dump=12, bot=1:10)

turb2 <-  turbsrc("hhb",07,070227,0640,070303,1200,dump=12, bot=4:10)

turb3 <-  turbsrc("hhb",07,070227,0640,070303,1200,dump=12, bot=4:10, type="power")

total(turb1,turb2,turb3)

start            end   type surr     yhat  n    r2      s     cv

turb1 02/27/07 06:40 03/03/07 12:00 linear turb 232210.8 10 0.813 24.286 15.426

turb2 02/27/07 06:40 03/03/07 12:00 linear turb 260911.1  7 0.890 22.280 13.892

turb3 02/27/07 06:40 03/03/07 12:00  power turb 248654.8  7 0.877 23.668 14.212

# Estimates vary by 12.3%

ttsplot("hhb",07,turb2)

total(turb2)

start            end   type surr     yhat n   r2     s     cv

turb2 02/27/07 06:40 03/03/07 12:00 linear turb 260911.1 7 0.89 22.28 13.892

Total load = 260911 kg,  CV = 13.89 %  Fair

-----------------------------------------------------

Storm 708, dump 13, 070326,1000,070329,1200

# Excellent stage and turbidity data

# Stage was shifted by about -0.01 to match staff plate at start of event

# Very good sample coverage (8 well-distributed bottles)

two()

turbsscplot("hhb",07,070326,1000,070329,1200)

qsscplot("hhb",07,070326,1000,070329,1200)

turb1 <- turbsrc("hhb",07,070326,1000,070329,1200)

flow1 <- flowsrc("hhb",07,070326,1000,070329,1200,bias="qmle")

flow2 <- flowsrc("hhb",07,070326,1000,070329,1200,type="pairs")

total(turb1,flow1,flow2)

start            end       type surr     yhat n    r2      s     cv

turb1 03/26/07 10:00 03/29/07 12:00     linear turb 204302.6 7 0.971 16.102  5.449

flow1 03/26/07 10:00 03/29/07 12:00 logxy:qmle flow 216473.3 7 0.872  0.413 20.560

flow2 03/26/07 10:00 03/29/07 12:00      pairs flow 209495.9 7    NA     NA     NA

# flow1 is not really a contender.  The others differ by just 2.5%

ttsplot("hhb",07,flow2)

ttsplot("hhb",07,turb1)

total(turb1)

start            end   type surr     yhat n    r2      s    cv

turb1 03/26/07 10:00 03/29/07 12:00 linear turb 204302.6 7 0.971 16.102 5.449

Total load = 204303 kg,  CV = 5.45 %  Very good