HHB 2008 Annual SAND and FINES load estimation

hhb08.flo <- read.flo("hhb",08)

hhb08.lab <- read.sand("hhb",08)

hhb08.sed <- merge.flo.sand("hhb",08)

# Now create the ssc column: it will represent only FINES

hhb08.sed$ssc <- hhb08.sed$totssc - hhb08.sed$sand

# Since it's named ssc, we can use existing functions to calc the fines load

dim(hhb08.flo)

[1] 28360     9

table(hhb08.flo$turbcode)

0     1     5     6

27358    10    12   980

10 questionable values of turbidity out of 28360 intervals to be estimated

None bad (code=2), 980 reconstructed from another site (KRW)

# Copy hhb08.flo to trb08.flo as is

trb08.flo <- hhb08.flo

# Develop model for good turb data

range(trb08.flo$turb)

[1]  0  705

range(hhb08.sed$turb)

[1]   61 703

# Big problem, low turbidities not represented

table(hhb08.sed$labcode2)

0  1

23  4

turbsscplot("hhb",08,txt="dump")

# Eliminate the following previously identifed outliers

Dump  Bot

12    11

14     7

trb08.sed <- hhb08.sed[!(hhb08.sed$dump==12 & hhb08.sed$bottle==11), ]

trb08.sed <- trb08.sed[!(trb08.sed$dump==14 & trb08.sed$bottle==7), ]

dim(trb08.sed)

[1] 25 11

# This is a relatively small sample on which to base an annual load

four()

turbsscplot("trb",08,type="power")

turbsscplot("trb",08,type="logxy")

turbsscplot("trb",08,type="loess")

turbsscplot("trb",08,type="loess",degree=2)

# Try a few different models for comparison

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",degree=2,var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2      s cv

turb1 10/20/07 15:10 05/04/08 13:40      power turb 3706795 25 0.935 70.795 NA

turb2 10/20/07 15:10 05/04/08 13:40 logxy:mvue turb 3356547 25 0.896  0.222 NA

turb3 10/20/07 15:10 05/04/08 13:40      loess turb 2244125 25 0.939 69.800 NA

turb4 10/20/07 15:10 05/04/08 13:40      loess turb 1391593 25 0.943 69.584 NA

> sum(turb3$predssc < 0)

[1] 24698

> sum(turb4$predssc < 0)

[1] 26493

min(trb08.sed$turb)

[1] 130

quantile(trb08.flo$turb,seq(0.90,0.99,0.01))

90% 91% 92% 93% 94% 95% 96% 97% 98% 99%

47  49  51  55  59  65  73  83  99 138

# Can't use linear or loess because of so many negative predictions

# The power and logxy estimates differ by 10%

# The real problem is that the minimum turbidity of any of the samples is 130

# and nearly 99% of the data to be predicted has turbidity below 130

total(turb2)

start            end       type surr    yhat  n    r2     s cv

turb2 10/20/07 15:10 05/04/08 13:40 logxy:mvue turb 3356547 25 0.896 0.222 NA

Total fines load = 3356547 kg

==================================================

# Now compute the total annual load based on this same smaller sample

# i.e. using only the 25 bottles that had sand fractions done and were retained above

trb08.sed$ssc <- trb08.sed$totssc

four()

turbsscplot("trb",08,txt="dump",type="linear")

turbsscplot("trb",08,txt="dump",type="loess")

turbsscplot("trb",08,txt="dump",type="power")

turbsscplot("trb",08,txt="dump",type="logxy")

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",degree=2,var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2       s cv

turb1 10/20/07 15:10 05/04/08 13:40      power turb 4241450 25 0.916 100.287 NA

turb2 10/20/07 15:10 05/04/08 13:40 logxy:mvue turb 3812261 25 0.881   0.254 NA

turb3 10/20/07 15:10 05/04/08 13:40      loess turb 2088733 25 0.921  98.847 NA

turb4 10/20/07 15:10 05/04/08 13:40      loess turb 1433708 25 0.925  98.846 NA

> sum(turb3$predssc < 0)

[1] 26097

> sum(turb4$predssc < 0)

[1] 26735

# Can't use linear or loess because of so many negative predictions

# The power and logxy estimates differ by 11%

# The real problem is again that the minimum turbidity of any of the samples is 130

total(turb2)

start            end       type surr    yhat  n    r2     s cv

turb2 10/20/07 15:10 05/04/08 13:40 logxy:mvue turb 3812261 25 0.881 0.254 NA

Total load based on reduced sample = 3812261 kg

This total is agrees surprisingly well with other estimates:

4046490 based on sum of storms and inter-storm loads

3972519 based on one annual regression model using all 120 samples

==================================================

==================================================

Total load 3812261 based on reduced sample

Total fines 3356547

Total sand  455714

Percent sand = 12.0%

Percent fines= 88.0%

--------------------------------------------------

If power models are used instead of logxy:

Total load 4241450 based on reduced sample

Total fines 3706795

Total sand  534655

Percent sand = 12.6%

Percent fines= 87.4%