Stage corrections SFM04 - Jack Lewis

Starting in January, a great majority of staff plate readings are below the electronic stage

(negative stage diffs).  I entered the stage diffs in a comma-delimited tables, starting with

12/26/2003, and then read them into R.

# Read in the stage diffs

sfm04.or <- read.csv("sfm/sfm04.or2",header=F)

# Read in the OR file created by TTS adjuster

tmp <- read.csv("sfm/sfm04.or",header=F)

names(tmp) <- c("year","mo","dy","time","code","staff")

row.names(tmp) <- make.chr(tmp$year,tmp$mo,tmp$dy,tmp$time)

names(sfm04.or) <- c("date","time","diff")

sfm04.or$chr <- chron(as.character(sfm04.or$date),sfm04.or$time)

sfm04.or$staff <- tmp[format(sfm04.or$chr),"staff"]

date     time   diff                 chr staff

1  12/26/2003 12:50:00 -0.012 (12/26/03 12:50:00)  3.88

2  01/03/2004 13:40:00 -0.016 (01/03/04 13:40:00)  5.39

3  01/04/2004 14:10:00 -0.025 (01/04/04 14:10:00)  4.48

4  01/08/2004 10:22:00 -0.009 (01/08/04 10:22:00)  2.95

5  01/10/2004 08:20:00 -0.018 (01/10/04 08:20:00)  3.09

6  01/24/2004 12:00:00 -0.037 (01/24/04 12:00:00)  4.68

7  01/24/2004 14:00:00 -0.051 (01/24/04 14:00:00)  4.62

8  01/28/2004 10:00:00 -0.029 (01/28/04 10:00:00)  5.10

9  01/29/2004 09:50:00  0.052 (01/29/04 09:50:00)  4.37

10 02/01/2004 13:20:00 -0.015 (02/01/04 13:20:00)  3.82

11 02/07/2004 10:20:00 -0.041 (02/07/04 10:20:00)  4.91

12 02/12/2004 11:00:00  0.007 (02/12/04 11:00:00)  2.85

13 02/16/2004 13:30:00 -0.086 (02/16/04 13:30:00)  2.95

14 02/17/2004 08:50:00 -0.060 (02/17/04 08:50:00) 15.24

15 02/18/2004 09:00:00 -0.030 (02/18/04 09:00:00) 12.60

16 02/19/2004 10:20:00 -0.072 (02/19/04 10:20:00)  6.81

17 02/22/2004 07:40:00 -0.008 (02/22/04 07:40:00)  3.64

18 02/22/2004 11:40:00 -0.010 (02/22/04 11:40:00)  3.58

19 02/26/2004 11:00:00 -0.080 (02/26/04 11:00:00)  9.43

20 02/27/2004 08:30:00 -0.090 (02/27/04 08:30:00)  7.93

21 02/29/2004 14:10:00 -0.016 (02/29/04 14:10:00)  4.35

22 03/12/2004 16:50:00 -0.005 (03/12/04 16:50:00)  2.28

23 03/24/2004 18:00:00 -0.009 (03/24/04 18:00:00)  1.75

24 03/28/2004 08:00:00  0.391 (03/28/04 08:00:00)  3.34

25 04/01/2004 07:10:00  0.411 (04/01/04 07:10:00)  3.42

26 05/18/2004 15:20:00  0.108 (05/18/04 15:20:00)  3.16

27 05/18/2004 17:30:00 -0.005 (05/18/04 17:30:00)  2.95

28 07/01/2004 10:00:00  0.027 (07/01/04 10:00:00)  1.21

# Look at trend in differences with time

scatter.smooth(sfm04.or$chr, sfm04.or$diff, ylab="Staff - Elec", xlab="", axes=F)

axis.time(sfm04.or$chr)

axis(2)

box()

There is not a compelling trend with time, except maybe thru February.

But is there a relationship with stage?

scatter.smooth(sfm04.or$staff,sfm04.or$diff,span=1,xlab="Staff plate",

ylab="Staff - elec")

Eliminate the biggest outliers in x and y directions:

scatter.smooth(sfm04.or$staff[-c(14:15,24:25)],sfm04.or$diff[-c(14:15,24:25)],span=1,xlab="Staff plate", ylab="Staff - elec")

Removing the outliers leaves a fairly linear relationship, suggesting that a y-proportional shift should fix most problems.  There is a field note that there was an error in the staff plate reading whose y=0.052.  And the one at y=0.108 was late in the year (May 18).  So eliminate those two.

plot(sfm04.or$staff[-c(9,14:15,24:26)],sfm04.or$diff[-c(9,14:15,24:26)],span=1,xlab="Staff plate", ylab="Staff - elec")

tmpfit <- lm(diff ~ staff, data=sfm04.or, subset = -c(9,14:15,24:26))

abline(tmpfit)

Coefficients:

(Intercept)        staff

0.02368     -0.01210

# Get predicted values from 0 to 8 ft by 0.5

0.02368 - 0.01210 * seq(0,8,0.5)

[1]  0.02368  0.01763  0.01158  0.00553 -0.00052 -0.00657 -0.01262 -0.01867

[9] -0.02472 -0.03077 -0.03682 -0.04287 -0.04892 -0.05497 -0.06102 -0.06707

[17] -0.07312

A linear model says the correction goes from 0 at 2ft to -0.073 at 8ft of stage, but at 12 ft of stage and 15 ft, the OR’s indicated corrections of only -0.03 and -0.06, respectively.  Compromise: I used the y-proportional correction from Jan 1 to Mar 25, when other problems started occurring.  Then went back and again shifted (sticking with y-proportional) to match the ORs at 12 and 15 ft that didn't fit the relationship.

The OR on Feb 16 indicates an error of -0.086 at a stage of only 3 feet but the stage was increasing so rapidly that this could have simply been a synchronization error (i.e the staff plate was not read right at the wakeup).  Made no adjustment on May 18 for OR at 1520 because OR same day at 1730 indicates no error.

Turbiditiy corrections SFM04 - Jack Lewis

Replaced most of Clark's reconstructions prior to Dec. 6, 2003.

Recoded Dec. 6 1000-1140 as questionable based on scatterplot and spiky trace.

Redid reconstruction of recession of Dec. 8 - 10 more realistically.

Dec. 11-13 turbidity is not reconstructable - replaced with missing.

Dec. 13-17 turbidity data is terrible but bottoms of spikes were interpolated to give a realistic looking trace.

Dec. 18-20 again reconstruction was too speculative - replaced with missing.

Boom FINALLY installed on Dec 21 ending major problems.

Jan 12-23 replaced fabricated turbidity with missing.

Jan 27 1600-1820 replaced Clark's reconstruction with raw (which looks fine except for the downspike which I removed by interpolation)

Feb 8-12 recession redid recession.

Clark's corrections look fine for remainder of HY2004.