FTR 2005 Annual load estimation

ftr05.flo <- read.flo("ftr",05)

ftr05.lab <- read.lab("ftr",05)

ftr05.sed <- merge.flo("ftr",05)

nonstorms <- drop.storms("ftr",05)

dim(nonstorms)

[1] 50582	4

head(nonstorms, 5)

chr     q turb turbcode

1 (06/30/04 19:10:00) 3.676    2        0

2 (06/30/04 19:20:00) 3.691    2        5

3 (06/30/04 19:30:00) 3.721    2        5

4 (06/30/04 19:40:00) 3.752    2        5

5 (06/30/04 19:50:00) 3.782    2        5

> table(nonstorms$turbcode)

0     5     7

35390 15190     2

No bad turbidity values, just lots of interpolations

trb05.flo <- nonstorms[!is.na(nonstorms$turb), ]

dim(trb05.flo)

[1] 50582

# Develop model for good turb data

range(trb05.flo$turb)

[1]   0 218

range(ftr05.sed$turb,na.rm=T)

[1]   1 972

table(ftr05.sed$labcode2)

0   1   4   5   6

235   2   1   1   2

# 4 = spilled < 15%

# 5 = low volume (< 150 ml)

# 6 = organic debris > 15%

# Eliminate bad turbidity and lab codes

# Eliminate bad bottles identified in storm analyses

ftr05.sed[c(50,200,215,217,227), ]

chr dump bottle    ssc turb turbcode     q labcode1 labcode2

50  (12/08/04 03:30:00)    4      6 1062.0   NA        2 724.4        0        5

200 (04/09/05 22:50:00)   15      1  638.1   62        0 166.3        0        0

215 (05/08/05 13:20:00)   16      9  250.6   61        0 115.8        0        0

217 (05/08/05 23:30:00)   16     11   89.1  213        0 274.8        0        0

227 (05/09/05 22:50:00)   16     21  437.5  107        0 224.5        0        0

trb05.sed <- ftr05.sed[-c(50,200,215,217,227), ]

turbsscplot("trb",05)

# Restrict range to range of non-storm turbidity

trb05.sed <- trb05.sed[!is.na(trb05.sed$turb), ]

trb05.sed <- trb05.sed[trb05.sed$turb < 240,]

turbsscplot("trb",05)

turbsscplot("trb",05,type="power")

turbsscplot("trb",05,type="logxy")

Eliminate the 2 severe outliers for logxy model

trb05.sed[c(2,28),]

chr dump bottle   ssc turb turbcode     q labcode1 labcode2

2  (09/25/04 12:50:00)    1     10 11.34    1        0 4.248        0        0

28 (10/31/04 11:10:00)    2     17 35.79    1        5 6.470        0        0

trb05.sed <- trb05.sed[-c(2,28), ]

turbsscplot("trb",05,type="logxy")

# Try a few different models for comparison

turb1 <- turbsrc("trb",05,040801,0000,050731,2400,type="power",var=F)

turb2 <- turbsrc("trb",05,040801,0000,050731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",05,040801,0000,050731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",05,040801,0000,050731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat   n    r2      s cv

turb1 8/01/04 07/16/05 14:10      power turb 272307.6 210 0.944 22.017 NA

turb2 8/01/04 07/16/05 14:10 logxy:mvue turb 292703.2 210 0.902  0.426 NA

turb3 8/01/04 07/16/05 14:10      loess turb 228343.1 210 0.937 23.289 NA

turb4 8/01/04 07/16/05 14:10     linear turb 261242.6 210 0.921 26.045 NA

> sum(turb3$predssc < 0)

[1] 35316

> sum(turb4$predssc < 0)

[1] 38531

# Can't use linear or loess: too many negative predictions

# Power model is the best fit; result is about the same as logxy model

turbsscplot("trb",05,type="power")

total(turb1)

start            end  type surr     yhat   n    r2      s cv

turb1 8/01/04 07/16/05 14:10 power turb 272307.6 210 0.944 22.017 NA

Total load = 272308 kg

# Should be quite a good estimate

====================================================================

# Total non-storm load

272308 kg = 272 metric tons

# Total storm load = 3379694 kg = 3380 metric tons

# Total annual load

3380 + 272 =  3652 metric tons total annual load

# Storm load is 90.9% of annual load

3380/3652

[1]  0.9255203