Data corrections KRW 08 - Jack Lewis

Changed dump sequence from 19, 1, 3, 4, etc to 1, 2, 3, 4, etc

Renumbered ISCO samples from dump 1 as dump 2.

Note there is no record of a bottle 13 in the above dump, but it's in the lab data  with SSC=54.2.

Renamed krw080128.dat to krw080129.dat

Krw080307.dat last 3 records are labeled dump 11.  TTS adjuster called them dump 10.

There is a 28-day gap between this file and the next.  Day 85 to day 113.

Krw080325.dat had only 3 records.  Started on day 113: 080422.

The next file was krw080422.dat, so I moved these 3 records into krw080422.dat

And deleted krw080325.dat.  Note however there is another 28-day gap between days

113 and 141.

i.e. there is a 56-day gap from Mar 25 to May 20, except for these 3 records

226,2008,113,1350,12,1,1,3,2.325,303,8.7,0

226,2008,113,1400,12,2,1,1,2.344,124,8.9,0

226,2008,113,1410,12,3,2,1,2.364,13,8.9,0

And there are no Campbell records corresponding to any of the bottles in dump 11 or 12

Except the 3 above.

Observer record corrections – Jack Lewis

Rating equation

Stage corrections - Jack Lewis

Digitized dump 11 stage record and sample times using a plot from Jesse, that I’ve named krw08dump11.jpeg. Converted to bitmap at 125% expansion and digitized with xyExtract program:

http://www.newfreedownloads.com/Home-Education/Science/xyExtract-Graph-Digitizer.html

See Converted data, interpolated in R, saved in flo file, then in TTSAdjuster, interpolated to remove kinks, and shifted to match the

last recorded data at 25Mar2008:1750. See KRW08 Digitizing.rtf for details of how the manipulations in R were done. Sample times for dump 11 were also estimated by digitizing Jesse’s graph.  This was somewhat crude, but nothing of importance was happening at this time. The 3 data points mentioned above do not fit with the interpolated stages or the observer record at 22Apr:1300.  I interpolated them out to get a smooth hydrograph. Further shifts will be required.

plot.or("krw",08)

At the time of the outlier the stage was changing very quickly per FF notes.

I'll attribute discrpancy to inaccurate time-recording

krw08.or <- krw08.or[-10,]

plot.or("krw",08)

plot.or("krw",08,x="stage")

Most of the errors are negative but are not well-related to time or stage.

mean(krw08.or$staff-krw08.or$rawstg)

[1] -0.04474762

I'll start by shifting the whole year by -0.045, then fine-tune.

Reconstructed 23Sep2007:1200 to 05Oct2007:1610 from SFM using linear relation.

Reconstructed missing segment from 22Apr to 04May using FTR. Assumed identical

flow on 21Apr:1500 and 04May:1000 to get a right-hand anchor.  No data from any

other stations after May 4.  Reconstructed the remainder from KRW using a lag

of -276500, i.e. the hydrograph recession starting on 20Oct2007:2200

26Jan2008:1040 there is an undocumented hydrograph shift of +0.45 which is very

unrealistic if not impossible.  Another big unexplained shift of +0.30 on 24Feb2008:0810.

I cannot reconcile this hydrograph with the ORs, so I'm ignoring the OR on 21Feb:1500.

I've made quite a few time-variable shifts to correct the big jumps in the hydrograph

and to get better agreement with the ORs.

Turbidity corrections - Jack Lewis

Bad data until 11Oct2008:1100

Many sunspikes removed by interpolation from 11Oct to 18Nov

Data are excellent during the storm season Dec thru Feb.

Sunspikes reappear in March.  Removed by interpolation