HHB 2005 Annual load estimation

hhb05.flo <- read.flo("hhb",05)

hhb05.lab <- read.lab("hhb",05)

hhb05.sed <- merge.flo("hhb",05)

nonstorms <- drop.storms("hhb",05)

dim(nonstorms)

[1] 29149     4

head(nonstorms, 5)

chr      q turb turbcode

1 (11/26/04 17:50:00) 0.2716   NA        2

2 (11/26/04 18:00:00) 0.2832   NA        2

3 (11/26/04 18:10:00) 0.3301   NA        2

4 (11/26/04 18:20:00) 0.3539   NA        2

5 (11/26/04 18:30:00) 0.3898   NA        2

> table(nonstorms$turbcode)

0     2     5

27985  1123    41

1123 values of bad turbidity out of 19149 intervals to be estimated

trb05.flo <- nonstorms[!is.na(nonstorms$turb), ]

flo05.flo <- nonstorms[is.na(nonstorms$turb), ]

dim(trb05.flo)

[1] 28026     4

dim(flo05.flo)

[1] 1123    4

28026+1123

[1] 29149

# Develop model for good turb data

range(trb05.flo$turb)

[1]   0 286

table(hhb05.sed$labcode2)

0   3   4   7

177   1   1   1

# 3&4 = minro weighing errors 7=major weighing error

# Eliminate bad turbidity and lab codes

trb05.sed <- hhb05.sed[!is.na(hhb05.sed$turb) & hhb05.sed$labcode2 != 7, ]

turbsscplot("trb",05)

# Restrict range to range of non-storm turbidity

trb05.sed <- trb05.sed[trb05.sed$turb < 300,]

turbsscplot("trb",05,type="power")

table(trb05.sed$turbcode)

0   5

267   2

# Try a few different models for comparison

turb1 <- turbsrc("trb",05,040801,0000,050731,2400,type="power",var=F)

turb2 <- turbsrc("trb",05,040801,0000,050731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",05,040801,0000,050731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",05,040801,0000,050731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat   n    r2      s cv

turb1 12/04/04 13:00 07/16/05 14:40      power turb 474296.1 156 0.941 22.505 NA

turb2 12/04/04 13:00 07/16/05 14:40 logxy:mvue turb 509515.7 156 0.897  0.329 NA

turb3 12/04/04 13:00 07/16/05 14:40      loess turb 428717.5 156 0.939 23.096 NA

turb4 12/04/04 13:00 07/16/05 14:40     linear turb 422789.4 156 0.936 23.555 NA

> sum(turb3$predssc < 0)

[1] 16047

> sum(turb4$predssc < 0)

[1] 18231

# Can't use linear or loess: too many negative predictions

# Power model is the best fit

total(turb1)

start            end  type surr     yhat   n    r2      s cv

turb1 12/04/04 13:00 07/16/05 14:40 power turb 474296.1 156 0.941 22.505 NA

Total load = 474296 kg

# Should be quite a good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo05.flo$q)

[1] 0.2716 10.6200

> range(hhb05.sed$q)

[1] 5.768 1953.000

sum(hhb05.sed$q < 10.62)

[1] 2

# There are only 2 data points below 10.62 cfs

qsscplot("hhb",05)

qsscplot("hhb",05,type="loess")

# Eliminate data above 25 m3/sec so it will remain linear

flo05.sed <- hhb05.sed[hhb05.sed$q < 25*35.315, ]

qsscplot("flo",05)

qsscplot("flo",05,type="loess")

flow1 <- flowsrc("flo",05,040801,0000,050731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",05,040801,0000,050731,2400,type="power",var=F)

flow3 <- flowsrc("flo",05,040801,0000,050731,2400,type="loess",var=F)

flow4 <- flowsrc("flo",05,040801,0000,050731,2400,type="linear",var=F)

total(flow1,flow2,flow3,flow4)

start            end       type surr     yhat   n    r2      s cv

flow1 11/26/04 17:50 12/04/04 12:50 logxy:mvue flow  473.640 160 0.554  0.713 NA

flow2 11/26/04 17:50 12/04/04 12:50      power flow  463.941 160 0.430 87.087 NA

flow3 11/26/04 17:50 12/04/04 12:50      loess flow 1917.388 160 0.430 87.281 NA

flow4 11/26/04 17:50 12/04/04 12:50     linear flow 1981.331 160 0.430 87.051 NA

sum(flow3$predssc <0)

[1] 0

> sum(flow4$predssc <0)

[1] 0

#logxy looks like the best model to me, highest r2 by far

#and it should fit the low flow data best

total(flow1)

start            end       type surr   yhat   n    r2     s cv

flow1 11/26/04 17:50 12/04/04 12:50 logxy:mvue flow 473.64 160 0.554 0.713 NA

Total load = 474 kg

==================================================

# Total non-storm load

474296 + 474 = 474770 kg = 475 metric tons

# Total storm load = 4910054 kg = 4910 metric tons

# Total annual load

4910 + 475 =  5385 metric tons total annual load

# Storm load is 91.2% of annual load

4910/5385

[1] 0.911792