Estimating the error of the calculated annual suspended sediment loads

There are many sources of error in estimating suspended sediment loads and these depend on the procedures employed.  The following outline lists the primary sources of error in estimating loads from TTS data sets.  Those that are likely most important for Salmon Forever are highlighted in yellow and those that may occasionally be significant are highlighted in cyan.

- Stage-discharge rating equation

  - Inadequate data set

    - Data not representative of full range of discharge

    - Data not representative of full time period (if rating equation was shifting)

    - Inadequate number of discharge measurements to estimate rating equation reliably

  - Bias in electronic stage/or and discharge

    - Transducer not indexed accurately to staff plate

    - Transducer drift (changes in calibration)

    - Current meter spins too slow

    - Discharge not measured at staff plate location

  - Regression model has high variance (can be statistically estimated)

    - Errors in reading staff plate and derivation of mean gage height

    - Measurement errors in discharge field procedure or calculations

    - Random scour and fill occurred during time period represented by regression data

  - Regression model is inappropriate for the data

    - Selected model does not fit entire range of data

    - Relationship shifted but only a single model was applied

- Sediment sampling and estimation

  - Inadequate data set

    - Data not representative of full range of SSC

    - Data not representative of full time period (esp. when ISCO is out of bottles)

    - Inadequate number of samples were collected to reliably estimate relation of SSC to turbidity (in small events or when ISCO is out of bottles)

  - Bias in SSC and turbidity

    - Unrepresentative pump intake location if SSC not spatially uniform

    - Inadequate pumping sampler line velocity for coarse particles

    - Algal growth from delays in processing unrefrigerated samples

    - Turbidity sensor drift (calibration changes)

  - Regression model has high variance (can be statistically estimated)

    - Pump sample malfunctions

      - Inadequate purge cycle (residual sediment left in pump tubing leading to cross-contamination between samples)

      - Overfilled bottles

    - Turbidity measurement errors

      - Minor biofouling, not corrected

      - Interpolation and reconstruction errors

      - Changes in T-probe location if turbidity not spatially uniform

    - SSC (lab) measurement errors

      - Spillage of sediment or sample

      - Weighing errors

      - Bookkeeping errors (e.g. mislabeled samples)

      - Calculation errors

  - Regression model is inappropriate for the data

    - Selected model does not fit entire range of the data

    - Relationship shifted but only a single model was applied

    - Model is based on external data due to lack of samples

For Salmon-Forever I have employed a heuristic semi-quantitative method of estimating errors.  As the load for each storm and interstorm period is computed a quality rating is assigned to each, accounting for errors related to the number and timing of samples and statistical modeling of the relationships between turbidity (or discharge) and SSC.  The coefficient of variation is usually available from regression-estimated loads and this value is taken as a starting point, then modified depending on specific circumstances, such as those listed in 2a, 2c, and 2d above, that could compromise the statistical estimate of error.  The variation in loads between alternative regression models is strongly weighted. For semi-quantitatively estimating errors, qualitative ratings are quantified as follows: poor = 25%, fair = 16%, good = 10%, very good = 5%, excellent = 2%.

Error estimates can be propagated through computational steps using a few simple statistical rules governing variances.  For any two independent random variables A and B, with means MA and MB  and variances VA and VB, and any constant k, we have

VA+B=VA+VB	(1)

VAB=VaVB+VAMB2+VBM	(2)

VkA=k2VA	(3)

Since individual loads are summed to arrive at an annual load, their errors can be combined statistically using equation (1) above by assuming the errors in the storm and interstorm loads are independent. These calculations and those described below are displayed in worksheets for each station and year in Error Summary.xls.  The individual variances are estimated as the squares of the semi-quantitative error estimates.  However this estimate of the error of an annual sediment load does not include errors due to the stage-discharge rating equation (items 1a, 1b, 1c, and 1d above) or those due to unrepresentative pumped samples (as in 2b above).

The error from these three sources can be combined statistically by expressing the sediment load as the product of three quantities, i.e. L=bQ {c, where L is the annual load, b represents the bias due to unrepresentative pumped samples, Q is the annual discharge, and c is the annual discharge-weighted concentration.  If the errors in these quantities are independent then the variance of the product can be computed from the individual variances and means using equation (2) above. For the means we assumeMb=1, and use the calculated discharge and sediment loads to estimateMQ=Q, and Mc=L/Q.

The bias due to possibly unrepresentative pumped samples is not known at this time.  For the purpose of error estimation, we assume b has a mean of 1 and standard deviation of 0.08.  In other words, we are assuming that this source would typically result in about +/- 8% error in any given year.  As better information becomes available from analyzing the relation between simultaneous pumped and depth-integrated samples, it can easily be  incorporated into the spreadsheet calculations.

We can conservatively approximate the error in c by treating Q as a constant, so that all the variability in the load is attributed to c.  The variance of c is then estimated using equation (3) above, i.e. by dividing the estimated variance of the annual load (which was obtained by summing semi-quantitative variances of individual loads) by the square of discharge.

We can estimate the error in Q by considering the variability in the stage-discharge rating.  The error of an individual fitted value in a regression can be estimated using standard equations.  It is assumed that, if the rating equation is biased, positive and negative errors in the fitted values would not fully offset each other, and the expected total error would be some sort of average of the absolute values of the individual errors.  Since Q is the sum of an entire year’s worth of fitted discharge values, a logical estimate of the error in Q can be obtained as the discharge-weighted mean of the standard errors of the fitted values.  That is, the standard error (always a positive number) of each of the fitted values in the year is weighted by discharge in calculating a mean.

If the regression response is the natural logarithm of Q,  the standard error, s, of a fitted value represents a difference between logarithms, so the antilog of s, exp(s), represents a ratio between the untransformed observed and fitted values. Modeling the regression response as a logarithm then leads to a simple way of estimating relative error. The dependent variable for the rating equations that have been used so far for these stations is Q, rather than log(Q).  However, these curves can be approximated by equations for log(Q).  The rating curves and approximating curves are shown in the document Rating Curves.doc.  In some cases, to achieve a good fit to the data, separate regressions were computed for two different ranges of stage. The approximating curves were used to calculate the discharge-weighted mean standard error s. The expected value of positive relative errors is given by exp(s)−1 and the expected value of negative relative errors is given by 1−exp(−s), therefore the expected average of all relative discharge errors can be estimated as

eQ=0.5exp(s)−1+1−exp(−s)=0.5exp(s)−exp(−s)

This is a good estimate if the approximating curve has similar residual error as the rating equation.  However, there is usually some difference.  To correct for such differences, the root mean squared error (RMSE) of both the rating curve and approximating curve were calculated and their ratio was multiplied by eQ.  For example, if the rating curve had 10% smaller RMSE, then the value of eQ calculated from the approximating curve was multiplied by 0.90.

Many of the potential errors in discharge will be reflected in the variance of the rating curve regression, hence in the discharge-weighted mean error s.  These would include, for example, random scour and fill during the period encompassed by the rating curve data set, and most errors in reading the staff plate and in carrying out discharge measurements.  Other errors will not be fully accounted for by s.  For example, it is very difficult to accurately measure discharge when overbank flow occurs during flood stages. There may be few or no measurements under these conditions, and the change in discharge is not expected to be the same, for a given change in stage, than at lower flows.  Therefore, use of the discharge rating curve at flood stages is likely to result in greater errors, especially at stages beyond the range of the rating curve measurements.  Extrapolation above the range of the data used in development of the rating equations was required in 13 of 22 data sets. The extent of extrapolation is shown for each station and year by the vertical dashed lines on the graphs in Rating Curves.doc. Although it is impossible to accurately quantify the influence of extrapolation; we know that the error is more severe when predictions are made further beyond the range of the data set used to develop the regression.  Rather than entirely ignoring extrapolation error, I assumed that discharge errors would be increased by the following arbitrary percentages, depending on how high each flow exceeded the highest discharge measurement used in the regression.

| Extrapolation above highest flow measurement | Assumed error, ei, in discharge |
| --- | --- |
| 0-10% | 15% |
| 10-20% | 20% |
| 20-40% | 30% |
| > 40% | 40% |

I multiplied the extrapolation errors by the proportion of flow occurring in each of these classes to determine the additional extrapolation error attributable to each flow class.  To summarize the adjustment for RMSE and extrapolation errors, the adjusted value of annual discharge error can be written as

eQ'=RMSErating curveRMSEapproximationeQ+i=14piei

where pi  and  ei are the proportion of annual flow and discharge error in each of the 4 flow classes defined in the table above. The variance of Q was estimated as the square of eQ' in calculating the error of the product L=bQ {c, using equation (2).  The calculation of eQ' was performed in R, and is documented in the following files:

- Discharge error analysis SFM030405.doc

- Discharge error analysis SFM060708.doc

- Discharge error analysis KRW.doc

- Discharge error analysis FTR030405.doc

- Discharge error analysis FTR060708.doc

- Discharge error analysis HHB.doc

- Discharge extrapolation error.rtf

The results of all these calculations are listed on the summary page of the document Error Summary.xls.  Referring to the summary page, it is apparent that the dominant sources of error are those due to the discharge rating equation. The error due to unrepresentative pumped samples should be small in these streams which are well-mixed and dominated by fine sediments that are easily sampled by a pumping sampler.  For this analysis, we have conservatively assumed that it is 8%, but it could be quantified through analysis of simultaneous pairs of pumped and depth-integrated samples.  Salmon-Forever has been collecting such data, but it is not yet available for analysis. The smallest sources of error are those due to sampling (the number and timing of samples) and statistical modeling (primarily regression).  These results signal a major triumph for TTS whose express purpose is to reduce sampling and modeling errors. In traditional monitoring programs  with no turbidity monitoring and little or no sampling during storm events, such errors are by far the most important, commonly in the 30-60% range.