Stage corrections KRW03 - Jack Lewis

# First I added ORs at 26Mar2003:1010 and 26Apr2003:1020

# Note stage offset was changed 4 times:  16Nov02, 21Jan03, 27Jan03, and 29Jan03

plot.or("krw",03)

First investigate outliers |diff| > 0.1

chr dump staff rawstg corstg rawdiff

11 (01/29/03 13:50:00)    9  2.80  3.100   2.80  -0.300

18 (03/26/03 20:20:00)   14  6.77  6.920   6.92  -0.150

20 (04/14/03 12:40:00)   16  6.68  6.959   6.68  -0.279

21 (04/25/03 17:50:00)   17 13.71 13.910  13.91  -0.200

Jan 29: Rand was present, offset was reduced by 0.38.

Mar 26: No evidence of error.  At 1010 same day diff at much higher stg was -0.05

Apr 14: No evidence of error

Apr 25: No evidence of error

Need to deal with record in pieces, between stage offset changes.

Estimate Std. Error t value Pr(>|t|)

(Intercept) -0.003437   0.019609  -0.175   0.8658

elecstg     -0.011331   0.005523  -2.052   0.0793 .

-0.003437-0.01133*(1:8)

[1] -0.014767 -0.026097 -0.037427 -0.048757 -0.060087 -0.071417 -0.082747 -0.094077

Fixed thru 21Jan2003 by two stage-proportional shifts.

The large offset changes on 27Jan and 29Jan nullified each other. I have to assume that two-day segment and the OR on 27 Jan must have been in error, because subsequent errors are generally small.

The remaining errors are mostly at stages above 6 feet, but a stage-proportional adjustment isn't real satisfying.

Made time variable adjustments from 16Mar2003 to 27Apr2003

SSC corrections KRW03  Jack Lewis

No TTS samples were collected until dump 10, 29Jan2003;  However 131 fixed-time samples

were collected before TTS sampling was begun.

First I entered the 131 time-mode samples into the .isc file with dump numbers corresponding to the

flo file (dumps 4 to 9), and sequential bottle numbers.  All existing bottle numbers were set to zero in

flo file, and bottle numbers were entered for records matching the pump sample times.

Turbidity corrections KRW03  Jack Lewis

OBS-3 installed 12Dec2002

Clark reconstructed much turbidity data, initially using relationships between Hach and OBS

turbidity, based on pumped samples.  Later reconstructions freehanded.

JL replaced with raw data on 19Dec02: 1530 to 1630.