HHB 2008 Annual load estimation

hhb08.flo <- read.flo("hhb",08)

hhb08.lab <- read.lab("hhb",08)

hhb08.sed <- merge.flo("hhb",08)

nonstorms <- drop.storms("hhb",08)

dim(nonstorms)

[1] 25549     4

head(nonstorms, 5)

chr     q turb turbcode

1 (10/20/07 15:10:00) 137.6   52        0

2 (10/20/07 15:20:00) 134.0   52        0

3 (10/20/07 15:30:00) 132.7   52        0

4 (10/20/07 15:40:00) 132.2   51        0

5 (10/20/07 15:50:00) 129.7   52        0

> table(nonstorms$turbcode)

0     1     5     6

24591    10    12   936

10 questionable values of bad turbidity out of 25549 intervals to be estimated

None bad (code=2), 936 reconstructed from another site (KRW)

# Copy hhb08.flo to trb08.flo as is

trb08.flo <- nonstorms

# Develop model for good turb data

range(trb08.flo$turb)

[1]  0  194

range(hhb08.sed$turb)

[1]   0 703

table(hhb08.sed$labcode2)

0   1   9

105  23   1

# Code 9 means spilled > 15% of sediment mass during or after filtering

# Scatterplots show that bottle is not an outlier

turbsscplot("hhb",08,txt="dump")

# Eliminate the following previously identifed outliers

Dump  Bot

12    11

14     7

14     8

trb08.sed <- hhb08.sed[!(hhb08.sed$dump==12 & hhb08.sed$bottle==11), ]

trb08.sed <- trb08.sed[!(trb08.sed$dump==14 & trb08.sed$bottle %in% 7:8), ]

# Eliminate samples at 0 NTU and above 200 NTU

# These samples are beyond the prediction range but could influence regression a lot

trb08.sed <- trb08.sed[trb08.sed$turb > 0 & trb08.sed$turb < 200, ]

turbsscplot("trb",08)

turbsscplot("trb",08,type="power")

turbsscplot("trb",08,type="logxy")

# Try a few different models for comparison

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="linear",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb4 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat   n    r2      s cv

turb1 10/20/07 15:10 05/04/08 13:40     linear turb 723445.7 108 0.827 26.778 NA

turb2 10/20/07 15:10 05/04/08 13:40      loess turb 720542.1 108 0.838 26.110 NA

turb3 10/20/07 15:10 05/04/08 13:40      power turb 736410.7 108 0.839 25.874 NA

turb4 10/20/07 15:10 05/04/08 13:40 logxy:mvue turb 755329.6 108 0.809  0.421 NA

> sum(turb1$predssc < 0)

[1] 17705

sum(turb2$predssc < 0)

[1] 14686

quantile(trb08.flo$turb,seq(0.90,0.99,0.01))

90% 91% 92% 93% 94% 95% 96% 97% 98% 99%

36  38  40  42  45  47  52  59  70  92

# Can't use linear or loess because of so many negative predictions

# The power and logxy estimates differ only by 2.6%, both are very good

# Because 90% of the predicting is below 36 NTU, use the logxy model

total(turb4)

start            end       type surr     yhat   n    r2     s cv

turb4 10/20/07 15:10 05/04/08 13:40 logxy:mvue turb 755329.6 108 0.809 0.421 NA

Total load = 755330 kg  Good

==================================================

# Total non-storm load = 755 metric tons

# Total storm load = 3291160 kg = 3291 metric tons

# Total annual load

3291 + 755 = 4046 metric tons total annual load

# Storm load is 3291/4046 = 81.3% of annual load