Data corrections HHB08 - Jack Lewis

Dump 2 data was lost.

Observer record corrections – Jack Lewis

Changed

2008,02,01,1220,12,4.45

to

2008,02,01,1720,12,4.45

based on electronic stage and turbidity recorded on FF

Rating equation

Stage corrections - Jack Lewis

plot.or("hhb",08)

OR must be incorrect.  See correction under Observer records above.

After correction:

That point is still somewhat anomalous.

plot.or("hhb",08,x="stage")

No consistent trend of OR differences with time or stage.

Adjustments handled on an ad hoc basis:

Stage offset was changed on 17Nov.  Entire record prior to this date

Time-variable shifts from 20Oct to stage offset change on 17Nov

No changes from 17Nov to 12Dec

Variable shifts averaging about -0.02 from 12 Dec to 28Jan

Variable shift of about -0.04 from to 28Jan to 12Mar

Variable shift from -0.039 to 0 from 12Mar to 04May

Final result:

plot.or("hhb",08,cor=T)

Turbidity corrections - Jack Lewis

Turbidity data are generally excellent for HHB08

Interpolated missing data at 17Nov2007:1340-1450

Reconstructed from 27Nov2007:1320 to 29Nov2007:0840 from HHB (self), using

linear relation and lag of -11900 i.e. copied the recession of 19Nov.

There's a bad section from 26Feb08:1650 to 28Feb2008:1230.  Field note says

boom may have been lowered too far.  I ended up reconstructing this from KRW

using linear relationship.  This clarified that bottles 7 and 8 were probably

bad bottles related to the high volumes at bots 5 and 6.

Samples - quite a few mismatches