| ## Contact Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First Name: |  |  | Jesse |  |  |  |  |  | Last Name: |  |  | Noell |  |  |  |  |  |  |  |
| Organization: |  |  | Salmon Forever |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mailing Address: |  |  | 8050 Elk River Rd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Eureka, CA 95503 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Email: |  |  | jnoell@copper.net |  |  |  |  |  |  |  |  |  |  | Preferred Contact Method: |  |  |  |  |  |
| Phone: |  |  | (707) 445-9555 |  |  |  |  |  |  |  |  |  |  | Email |  |  | Phone |  |  |
| ## Submission Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Submittal Date: |  |  |  |  |  | 6/30/2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Region data intended for: (Check all that apply.) |  |  |  |  |  | (1) North Coast |  |  |  |  | (2) San Francisco |  |  |  |  | (3) Central Coast |  |  |  |
|  |  |  |  |  |  | (4) Los Angeles |  |  |  |  | (5) Central Valley |  |  |  |  | (6) Lahontan |  |  |  |
|  |  |  |  |  |  | (7) Colorado River |  |  |  |  | (8) Santa Ana |  |  |  |  | (9) San Diego |  |  |  |
| GIS map layers included: |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pollutant Categories: |  |  |  |  |  | Hydromodification |  |  |  | Other Organics |  |  |  |  | Toxicity |  |  |  |  |
|  |  |  |  |  |  | Metals/Metalloids |  |  |  | Pathogens |  |  |  |  | Trash |  |  |  |  |
|  |  |  |  |  |  | Nuisance |  |  |  | Pesticides |  |  |  |  | Miscellaneous |  |  |  |  |
|  |  |  |  |  |  | Nutrients |  |  |  | Salinity |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Other Inorganics |  |  |  | Sediment |  |  |  |  |  |  |  |  |  |
| Time Period Data Collected: |  |  |  |  |  | 11/16/2002 – 6/24/2008 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of Data: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This package consists of water and sediment discharge data for HY2003 through HY2008 from four stream gaging stations on Freshwater Creek and Elk River, tributaries to Humboldt Bay. Two gaging stations in each watershed were operated using the Turbidity Threshold Sampling system. Gaging station locations are South Fork Elk at Jesse Noell's house (SFM), North Fork Elk at Kristi Wrigley's house (KRW), Freshwater Creek at Terry Roelofs' house (FTR), and Freshwater Creek at Howard Heights Bridge (HHB). Storm event loads were determined for 64 storm events at SFM, KRW, and FTR, and 44 events at HHB, which was started later in HY2005. Interstorm loads were also computed and added to storm loads to obtain annual sediment loads. Over 5000 pumped samples were included in the computation of storm loads. A subset of samples was analyzed for sand fraction, and the percentage of load as sand (> 0.063 mm) was determined for 3 years at FTR and HHB, and 2 years at SFM and KRW. As a byproduct of the analysis, a record of SSC was produced at 10-minute intervals for all stations, from which quantiles of SSC and exceedence durations were extracted at various levels. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Document format: |  |  |  | Electronic |  |  | Hard Copy/Paper |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Internal Use Only |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reviewer: | First Last |  |  |  |  |  |  |  |  |  |  |  | Date Reviewed: |  |  |  |  | 1/1/2001 |  |
| Status: | Returned |  |  |  | Flagged |  |  | Forwarded |  |  |  |  |  |  |  |  |  |  |  |
| Comments: | Enter status comment here. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reviewer: | First Last |  |  |  |  |  |  |  |  |  |  |  | Date Reviewed: |  |  |  |  | 1/1/2001 |  |
| Status: | Returned |  |  |  | Flagged |  |  | Forwarded |  |  |  |  |  |  |  |  |  |  |  |
| Comments: | Enter status comment here. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Control #: |  | Enter number |  |  |  |  |  |  |  |  |  |  |  | Date Received: |  |  |  |  | 1/1/2001 |
| Reference #: |  | ref#### (use semicolons to separate additional reference numbers) |  |  |  |  |  |  |  |  |  |  |  | QC Complete: |  |  |  |  | Yes |