HHB 2006 Annual SAND and FINES load estimation

hhb06.flo <- read.flo("hhb",06)

hhb06.lab <- read.sand("hhb",06)

hhb06.sed <- merge.flo.sand("hhb",06)

# Now create the ssc column: it will represent only FINES

hhb06.sed$ssc <- hhb06.sed$totssc - hhb06.sed$sand

# Since it's named ssc, we can use existing functions to calc the fines load

head(hhb06.sed,6)

chr dump bottle sand totssc turb turbcode     q labcode1 labcode2   ssc

1 (11/06/05 12:10:00)    1      9 29.7  202.4  190        0 101.4        0        0 172.7

2 (11/06/05 14:30:00)    1     10 25.2  157.5  148        0 147.3        0        0 132.3

3 (11/07/05 10:00:00)    1     14 27.7  208.2  177        0 298.8        0        0 180.5

4 (11/07/05 11:40:00)    1     15 27.7  223.8  158        0 321.0        0        0 196.1

5 (11/14/05 05:30:00)    2      3 12.2  161.2  100        0 311.1        0        0 149.0

6 (12/01/05 01:10:00)    4      1  8.9  118.2  118        0 104.7        0        0 109.3

summary(hhb06.sed$ssc)

Min. 1st Qu.  Median    Mean 3rd Qu.    Max.

56.8   201.7   325.5   424.0   580.3  1349.0

dim(hhb06.flo)

[1] 30838     9

table(hhb06.flo$turbcode)

0     1     5     7

29700   914   108   116

There are 914 values of questionable turbidity and none bad out of 30838 intervals to be estimated.  Most were 15-20Oct2005 or 25-29Mar2006

The Oct values are below 12 NTU and inconsequential

The Mar values are mostly corroborated by the SSC

# No need for a flo06.flo; there are no missing turbidity data

# Develop model for good turb data

range(hhb06.flo$turb)

[1]    1 1194

range(hhb06.sed$turb)

[1]   62 1072

# Referring back to "hhb06 Annual load.rtf", there are no bad lab codes to worry about

table(hhb06.sed$turbcode)

0   5

116   1

# 5=interpolated

turbsscplot("hhb",06,type="loess")

# Quite a lot of scatter (and this is fines only!)

four()

turbsscplot("hhb",06,txt="dump")

turbsscplot("hhb",06,txt="dump",type="power")

turbsscplot("hhb",06,txt="dump",type="logxy")

turbsscplot("hhb",06,txt="dump",type="loess")

# Eliminate the logxy outlier

turb1 <- turbsrc("hhb",06,050801,0000,060731,2400,type="power",var=F)

turb2 <- turbsrc("hhb",06,050801,0000,060731,2400,type="logxy",var=F)

turb3 <- turbsrc("hhb",06,050801,0000,060731,2400,type="loess",var=F)

turb4 <- turbsrc("hhb",06,050801,0000,060731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat   n    r2       s cv

turb1 10/08/05 14:00 05/10/06 17:30      power turb 12693947 117 0.849 109.973 NA

turb2 10/08/05 14:00 05/10/06 17:30 logxy:mvue turb 12048817 117 0.868   0.231 NA

turb3 10/08/05 14:00 05/10/06 17:30      loess turb 11611117 117 0.861 105.928 NA

turb4 10/08/05 14:00 05/10/06 17:30     linear turb 13048949 117 0.844 111.582 NA

# 12% difference between largest and smallest

> sum(turb3$predssc < 0)

[1] 10347

> sum(turb4$predssc < 0)

[1] 0

# Cannot use loess because of negative predictions

# The linear model has higher variance than the power model

# So choice is between logxy and power.  These differ by 6%

# Power model is lower at upper end, yet has gives higher load

# My impression is that the power model overpredicts in the range below 250NTU

total(turb2)

start            end       type surr     yhat   n    r2     s cv

turb2 10/08/05 14:00 05/10/06 17:30 logxy:mvue turb 12048817 117 0.868 0.231 NA

Total fines load = 12048817 kg

# A reliable estimate for fines -- should definitely be within 10%

==================================================

# Now compute the total annual load based on this smaller sample

# Used only the 117 samples (out of 218) that had sand fractions done

hhb06.sed$ssc <- hhb06.sed$totssc

four()

turbsscplot("hhb",06,txt="dump")

turbsscplot("hhb",06,txt="dump",type="power")

turbsscplot("hhb",06,txt="dump",type="logxy")

turbsscplot("hhb",06,txt="dump",type="loess")

turb1 <- turbsrc("hhb",06,050801,0000,060731,2400,type="power",var=F)

turb2 <- turbsrc("hhb",06,050801,0000,060731,2400,type="logxy",var=F)

turb3 <- turbsrc("hhb",06,050801,0000,060731,2400,type="loess",var=F)

turb4 <- turbsrc("hhb",06,050801,0000,060731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat   n    r2       s cv

turb1 10/08/05 14:00 05/10/06 17:30      power turb 15833556 117 0.736 230.890 NA

turb2 10/08/05 14:00 05/10/06 17:30 logxy:mvue turb 15035396 117 0.849   0.272 NA

turb3 10/08/05 14:00 05/10/06 17:30      loess turb 12720522 117 0.748 226.244 NA

turb4 10/08/05 14:00 05/10/06 17:30     linear turb 14471455 117 0.737 230.117 NA

> sum(turb3$predssc < 0)

[1] 22751

> sum(turb4$predssc < 0)

[1] 15498

# Can't use the linear or loess models because they go negative

# The power and logxy models agree to 6%, with the logxy model being superior

total(turb2)

start            end       type surr     yhat   n    r2     s cv

turb2 10/08/05 14:00 05/10/06 17:30 logxy:mvue turb 15035396 117 0.849 0.272 NA

Total load = 15035396 kg

This total agrees very well with the previous estimate

of 15076659 based on sum of storms and inter-storm loads

==================================================

==================================================

Total load 15035396

Total fines 12048817

Total sand  2986579

Percent sand = 19.9%

Percent fines= 80.1%