Stage corrections KRW05 - Jack Lewis

Discharge rating equation goes negative at stage < 0.72, but minimum stage  for the year is higher.

Stage corrections - Jack Lewis

31Oct2004:1410 to 03Nov2004:0910 reconstructed from SFM using linear relation.

08Dec2005:1500 to 08Dec2005:1820 reconstructed from SFM using time-linear ratio.

28May2005:1300 to 14Jun2005:0930 reconstructed from SFM using linear relation.

Oct 23 staff plate reading is off by 0.84 feet!  Must be an observer error.  Except for this and a couple of other outliers, there is a clear trend in the errors with stage:

Remove the big outlier and fit a linear trend.

Krw05.or <- krw05.or[-4,]

Stage-proportional correction is needed.  Linear trend is

Estimate Std. Error t value Pr(>|t|)

(Intercept)  0.0003134  0.0142461   0.022    0.983

elecstg     -0.0147154  0.0025273  -5.822 5.34e-07 ***

0.0003 -0.014715 * (1:17)

[1] -0.014415 -0.029130 -0.043845 -0.058560 -0.073275 -0.087990 -0.102705

[8] -0.117420 -0.132135 -0.146850 -0.161565 -0.176280 -0.190995 -0.205710

[15] -0.220425 -0.235140 -0.249855

After making stage-proportional corrections to entire year:

Errors are generally small except for the previously noted outlier and the 0.24 (May 9) and -0.21 (Jan 15).  The same is true with stage as the “x” axis.  I do not have a field form for May 9.  These errors are isolated and contradict neighboring observer readings, so I will regard them as staff plate reading errors and make no further corrections.

Turbidity corrections - Jack Lewis

31Oct2004:1410 to 08Dec2004:1810 DTS12 not responding. Field notes provide no clue and imply that observers were unaware of the problem!  I replaced all turbidity data, including Clark’s reconstructions, with missing.

26Dec2004:1100 to 29Dec2004:1440 DTS12 again not responding.  This time DTS-12 was replaced with loaner and problem appears to be fixed.

14Jan2005 to 18Jan2005 noise removed by interpolation.

17apr2005:1240 to 22Apr2005:0540 noise removed by reconstruction from SFM using linear relation.

25Apr2005 to 02May2005 noise removed by interpolation.

26May2005:0130 to 14Jun2005:0930 reconstructed from HHB using linear ratio (HHB only station with good turbidity data).  This is a long period but nothing was going on. Would have interpolated except for the tiny peak on June 9.