SFM DISCHARGE ERROR ANALYSIS HY06-08

# Calculate weighted average error using weighthalfwidth function

# Weights the error in the fitted value of discharge using the

# discharge values that occurred in a given year.

qsfm <- read.csv("sfm/Q and rating curves/sfmratingdata0308.csv")

attach(qsfm[c(1:15,19:26),])

sfmfit <- lm(I(q+32) ~ -1+stg+I(stg^2))

Coefficients:

stg  I(stg^2)

19.266     2.583

plot(stg,q)

qfunsfm07 <- function(s) 2.5828*s^2+19.266*s-32

x0 <- seq(0,17,0.2)

y0 <- qfunsfm07(x0)

lines(x0,y0,col=1)

# Develop model for log(q)

fit1 <- lm(log(q) ~ log(stg))

fit2 <- lm(log(q) ~ log(stg) + I((log(stg))^2))

fit3 <- lm(log(q) ~ log(stg) + I((log(stg))^2) + I((log(stg))^3))

fit4 <- lm(log(q) ~ I(log(stg)^0.25))

y1 <- predict(fit1,newdata=data.frame(stg=x0))

y2 <- predict(fit2,newdata=data.frame(stg=x0))

y3 <- predict(fit3,newdata=data.frame(stg=x0))

y4 <- predict(fit4,newdata=data.frame(stg=x0))

lines(x0,exp(y1),col=2)

lines(x0,exp(y2),col=3)

lines(x0,exp(y3),col=4)

lines(x0,exp(y4),col=6)

legend(locator(1),c(”Rating equation”,”log(q) ~ poly(1)”,”log(q) ~ poly(2)”,”log(q)  ~ poly(3)”,”log(q) ~ log(stg)^0.25”),lty=1,col=c(1:4,6))

The log(q) ~ log(stg)^0.25 is closest model to the original rating.  How does it compare in terms of RMSE?

q1 <- qfunsfm07(stg)

q2 <- exp(predict(fit4,newdata=data.frame(stg)))

sqrt(mean((q-q1)^2))

[1] 35.54304

sqrt(mean((q-q2)^2))

[1] 40.01989

# Has somewhat greater error than the original rating equation

# SFM06

m2 <- (log(sfm06.flo$stg))^0.25

weighthalfwidth(fit3, m1, m2, sfm06.flo$q)

[1] 0.1056029

avg.err(0.1056029)

[1] 0.1057993

# Adjust for RMSE difference

0.1057993 * 35.54/40.02

[1] 0.0939557

# Adjusted estimated average error = 9.4%

# SFM07

m2 <- (log(sfm07.flo$stg))^0.25

weighthalfwidth(fit3, m1, m2, sfm07.flo$q)

[1] 0.1033220

avg.err(0.1033220)

[1] 0.1035059

# Result is virtually identical to that of SFM08, error = +/- 6.6%

# Adjust for RMSE difference

0.1035059 * 35.54/40.02

[1] 0.09191903

# Adjusted estimated average error = 9.2%

# SFM08

m1 <- (log(stg))^0.25

m2 <- (log(sfm08.flo$stg))^0.25

weighthalfwidth(fit3, m1, m2, sfm08.flo$q)

[1] 0.1034469

avg.err <- function(e) (exp(e) - exp(-e))/2

avg.err(0.1034469)

[1] 0.1036315 # average error

# Adjust for RMSE difference

0.1036315 * 35.54/40.02

[1] 0.09203057

# Adjusted estimated average error = 9.2%