Surface Water Ambient Monitoring Program (SWAMP)

# Suspended Sediment and Turbidity Monitoring

# Freshwater Creek and Elk River

PROCEDURE FOR SURVEYING

SEDIMENT DEPOSITION, SCOUR OF STREAM BEDS AND BANKS

Prepared for:

Salmon Forever

Eureka, CA

Prepared by:

Jesse Noell

Salmon Forever

PO Box 928

Eureka, CA 95502

Version 1.0

May 2008

A1

OPERATING PROCEDURE

Survey Procedures For Determining Sediment Deposition and Scour of Streambed, Streambanks, and Overbank Areas on Elk and Freshwater rivers

Elk River and Freshwater Creek, Humboldt County, California

Salmon Forever 5-4-08

Prepared By: Jesse Noell	Date May, 2008

Approved By___________________________________	Date  _____________________

2.0

Table of Contents

- APPROVAL SIGNATURES

- TABLE OF CONTENTS

- PROCEDURES

  - Scope 	and Applicability

  - Summary of Method

  - Definitions

  - Health and Safety Warnings

  - Cautions

  - Interferences

  - Personnel Qualifications/Responsibilities

  - Equipment and Supplies

  - Suppliers

  - Procedure

    - Instrument or Method Calibration and Standardization

    - Channel Morphology Considerations

    - Layout of Cross-Sections

    - Layout of Soil Pin Arrays

    - Layout of Scour Chains

    - Troubleshooting

    - Data Acquisition, Calculations and Data Reduction Requirements

  - Data and Records Management

- QUALITY CONTROL and QUALITY ASSURANCE

  - QC Self verification

  - QC limits

  - QC procedures for reporting data and results

  - QC Procedures: These self checks include calibrations, recounting. Describe the frequency of required calibration and checks. Discussion / the rationale for decisions.

  - QC Material: Purpose is to demonstrate successful performance of the method. Unique ID numbers. Survey checks on benchmark elevations and x,y coordinates. Survey checks on soil pin station distances, missing pins, bent pins.

- References

APPENDIX

A3

Distribution List

| Title | Name | Phone No. | Hard Copies |
| --- | --- | --- | --- |
| Project Director | Jesse Noell | 707-445-9555 | 1 |
| Licensed Surveyor | Ed Schillinger | 707-442-7046 | 1 |
| NCRWQCB | Bernadette Reid | 707-576-2220 | 1 |
| QA Officer | Tyler Ledwith | 707-269-2058 | 1 |
| Grantee Grant Manager; Co-Director RCAA | Don Allan | 707-269-2063 | 1 |
| Water Resource Control Engineer | Adona White | 707-479-2342 | 1 |
| State Board Quality Assurance Program Manager | William Ray | 916-341-5583 | 1 |

3.0	PROCEDURES

(a.) Scope

This Standard Operating Procedure is written to guide stream monitoring activities to be performed by Salmon Forever in its capacity as subcontractor under a grant to RCAA, from the State Water Resource Control Board administered through the Regional Board. This document details the field methods for conducting surveys of channel topographic characteristics, sediment deposition and scour on the bed, banks and over-bank areas of  Elk River and Freshwater creeks in Humboldt County, California.  One purpose of this grant application is to provide analysis of the capacity of Elk River’s flood reaches to assimilate the existing suspended sediment load and bed-load so that these values can be used to establish a Total Maximum Daily Load or TMDL. The study sites are located on the North Fork of Elk River, South Fork of Elk River, and Main-stem Elk River. The study area encompasses the low gradient reaches that are located downstream of industrial timber operations.

The field survey data will be used to:

- determine changes is cross sectional area over time,

- collect data regarding rates of deposition and scour as a function of: 1) horizontal and vertical distance to the channel thalweg, 2) stream flow magnitude and duration events, 3) channel features, 4) vegetation-induced hydraulic roughness, 5) stream velocity, and 6) flood flow duration,

- determine changes is cross sectional area over time,

- map the lateral flood elevation extents resulting from discrete flow events,

- map inundation of septic systems, structures, fences, crops, roads and houses,

- map potential sites for engineering projects such as sediment deposition basins and/or bank modification to enhance sediment sequestering and removal.

3.0 (b.) Experimental Design for Field Survey, Summary:

In Elk River significant hydrologic changes due to road construction and vegetation removal over the past 15 years in combination with accelerated erosion have been well documented.

The purpose of the proposed field survey is to provide data relevant to the following questions:

- has the channel which has gradually narrowed over the past 10 years begun to rapidly increase the relative height of its banks and perhaps its bed,

- does sediment deposition in over-bank and stream-bank areas of the nuisance flood reaches exceed scour,

- what is the present volumetric rate of sediment storage and/or scour,

- how do FEMA flood map elevation lines compare with discrete flood events,

- how do elevations of septic systems and buildings, and anthropogenic encroachments compare with discrete flood events,

- in what vegetation types are sediment deposition rates the highest and how do they vary as a function of stream geometry, encroachments, and backwater effects,

- where are the best locations for remediation activities designed to reduce suspended sediment loading.

First,  total-station level surveying will be used to characterize present conditions and document the extent to which stream reaches in the study area of the Elk River system change during the flow events of the study period. Where historical information is available the survey data will provide data to estimate the historic extent of change.

Second, bed and bank elevation change surveying will be used to provide increased understanding of how Elk River is routing its sediment and flood waters and could tell us whether or when the channel has reached equilibrium. Surveying will also provide increased understanding regarding the present capacity of the  flood reaches to assimilate the suspended sediment load and bedload.

Third, mapping of discrete floods provides a baseline for risk analysis and emergency response planning for residents living in the low gradient reaches of Elk River.

3.0 (c.) Summary of Methods

This proposed operational standard is primarily based upon procedures from Surveying Principles for Civil Engineers (cite date, authors), and applies these surveying principles to survey cross-sections for representative deposition and scour determination and mapping of high water evidence. These procedures also rely on methods established in “Stabilization of a gravel channel by large streamside obstruction and bedrock bends, Jacoby Creek, Northwestern California” by Thomas E. Lisle of the Pacific Southwest Forest and Range Experiment Station, United States Forest Service, Arcata, California 95521 to characterize data collected at specific locations as a function of its propensity to undergo scour and deposition during flood flows.

3.0 (d.) Definitions

- Cross-section  a linear representation of the channel geometry at right angle to stream flow existing at a given moment in time.

- Thalweg- a profile of the lowest streambed elevations along the stream course.

- Mud-line or debris-line mapping- a lateral delineation of the highest extent of flood inundation established after the flood event based upon evidence of mud or debris left by the high water.

- Scour Chain -steel chain comprised of small links is placed vertically into the stream-bed substrate. When flow events scour the substrate, the scoured links are strewn horizontally downstream, providing a dynamic representation of the vertical extent of sediment eroded and subsequently filled through a river-bed cross-section over a period of time.

- Soil Pin Array  soil pins and washers placed along a cross-sectional through the stream-banks and over-bank areas provide a dynamic representation of sediment routing through a river bank and over bank deposition or erosion over a period of time.

- Secondary Benchmarks:  iron rods and metal t-posts

- Project control benchmarks- vertical and horizontal points with coordinates established by high-resolution GPS survey conducted by a licensed surveyor

  - Water Surface Elevation (WSE) points established from benchmarks

  - Right Edge of Water (REW), Left Edge of Water (LEW) points similarly established

3.0 (e.) Health and Safety SAFETY FIRST!

Establish a safe path to the site: streambanks are soft and slippery.

Be careful. Please don't wade deeper than knee deep in moving water when surveying. Remember, when you go out in the field, you do so as a volunteer and must assume responsibility for your own safety. Please take your time and be careful.

3.0 (f.) Cautions

Data can be compromised from improper use of survey equipment, failure to take accurate or adequate notes, failure to follow survey methods, or an incorrect calibration.

3.0 (g.) Personnel Qualifications

Ed Schillinger, licensed surveyor, will provide consultation for use of the Total Station to Jesse Noell and Kristi Wrigley. Other personnel will receive training administered by the Project Director and will work under the direction of the Project Director or Kristi Wrigley to assure that procedures are followed.  Additionally the QA Officer will periodically review the competency of field personnel

- in calibrating equipment, assuring that levels of accuracy are within tolerance,

- in safety considerations,

- whether the procedures are followed,

- records are taken and kept as required by this guideline,

- how to hold a stadia rod or prism rod vertical

- hand signals and verbal communication

3.0 (h.) Equipment and Supplies

- Cross-section resurvey: Auto Level, Stadia Rod, Tape measure(s), GPS, Rite in the Rain Notebooks

- Cross-section bank pin establishment: Sokkia Total Station and Bezel, Tape measure(s), xxx GPS, Rite in the Rain Notebooks

- Scour chain establishment: Scour chains, auger, sleeve, 300’  tape, plumb bob, notebook, camera, reference stakes

- Scour chain resurvey:  metal detector, survey equipment, shovels, reference photos and notes

- Soil pin array establishment: Steel Dowel Rods of 3/8 inch diameter cut 4 foot in length, large washers with 3/4 inch holes, unique I.D. tags, Enamel paint, Camera, Millimeter scale or ruler. Waterproof boots, Machete, Weed whip, Sledge, Post driver, Hack saw, Chop saw, construction adhesive, Paint pens, Aluminum tree tags, Drywall square, string, light weight plumb bob.

- Soil pin array resurvey: Camera, Millimeter scale and modified triangle desk ruler, string, light weight plumb bob, Aluminum tree tags, 300’ Reinforced nylon tape, metal detector.

3.0 (i.) Equipment Suppliers

All supplies used will be either ordered from the manufacturer or through a scientific supply house or local supply house. Supplies will not be accepted unless in proper working order. All supplies and equipment are purchased and inspected under the supervision of the field leader. Copies of equipment invoices shall be kept in the Sediment Lab and Salmon Forever Office

3.0 (i.) Data needs: an assemblage of local bench marks established by licensed survey including property boundaries, section corners, power poles and bridge abutments for bench marks. Property corner or line monuments are in many cases considered to be "well founded" meaning that the construction of the monument was designed to be stable and permanent. Whereas a bench mark, has a known elevation (z) in addition to position (xy),- a monument may not have an established elevation.

In order to provide for data needs, Project Control will be established by Points West survey using GPS equipment. These monument pairs will be established on the North Fork Elk near cross-sections KRW x-A1 and KRWx-A3, on the North Fork Elk near the concrete bridge, on the ridge between the South Fork and the North Fork Elk near the concrete bridge, on the Mainstem Elk near the confluence, and on the Mainstem Elk near Berta Road and Zane Rd. From this project control, control will be extended to the cross-section bank pins.

3.0 (j). I. Calibration and Standardization:

- Backsight Checks All surveys require closing the loop with a backsight check.

- Auto Level AutoLevel survey instrument requires calibration using the double peg test to be conducted each month, and before use if the instrument has experienced any significant shock. The following steps will be used to conduct the double-peg test:

- Drive two stakes into the ground 100-200 feet apart.

- Set up the auto level approximately half way between the two stakes.

- Foresight each of the stakes and record to +- .0002 using a rod level.

- Compute the difference and record.

- Set up the auto level near one stake. Foresight each of the stakes and record.

- Compute the difference.

- Compare the two computations of difference.

- Calibration and Certification of the Total Station The Sokia total station instrument Set 5-e is calibrated and certified once each year by Pacific Survey Supply of Medford, Oregon or another licensed certifier of Total Station instruments for purposes of this research.

- Extending Control When control is extended to new benchmarks by running courses, individual angles and distances taken by turning the gun clockwise from control will be checked by using a double-angle mean procedure. The check will consist of adding the telescope direct and telescope reversed and comparing the initial value with the mean value. The two electronic distance measurements will also be compared.

- Balancing Closed Traverses Because no matter how carefully any survey is performed not all of the sources of error can be eliminated, for closed traverses, the survey data will be adjusted to balance the error. The following steps will be used: In every extension of control we will show that the relevant closure meets our specification of 1/5000 for closure.

3.0 (j.) II. Channel Morphology Considerations:

- Determine when to establish and monitor the Cross-Sections,  Scour Chains,  Soil Pin Arrays, and document Mud/ Debris lines:

  - Cross-Sections and Scour Chains: Low flow periods are required for survey of cross-sections and installation and/or recovery of scour chains.

  - Soil Pins Arrays: To a lesser degree, low flow periods are required to install soil pin arrays and recover deposition/scour data.  However low flow events during winter and spring are rare. Therefore survey efforts dependent on low flow conditions is best conducted in the summer and fall season.

  - Mud/ Debris lines: Winter and spring high flow events are the appropriate time to establish the lateral extent and elevation of over-bank flood events, and this must be accomplished before rain washes away the mud line. Survey site visibility is best during winter and spring because less vegetative cover exists. Winter and spring low flow periods can be a good time to install bank-full and over-bank soil pins, especially following an over-bank event with heavy sediment deposition. Sediment deposits can be expected to compact with time. Recent storm deposits of silt are expected to compact over the summer period, and organic matter is likely to decay into the soil. Soil pins installed after a large flow with heavy deposition are valuable because they provide information regarding the compaction of sediment deposits.

  - Characteristics of the deposits are to be described in the field notes to help distinguish between incidents of river-scour, compaction, and surface erosion associated with rainfall. Rilling of the sediment deposits should be photographed and dated.

- Characterize the propensity of the Cross-Section to undergo scour:

Using the findings of Lisle, T. E.,  Stabilization of a gravel channel by large streamside obstruction and bedrock bends, Jacoby Creek, Northwestern California, denote the following information on the Channel Morphology in field sheet: 1) Does the cross-section transect a stationary gravel bar located within -1, 3, or 4 channel widths of a large root wad, a large woody debris jam, a channel bend, or a bedrock out crop.  Photo points will be established at each cross-section and identified in the notes including camera, lens, lens setting if zoom lens is used, and a description of the photo point and objects of interest observable in the photo.

- Characterize the propensity of the Cross-Section to undergo scour:

3.0 (j) III	ESTABLISHING THE SITE for cross-sections and soil pin arrays

- Obtain permission from the landowner if a site is to be established on private property.

- Cross-sections spacing should be established at approximately 10 times the bankfull width.

- Naming the Cross-section: Locate an appropriate cross-section site with relation to project control bench marks established by Points West licensed surveyor and give it an alpha-numeric name, or if the cross-section is previously named by Schillinger, 2001, use the previous designation. Three letters followed by digits are preferred with the first two letters indicating the general stream reach, the third letter indicating a specific portion of the general reach, and the numeral indicating the individual cross-section . The third, fourth, and possibly fifth letters commonly indicate a bridge or property ownership. Letters following “x” indicate individual cross-sections within a suite of cross-sections. (SFMxA-4, NFCx-1, MEWx-3 etc.)

- Project Control benchmarks are defined in Universal Transverse Mercator (UTM) Zone 10 North American Datum (NAD) 83 co-ordinate base “UTM Z10 NAD 83” to correspond with the 2005 LIDAR map data set flown for the North Coast Regional Water Quality Control Board.

Establish the Benchmark Cross-section Bank Pins (a.k.a. Bank Pins) by driving (two or more) seven foot tall metal fence “t-posts” three to four feet into the ground or by driving four-foot-long ½” rebar “iron rods” into the ground. Commonly upper and lower bank pins are set. Each Cross-section’s Bank Pins should be located 5 to 30 meters from the channel thalweg, but should not be situated where it will interfere with mowing of hayfields, vehicles, or create a nuisance for animals or people. Use or place t-post and iron rods in the line of existing fence posts where possible. The t-post or iron rod will serve as secondary control once it is surveyed for elevation and coordinates by a survey crew using a total station.

3.0 (j) IV 	Laying out a Soil Pin Array:

a) Place the dowels at approximately 1 meter intervals on a transect located approximately 1 meter downstream or upstream of the cross-section in undisturbed vegetation. The idea is to place the pins off of the traveled path of the cleared and compacted cross-section. Some 1 meter intervals may not be usable for a variety of reasons, and if so, utilize the next acceptable location.

b) To place dowels on the transect: First, pull a tape taunt between the cross-section end point to determine the 1 meter stations. Second, hang a light-weight plumb bob from the cross-section tape at the soil pin station distance, then use a drywall square (horizontal orientation) to locate the transect line and soil pin station point.

c) Use a 4 foot “drywall square” to orient the pin normal to the average cross-sectional topography at the pin station.

d) Place a washer flat with the ground surface over the soil pin. Tie 3 washers to the soil pin cap using rebar tie wire. Push the dowel through the washers into the ground until 32cm remain between the soil pin cap and the washer. Use a specially altered triangle desk rule to determine the 32 cm distance. Using a fine toothed hacksaw blade, saw a notch into the pin at the top of the washer that lies flat on the ground surface.

e) For Elk River silt-bank conditions, use dowels that are approximately four feet long to assure 3 feet of secure contact with the substrate.

f) Use washers with a hole diameter twice that of the dowel over the dowel.

g) Mark the three letter Alpha-Numeric X-Sec ID on an aluminum tree tag plus the date of installation and the soil pin sequential number and tie this to both the soil pin and rebar tie wire that suspends the three washers to the soil pin safety cap.

h) Using a “paint pen” paint the Alpha-Numeric X-sec ID and soil pin sequential number on the top of the soil pin cap.

i) Photograph the soil pin depicting the 32cm ruler in position between the washer and soil pin safety cap and the drywall square normal to the cross-sectional topography at the pin station point such that the Alpha-Numeric X-sec ID can be read.

j) Photograph the cross-section at intervals so that the soil pin sequence can be determined.

k) Note the Alpha-Numeric X-sec ID and soil pin sequential number, number of photos taken, soil pin station distance, etcetera. in the survey notes for the soil pin point.

l) Note the date, Alpha-Numeric X-sec ID, transect offset distance/upstream downstream information in the beginning of the survey notes, with a location sketch map- Photograph from upstream, downstream- and from each side. Describe the lens and focal length used for future use. Return the survey notes and photographs to Salmon-Forever.

m)  After a series of storms or individual storms, or at yearly intervals, document sedimentation or scour at each dowel pin using a millimeter scale. Note ground surface condition at each pin (organic debris or mineral sediment, recent deposit or older, compact soil or disturbed) and photograph with a digital time-dated camera describing the pin Alpha-Numeric X-sec ID. Push the millimeter  rule down into the deposit until the washer is reached. Record the depth of the deposit on all four sides. Paint the exposed portion of the dowel with a different color high quality enamel paint (the lower limit of the paint indicates the soil ground surface at this point in time).

WRITE ON YOUR FIELD FORM: date, time, location, the Cross-section ID #, paint color, soil surface condition, camera, pin# or scour chain#.

3.0 (j) V. 	Installation of Scour Chain Arrays:

a) Locate the scour chains at approximately 1 meter or 3 foot intervals on a cross-section transect. Some 1 meter or 3 foot  intervals will not be usable for a variety of reasons, if so, utilize the next acceptable location.

b) To place scour chains on the transect: First, pull a cloth tape tight between the cross-section end point to determine the 1 meter stations. Second, hang a light weight plumb bob from the cross-section tape at the soil pin station distance to locate the transect line and scour chain station point.

c) Given the soft silt/sand and gravel substrate of much of the bed of Elk River in the Study Area, 4 foot lengths of zinc coated #3 double loop steel chain can commonly be pushed down through the substrate using a 4 foot length of ¼ inch smooth steel rod. This is the rod that we use for the soil pins. To use this rod, grind one end to accept the chain loop such that the chain is affixed to the end of the rod. If you encounter an obstruction of buried wood, move over several inches and try again. If this proves impossible, revert to the use of the soil auger and sleeve.

d) Push the dowel with the chain affixed vertically down into the stream bed substrate until 4 to 6” remain on the surface. Using the compound leverage pliers, clip the chain until 4” remain and lay this portion in a downstream orientation. Attach an aluminum tree tag with the date and length of horizontal portion noted, as well as Alpha-Numeric X-sec and scour chain sequence ID number.

e) For typical Elk River silt/sand and gravel conditions, use scour chains that are approximately four feet long to assure a sufficient length of secure contact with the substrate.

f) Photograph the cross-section at intervals so that the scour chain sequence can be determined. Note the Alpha-Numeric X-sec ID and soil pin sequential number, number of photos taken, scour chain station distance, etcetera. in the survey notes for the scour chain point. Photograph from upstream, downstream- and from each side with a . Describe the lens and focal length used for future use. Return the survey notes and photographs to Salmon-Forever.

g)  After a series of storms or individual storms, or at yearly intervals, use a metal detector in addition to the station distance to help relocate the scour chain. The metal detector may help to identify how far the end of the chain is located from the vertical portion of the chain at the scour chain station point. This information may offer guidance as to how deep you will have to carefully dig to expose the downstream length of strewn chain.  Note the substrate ground surface condition at each pin (organic debris, silt, sand, gravel sediment, and photograph with a digital time-dated camera describing the scour chain Alpha-Numeric X-sec ID. Carefully dig down to expose the chain.  Document sedimentation or scour at each scour chain location using a centimeter scale.

3.0 (j). VI Computer Hardware and Software Used:

Software used will be primarily Microsoft Word and Excel programs. Software may also include specialized statistical and graphing programs.

3.0 (k)	Data Management & Records Management:

Data is entered on data sheets in the field. Sample information is recorded on standardized field and data sheets. See Appendix  for examples of all data sheets. The Project Director is responsible to double check and copy Field Data sheets and file them in the Salmon Forever Office. Salmon Forever will keep the original field forms. Data will be transferred to as necessary to Excel spreadsheets and Word documents and copies kept at the Sediment Lab. All data sheets will include initials of the person entering data and the date of data entry. Sheets will be numbered sequentially. Data and calculations are double-checked as data is entered into spreadsheets.

4.0	QA / QC:

I.	Quality Assurance

- Training of Staff

- Methods

- SOPs

- Equipment maintenance

- Data acquisition and data storage

- Data processing

- Unique ID numbers

- Lost or bent pins and scour chains

- Survey checks on bench marks and bank pins

II.	Quality Assessment

- Precision

- Accuracy

III. Internal QC

- QC Self verification: frequency of calibrations and recounting

- QC limits

- QC procedures for reporting data and results

IV. External QC:

Ed Schillinger, licensed surveyor, has certified relevant portions of the operating procedures in this document as compliant with appropriate surveying techniques . Mr. Schillinger will perform training of personnel and provide at least one check per year to determine that certified personnel are following these guidelines. Certified personnel will include Jesse Noell and Kristi Wrigley. Mr. Noell and Ms. Wrigley are responsible for assuring that volunteers and workstudy students who assist with surveying following these guidelines.

Quality Assessment/Assurance (QA) generally refers to a broad plan for maintaining quality in all aspects of this program. Quality assurance/assessment is the assessment of the overall precision and accuracy of the data, after we have run any analyses. QA activities include training of staff. Documentation and development of methods and standard operating procedures, equipment maintenance, and appropriate handling, processing, and tracking of all data and samples collected. These activities are designed to ensure that study objectives are met.

Internal QA: Proficiency Checklist once a season

Proficiency checklists (Appendix) , listing the sequence of sampling and data collection tasks, and notes on proper execution of methods, have been prepared for evaluating implementation of methods by individuals and teams. These checklists will be used by the QA Officer and Project Director during training and observation of staff to document workstudy students and volunteer proficiency. Any deficiencies noted shall be discussed and corrected at that time.  Any methods that the volunteers find confusing will be noted. Volunteers will be required to perform all sampling procedures correctly. If volunteers do not pass the proficiency criteria. they will receive additional training until they are proficient or they will not be utilized in this study. The Project Director and/or QA/QC Officer  is responsible for implementing these assessments and to document and file these checklists. Results shall be reported to the Project Director.

Safety procedures for sampling and taking measurements in stormy or hazardous conditions will be explained at every training session. High stream flows during storm events will be the main hazard the volunteers will encounter. Under no circumstances is anyone to risk injury for data. Back-up plans for volunteers to cover for each other will be developed.

Requirements for volunteers and work-study students include good physical health, the ability to consistently repeat sampling procedures and the time to spend sampling and analyzing data.

QA Field crew checks:

Watershed Coordinators will meet every 2 months to compare progress, to discuss and resolve problems that they may have encountered, and to address any issues brought to their attention by the external audits of internal QA checks. These meetings are important to prevent data quality problems and variations from sampling procedures and to identify problems with sampling procedures 'or logistics in the field.

5.0	References: (use full citation formats throughout – all the USGS manuals have authors.

EPA:

EPA QA/G-6 Guidance for Preparing Standard Operating Procedures (SOP'S)  who wrote it USGSTechniques of Water-Resources Investigations of the USGS:

Field Methods for Measurement of Fluvial Sediment Chapter C2 Book 3

Surface Water Techniques: who wrote it

Dunne, T and Leopold L. B. (1995) Water in Environmental Planning

Harrellson, C. C., 1994, Stream Channel Reference Sites: An Illustrated Guide to Field Technique: USFS, Rocky Mountain Forest and Range Experiment Station, General Technical Report RM-245.

Lisle, T. E.,  Stabilization of a gravel channel by large streamside obstruction and bedrock bends, Jacoby Creek, Northwestern California

Need a surveying book citation