SFM08 Event Loads

(Based on HY03-07 rating curve; not yet updated)

stn,storm,sdate,stime,edate,etime

SFM,801,080104,0320,080106,1430  1030292 kg  Excellent

SFM,802,080108,0500,080111,1200   223411 kg  Good

SFM,803,080130,2230,080202,0410  3385106 kg  Excellent

SFM,804,080202,0420,080206,0000  1332853 kg  Very good

SFM,805,080224,0100,080227,0000   245187 kg  Excellent

SFM,806,080312,1850,080317,1200   159024 kg  Very good

-----------------------------------------------------

SUM of events:                   6073312 kg  Excellent

-----------------------------------------------------

sfm08.flo <- read.flo("sfm",08)

sfm08.lab <- read.lab("sfm",08)

mismatches("sfm",08)

sfm08.sed <- merge.flo("sfm",08)

Storm 801: dump6: SFM,801,080104,0320,080106,1430

Excellent turbidity and sample coverage

four()

turbsscplot("sfm",08,080104,0320,080106,1430)

turbsscplot("sfm",08,080104,0320,080106,1430,type="power")

turbsscplot("sfm",08,080104,0320,080104,0920,type="power")

turbsscplot("sfm",08,080104,0930,080106,1430)

There's a very subtle hysteresis

turb1 <- turbsrc("sfm",08,080104,0320,080106,1430)

# Need power for turb2 to avoid negative predictions

turb2 <- turbsrc("sfm",08,080104,0320,080104,0920,type="power")

turb3 <- turbsrc("sfm",08,080104,0930,080106,1430)

> total(turb1)

start            end   type surr     yhat  n    r2      s    cv

turb1 01/04/08 03:20 01/06/08 14:30 linear turb 983789.8 14 0.992 83.854 5.856

Total load = 983790 kg,  CV = 5.86 %

ttsplot("sfm",08,turb2,turb3)

> total(turb2,turb3)

start            end   type surr     yhat  n    r2      s    cv

turb2 01/04/08 03:20 01/04/08 09:20  power turb 141866.3  4 0.999 48.367 1.545

turb3 01/04/08 09:30 01/06/08 14:30 linear turb 888425.8 10 0.995 65.922 5.575

Total load = 1030292 kg,  CV = 4.81 %   Excellent quality

Storm 802: dump 7: SFM,802,080108,0500,080111,1200

Turbidity good but had to reconstruct what looks like a fouled recession

Sample coverage fair, only 2 on main hydrograph

Could pick up an extra sample on either end

turbsscplot("sfm",08,dump=7,bot=3:8)

turbsscplot("sfm",08,dump=7,bot=3:8,type="pairs")

turbsscplot("sfm",08,dump=7,bot=3:8,type="loess")

turbsscplot("sfm",08,dump=7,bot=3:8,type="power")

turb1 <- turbsrc("sfm",08,080108,0500,080111,1200,dump=7,bot=3:8)

turb2 <- turbsrc("sfm",08,080108,0500,080111,1200,type="pairs")

turb3 <- turbsrc("sfm",08,080108,0500,080111,1200,dump=7,bot=3:8,type="loess")

turb4 <- turbsrc("sfm",08,080108,0500,080111,1200,dump=7,bot=3:8,type="power")

start            end   type surr     yhat n    r2      s     cv

turb1 01/08/08 05:00 01/11/08 12:00 linear turb 230822.0 6 0.869 40.546 11.457

turb2 01/08/08 05:00 01/11/08 12:00  pairs turb 244783.0 4    NA     NA     NA

turb3 01/08/08 05:00 01/11/08 12:00  loess turb 223411.3 6 0.937 34.990     NA

turb4 01/08/08 05:00 01/11/08 12:00  power turb 223741.6 6 0.892 37.326 11.226

# Loess fits the main hydrograph best.  Variation in estimates is 9%

ttsplot("sfm",08,turb3)

start            end  type surr     yhat n    r2     s cv

turb3 01/08/08 05:00 01/11/08 12:00 loess turb 223411.3 6 0.937 34.99 NA

Total load = 223411 kg  Good

Storm 803: dumps 8-9: SFM,803,080130,2230,080202,0410

Excellent turbidity and sample coverage

four()

turbsscplot("sfm",08,080130,2000,080202,0410)

turbsscplot("sfm",08,080130,2000,080202,0410,type="power")

turbsscplot("sfm",08,080130,2000,080202,0410,type="loess")

turbsscplot("sfm",08,080130,2000,080202,0410,type="logxy")

turb1 <- turbsrc("sfm",08,080130,2230,080202,0410,080130,2000,080202,0410,type="loess")

turb2 <- turbsrc("sfm",08,080130,2230,080202,0410,080130,2000,080202,0410,type="power")

turb3 <- turbsrc("sfm",08,080130,2230,080202,0410,type="pairs")

time1 <- lineartime("sfm",08,080130,2230,080202,0410)

total(turb1,turb2,turb3,time1)

start            end   type surr    yhat  n    r2       s    cv

turb1 01/30/08 22:30 02/02/08 04:10  loess turb 3385106 14 0.979 108.052    NA

turb2 01/30/08 22:30 02/02/08 04:10  power turb 3444347 14 0.974 119.512 2.818

turb3 01/30/08 22:30 02/02/08 04:10  pairs turb 3427824 13    NA      NA    NA

time1 01/30/08 22:30 02/02/08 04:10 linear time 3241657 13    NA      NA    NA

# The max and min estimates differ by 6%.  time1 clearly underestimates the two peaks. Loess is the best model for this data

ttsplot("sfm",08,turb1)

start            end  type surr    yhat  n    r2       s cv

turb1 01/30/08 22:30 02/02/08 04:10 loess turb 3385106 14 0.979 108.052 NA

Total load = 3385106 kg  Excellent

Storm 804: dump 9: SFM,804,080202,0420,080206,0000

Very good turbidity and sample coveage.

Removed a spike and reconstructed last few hours of fouling.

four()

turbsscplot("sfm",08,dump=9,bot=1:11)

turbsscplot("sfm",08,dump=9,bot=1:6)

turbsscplot("sfm",08,dump=9,bot=1:6,type="loess",span=0.9)

turbsscplot("sfm",08,dump=9,bot=7:11,type="pairs")

turb1 <- turbsrc("sfm",08,080202,0420,080206,0000,dump=9,bot=1:11)

ttsplot("sfm",08,turb1)

start     end   type surr    yhat  n   r2      s    cv

turb1 02/02/08 04:20 2/06/08 linear turb 1255032 11 0.93 74.582 5.271

Total load = 1255032 kg,  CV = 5.27 %

turb2 <- turbsrc("sfm",08,080202,0420,080203,0240,dump=9,bot=1:6,type="loess",span=0.9)

turb3 <- turbsrc("sfm",08,080203,0250,080206,0000,type="pairs")

ttsplot("sfm",08,turb2,turb3)

total(turb2,turb3)

start            end  type surr     yhat n r2     s cv

turb2 02/02/08 04:20 02/03/08 02:40 loess turb 485866.9 6  1 9.727 NA

turb3 02/03/08 02:50 02/06/08 00:00 pairs turb 846986.4 6 NA    NA NA

Total load = 1332853 kg  Very good

Storm 805: dump 11 : SFM,805,080224,0100,080227,0000

Excellent turbidity and sample coverage.

The questionable secion near the end is corroborated by sample 18.

# Include bottle 3 from before defined start of storm

four()

turbsscplot("sfm",08,dump=11,bot=3:19)

turbsscplot("sfm",08,dump=11,bot=3:9)

turbsscplot("sfm",08,dump=11,bot=3:9,type="power")

turbsscplot("sfm",08,dump=11,bot=8:19)

turb1 <- turbsrc("sfm",08,080224,0100,080227,0000,dump=11,bot=3:19)

total(turb1)

start     end   type surr     yhat  n    r2      s    cv

turb1 02/24/08 01:00 2/27/08 linear turb 236198.7 17 0.979 32.151 4.014

Total load = 236199 kg,  CV = 4.01 %

turb2 <- turbsrc("sfm",08,080224,0100,080224,1800,dump=11,bot=3:9,type="power")

turb3 <- turbsrc("sfm",08,080224,1810,080227,0000,dump=11,bot=8:19)

ttsplot("sfm",08,turb2,turb3)

total(turb2,turb3)

start            end   type surr     yhat  n    r2      s    cv

turb2 02/24/08 01:00 02/24/08 18:00  power turb  38803.5  7 0.999  9.646 1.310

turb3 02/24/08 18:10 02/27/08 00:00 linear turb 206383.7 12 0.995 17.374 2.583

Total load = 245187 kg,  CV = 2.18 %  Excellent

Storm 806: dump 12: SFM,806,080312,1850,080317,1200

The first part of the turbidity is missing

Vey good sample coverage

four()

turbsscplot("sfm",08,dump=12,bot=5:14)

turbsscplot("sfm",08,dump=12,bot=5:9)

turbsscplot("sfm",08,dump=12,bot=5:10)

turbsscplot("sfm",08,dump=12,bot=11:14)

# Need to handle piece from 080312:1850 to 080313:0840 with no turbidity

four()

qsscplot("sfm",08,dump=12)

qsscplot("sfm",08,dump=12,bot=2:5)

qsscplot("sfm",08,dump=12,bot=2:6)

qsscplot("sfm",08,dump=12,bot=2:8,type="loess")

# Need to break this into two sections after the initial problem

flow1 <- flowsrc("sfm",08,080312,1850,080313,0840,dump=12,bot=2:8,type="loess")

turb1 <- turbsrc("sfm",08,080313,0850,080315,0650)

turb2 <- turbsrc("sfm",08,080315,0700,080317,1200,dump=12,bot=11:14)

ttsplot("sfm",08,flow1,turb1,turb2)

# Continuity is not good especially between turb1 and turb2

flow2 <- flowsrc("sfm",08,080312,1850,080313,0840,dump=12,bot=2:5,type="logxy")

turb3 <- turbsrc("sfm",08,080313,0850,080314,1400,dump=12,bot=5:9)

turb4 <- turbsrc("sfm",08,080314,1410,080317,1200,type="pairs")

ttsplot("sfm",08,flow2,turb3,turb4)

total(flow2,turb3,turb4)

start            end       type surr      yhat n    r2      s    cv

flow2 03/12/08 18:50 03/13/08 08:40 logxy:mvue flow   3024.98 4 0.993  0.150 9.070

turb3 03/13/08 08:50 03/14/08 14:00     linear turb  38654.53 5 0.987 14.196 4.245

turb4 03/14/08 14:10 03/17/08 12:00      pairs turb 117344.77 5    NA     NA    NA

Total load = 159024 kg  Very good