FTR 2007 Annual load estimation

ftr07.flo <- read.flo("ftr",07)

ftr07.lab <- read.lab("ftr",07)

ftr07.sed <- merge.flo("ftr",07)

nonstorms <- drop.storms("ftr",07)

dim(nonstorms)

[1] 28543     4

head(nonstorms, 5)

chr      q turb turbcode

1 (10/29/06 12:10:00) 0.3794    0        0

2 (10/29/06 12:20:00) 0.3794    0        0

3 (10/29/06 12:30:00) 0.3794    0        0

4 (10/29/06 12:40:00) 0.3795    0        0

5 (10/29/06 12:50:00) 0.3795    0        0

table(nonstorms$turbcode)

0     2     5

23964    46  4533

There are 46 values of bad turbidity out of 28543 intervals to be estimated.

trb07.flo <- nonstorms[nonstorms$turbcode %in% c(0,5), ]

flo07.flo <- nonstorms[nonstorms$turbcode ==2, ]

dim(trb07.flo)

[1] 28497	4

dim(flo07.flo)

[1] 6   4

28497 + 46

[1] 28543

# Develop model for good turb data

range(trb07.flo$turb)

[1]  0 144

range(ftr07.sed$turb)

[1]   8 670

# Don't need full range up to 670

table(ftr07.sed$labcode2)

0   1   4   9

164   9   4   1

# Just one bad lab code (9) to worry about

# (4=spilled < 15% of sediment, 9=spilled > 15% of sediment)

ftr07.sed[ftr07.sed$labcode2 %in% c(4,9), ]

chr dump bottle    ssc turb turbcode       q labcode1 labcode2

20  (12/09/06 01:50:00)    5      1  11.88   24        0   1.731        0        4

105 (02/20/07 20:20:00)   10      2  46.53   86        0  27.920        0        4

114 (02/21/07 11:30:00)   10     11 903.63  554        0 751.700        0        4

117 (02/21/07 18:30:00)   10     14 616.44  408        0 885.100        0        4

119 (02/21/07 22:50:00)   10     16 443.08  299        0 806.700        0        9

two()

turbsscplot("ftr",07,dump=5)

turbsscplot("ftr",07,dump=10)

# None of the bots with labcode 4 or 9 are outliers, keep them

trb07.sed <- ftr07.sed

turbsscplot("trb",07,type="loess")

# Can see this goes negative at low turbidity

Eliminate turbidity above 200.  Induces curvature and exceeds range needed for prediction. Also eliminate the sample that had SSC = -4.24 (dump 8, bot 7)

trb07.sed <- ftr07.sed[ftr07.sed$ssc > 0 & ftr07.sed$turb < 200, ]

turbsscplot("trb",07,type="loess")

# Still goes negative a low turbidities

turbsscplot("trb",07,type="power")

turbsscplot("trb",07,type="logxy",txt="dump")

turb1 <- turbsrc("trb",07,060801,0000,070731,2400,type="power",var=F)

turb2 <- turbsrc("trb",07,060801,0000,070731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",07,060801,0000,070731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",07,060801,0000,070731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat   n    r2      s cv

turb1 10/29/06 12:10 06/10/07 14:30      power turb 176055.2 141 0.907 15.830 NA

turb2 10/29/06 12:10 06/10/07 14:30 logxy:mvue turb 195967.4 141 0.731  0.570 NA

turb3 10/29/06 12:10 06/10/07 14:30      loess turb 163279.9 141 0.901 16.345 NA

turb4 10/29/06 12:10 06/10/07 14:30     linear turb 158772.6 141 0.887 17.409 NA

> sum(turb3$predssc < 0)

[1] 18348

> sum(turb4$predssc < 0)

[1] 20606

# Can't use loess or linear: too many negative predictions

# The power model or logxy model differ by 11%, but the power model fits better

# logxy model is overly influenced by one bottle with turb < 10 (dump 13, bottle 1)

trb07.sed <- trb07.sed[trb07.sed$turb > 10, ]

turb5 <- turbsrc("trb",07,060801,0000,070731,2400,type="logxy",var=F)

total(turb5)

start            end       type surr     yhat   n    r2     s cv

turb5 10/29/06 12:10 06/10/07 14:30 logxy:mvue turb 193007.8 140 0.733 0.565 NA

# Without the high-leverage datum, estimate is 9.6% higher than power model

# Power model is still the preferred model

total(turb1)

start            end  type surr     yhat   n    r2      s cv

turb1 10/29/06 12:10 06/10/07 14:30 power turb 176691.5 140 0.907 15.826 NA

Total load = 176692 kg

# A fairly good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo07.flo$q)

[1] 0.9786 2.5910

> range(ftr07.sed$q)

[1]   1.731 888.300

# There are only 3 samples in the range of discharge needed

ftr07.sed[ftr07.sed$q < 3, ]

chr dump bottle   ssc turb turbcode     q labcode1 labcode2

20 (12/09/06 01:50:00)    5      1 11.88   24        0 1.731        0        4

75 (02/07/07 15:20:00)    8      1 95.63   44        0 2.495        1        0

76 (02/07/07 16:10:00)    8      2 64.50   95        0 2.883        1        0

# But these 3 may not be very representative of discharges that low

qsscplot("ftr",07)

# There doesn't appear to be a trend below 1 cms

flo07.sed <- ftr07.sed[ftr07.sed$q < 35, ]

qsscplot("flo",07)

Indeed no trend, just use the mean SSC for this set of samples

mean(flo07.sed$ssc)

[1] 26.36907

# Standard error of this mean is 3.3

sqrt(var(flo07.sed$ssc)/length(flo07.sed$ssc))

[1] 3.288282

# Calc the sed load

0.06 * 10 * 26.369 * sum(flo07.flo$q)/35.319

36.86759

# The load is practically negligible since this is for only 46 intervals

==================================================

# Total non-storm load

176692 + 37 = 176729 kg = 177 metric tons

# Quality is good

# Total storm load = 2437411 kg = 2437 metric tons

# Total annual load

2437 + 177 = 2614 metric tons total annual load

# Storm load is 2437/2614 = 93.2% of annual load