FTR04 Event Loads

FTR,401,031212,1110,031213,2020  360359 kg  Excellent

FTR,402,031213,2030,031217,0000  431081 kg  Excellent

FTR,403,031228,2010,031231,1300  533674 kg  Excellent

FTR,404,031231,1310,040105,0000  589062 kg  Excellent

FTR,405,040216,1010,040217,2240 1005734 kg  Excellent

FTR,406,040217,2250,040220,1200  330955 kg  Poor

FTR,407,040225,0120,040229,0000  173729 kg  Good

-----------------------------------------------------

SUM of events                   3424594 kg  Excellent

-----------------------------------------------------

ftr04.flo <- read.flo("ftr",04)

ftr04.lab <- read.lab("ftr",04)

mismatches("ftr",04)

# First sample of year 31Jul2003:1040 technically goes with HY03

# has no match in ftr04.flo

# Many unmatched samples in flo file, as usual.

ftr04.sed <- merge.flo("ftr",04)

Storm 401: dump 12: FTR,401,031212,1110,031213,2020

Excellent turbidity and sample coverage

four()

turbsscplot("ftr",04,031212,1110,031213,2020)

turbsscplot("ftr",04,031212,1110,031213,2020,type="power")

turbsscplot("ftr",04,031212,1110,031213,0020,type="power")

turbsscplot("ftr",04,031213,0030,031213,2200)

turb0 <- turbsrc("ftr",04,031212,1110,031213,2020,type="power")

ttsplot("ftr",04,turb0)

total(turb0)

start            end  type surr     yhat  n    r2     s    cv

turb0 12/12/03 11:10 12/13/03 20:20 power turb 363027.2 12 0.982 47.23 2.672

Total load = 363027 kg,  CV = 2.67 %

turb1 <- turbsrc("ftr",04,031212,1110,031212,2340,type="power")

turb2 <- turbsrc("ftr",04,031213,0110,031213,2020,dump=12,bot=5:12)

> endpoints(turb1)

start        end

11.34136 1202.99454

> endpoints(turb2)

start       end

983.70695  62.17425

time1 <- lineartime("ftr",04,031212,2350,031213,0100,ssc1=1200,ssc2=990)

ttsplot("ftr",04,turb1,time1,turb2)

start            end   type surr      yhat n    r2      s    cv

turb1 12/12/03 11:10 12/12/03 23:40  power turb  76709.94 5 0.995 34.446 2.781

time1 12/12/03 23:50 12/13/03 01:00 linear time  74715.71 0    NA     NA    NA

turb2 12/13/03 01:10 12/13/03 20:20 linear turb 208933.81 8 0.999  9.364 0.897

Total load = 360359 kg  Quality excellent

# Less than 1% difference between the two estimates

Storm 402: dump 12-13: FTR,402,031213,2030,031217,0000

Excellent turbidity and sample coverage

four()

turbsscplot("ftr",04,031213,2030,031217,0000)

turbsscplot("ftr",04,031213,2030,031217,0000,type="logxy")

turbsscplot("ftr",04,031213,2030,031217,0000,type="power")

turbsscplot("ftr",04,031213,2030,031217,0000,type="loess",degree=2)

turb1 <- turbsrc("ftr",04,031213,2030,031217,0000,type="loess")

turb2 <- turbsrc("ftr",04,031213,2030,031217,0000,type="power")

turb3 <- turbsrc("ftr",04,031213,2030,031217,0000,type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",04,031213,2030,031217,0000,type="pairs")

endpoints(turb1)

start       end

82.86599 -17.15788

# Linear and loess fits both go negative; power fit looks best

total(turb2,turb3,turb4)

start     end       type surr     yhat  n    r2      s    cv

turb2 12/13/03 20:30 2/17/03      power turb 431081.0 18 0.994 43.574 2.024

turb3 12/13/03 20:30 2/17/03 logxy:qmle turb 454493.3 18 0.985  0.219 6.786

turb4 12/13/03 20:30 2/17/03      pairs turb 428497.7 18    NA     NA    NA

ttsplot("ftr",04,turb2)

total(turb2)

start     end  type surr   yhat  n    r2      s    cv

turb2 12/13/03 20:30 2/17/03 power turb 431081 18 0.994 43.574 2.024

Total load = 431081 kg,  CV = 2.02 %  Excellent

Storm 403: dump 14: FTR,403,031228,2010,031231,1300

Two peaks, second is slightly smaller

Excellent turbidity and sample coverage

four()

turbsscplot("ftr",04,031228,2010,031231,1300)

turbsscplot("ftr",04,031228,2010,031231,1300,type="power")

turbsscplot("ftr",04,031228,2010,031231,1300,type="logxy")

turbsscplot("ftr",04,031228,2010,031231,1300,type="loess")

Clearly goes negative, will have to go with power or logxy (or pairs)

logxy looks like the better fit

turb1 <- turbsrc("ftr",04,031228,2010,031231,1300,type="power")

turb2 <- turbsrc("ftr",04,031228,2010,031231,1300,type="logxy",bias="qmle")

turb3 <- turbsrc("ftr",04,031228,2010,031231,1300,type="pairs")

total(turb1,turb2,turb3)

start            end       type surr     yhat  n    r2      s    cv

turb1 12/28/03 20:10 12/31/03 13:00      power turb 547652.1 15 0.982 29.718 1.888

turb2 12/28/03 20:10 12/31/03 13:00 logxy:qmle turb 551162.1 15 0.992  0.085 2.653

turb3 12/28/03 20:10 12/31/03 13:00      pairs turb 533673.9 15    NA     NA    NA

# Range of estimates varies by 2.6%

ttsplot("ftr",04,turb1)

# turb2 overestimates the long recession starting with bottle 13

ttsplot("ftr",04,turb3)

start            end  type surr     yhat  n r2  s cv

turb3 12/28/03 20:10 12/31/03 13:00 pairs turb 533673.9 15 NA NA NA

Total load = 533674 kg  Excellent

Storm 404: dump 15-16: FTR,404,031231,1310,040105,0000

Excellent turbidity and sample coverage

four()

turbsscplot("ftr",04,031231,1310,040105,0000)

turbsscplot("ftr",04,031231,1310,040105,0000,type="power")

turbsscplot("ftr",04,031231,1310,040105,0000,type="logxy")

turbsscplot("ftr",04,031231,1310,040105,0000,type="pairs")

Once again, linear plot goes negative

Power and logxy plots miss samples 13 and 1

turb1 <- turbsrc("ftr",04,031231,1310,040105,0000,type="pairs")

ttsplot("ftr",04,turb1)

start     end  type surr     yhat  n r2  s cv

turb1 12/31/03 13:10 1/05/04 pairs turb 581187.6 14 NA NA NA

Total load = 581188 kg

# Does pretty well but messes up the details on the peak and end of recession

# Split into two pieces

four()

turbsscplot("ftr",04,031231,1310,040101,0800)

turbsscplot("ftr",04,031231,1310,040101,0800,type="power")

turbsscplot("ftr",04,040101,0900,040105,0000)

turbsscplot("ftr",04,040101,0900,040105,0000,type="power")

turb2 <- turbsrc("ftr",04,031231,1310,040101,0800,type="pairs")

turb3 <- turbsrc("ftr",04,040101,0900,040105,0000)

> tail(turb2$predssc,5)

[1] 1068.124 1205.971 1280.000 1358.403 1412.544

> head(turb3$predssc,5)

[1] 1163.0881 1111.1430 1048.8088  986.4746  926.2183

time1 <- lineartime("ftr",04,040101,0810,040101,0850,ssc1=1430,ssc2=1200)

ttsplot("ftr",04,turb2,time1,turb3)

total(turb2,time1,turb3)

start            end   type surr      yhat  n    r2      s    cv

turb2 12/31/03 13:10 01/01/04 08:00  pairs turb 137233.63  4    NA     NA    NA

time1 01/01/04 08:10 01/01/04 08:50 linear time  85533.71  0    NA     NA    NA

turb3 01/01/04 09:00 01/05/04 00:00 linear turb 366294.35 10 0.998 15.842 2.839

Total load = 589062 kg  Excellent

Storm 405: dump 19: FTR,405,040216,1010,040217,2240

3 peaks in this storm;

Excellent turbidity and sample coverage

four()

turbsscplot("ftr",04,040216,1010,040217,2240)

turbsscplot("ftr",04,040216,1010,040217,2240,type="power")

turbsscplot("ftr",04,040216,1010,040217,0150,type="power")

turbsscplot("ftr",04,040217,0300,040217,2240)

turb0 <- turbsrc("ftr",04,040216,1010,040217,2240,type="power")

ttsplot("ftr",04,turb0)

total(turb0)

start            end  type surr    yhat  n    r2      s    cv

turb0 02/16/04 10:10 02/17/04 22:40 power turb 1004036 16 0.946 83.564 3.445

Total load = 1004036 kg,  CV = 3.45 %

turb1 <- turbsrc("ftr",04,040216,1010,040217,0150,type="power")

turb2 <- turbsrc("ftr",04,040217,0300,040217,2240)

> tail(turb1$predssc,5)

[1] 1331.112 1442.650 1539.649 1611.681 1703.456

> head(turb2$predssc,5)

[1] 1208.925 1151.654 1115.001 1073.767 1009.624

time1 <- lineartime("ftr",04,040217,0200,040217,0250,ssc1=1750,ssc2=1250)

ttsplot("ftr",04,turb1,turb2,time1)

total(turb1,turb2,time1)

start            end   type surr     yhat n    r2      s    cv

turb1 02/16/04 10:10 02/17/04 01:50  power turb 249926.3 8 0.984 59.188 3.492

time1 02/17/04 02:00 02/17/04 02:50 linear time 120252.3 0    NA     NA    NA

turb2 02/17/04 03:00 02/17/04 22:40 linear turb 635555.6 8 0.985 32.239 2.567

Total load = 1005734 kg  Excellent

# The two estimates differ by only 0.2%

Storm 406: dump 20: FTR,406,040217,2250,040220,1200

Two peaks with good turbidity and NO samples.  Pump sample malfunction.

Two huge turbidity spikes late on recession maxed out probe

Both occurred at nearly the same time: from 0940 to 1330 hours.  That leads

me to believe it was an artificial disturbance, e.g. from construction upstream.

Six samples with significant sediment were collected on the second spike.

Clark noted what happened but had no explanation.

Estimate for this storm will be poor.  I have assumed spikes are false.

turbsscplot("ftr",04,dump=c(19,21:22),type="power")

turb1 <- turbsrc("ftr",04,040217,2250,040220,1200,dump=c(19,21:22),type="power")

total(turb1)

start            end  type surr     yhat  n    r2      s    cv

turb1 02/17/04 22:50 02/20/04 12:00 power turb 330955.5 41 0.978 53.276 5.407

Total load = 330955 kg,  CV = 5.41 %  Poor quality

Storm 407: dump 22: FTR,407,040225,0120,040229,0000

Two turbidity peaks

Strange dip in turbidity around sample 12, centered on 12 noon

Mostly very good turbidity and good sample coverage

four()

turbsscplot("ftr",04,dump=22,bot=2:13)

turbsscplot("ftr",04,dump=22,bot=2:13,type="power")

turbsscplot("ftr",04,dump=22,bot=2:13,type="logxy")

turbsscplot("ftr",04,dump=22,bot=2:13,type="sqrt")

turb1 <- turbsrc("ftr",04,040225,0120,040229,0000,dump=22,bot=2:13)

turb2 <- turbsrc("ftr",04,040225,0120,040229,0000,dump=22,bot=2:13,type="logxy",bias="qmle")

turb3 <- turbsrc("ftr",04,040225,0120,040229,0000,dump=22,bot=2:13,type="sqrt")

turb4 <- turbsrc("ftr",04,040225,0120,040229,0000,dump=22,bot=2:13,type="loess")

total(turb1,turb2,turb3,turb4)

start     end       type surr     yhat  n    r2      s    cv

turb1 02/25/04 01:20 2/29/04     linear turb 173728.6 12 0.910 25.979 7.627

turb2 02/25/04 01:20 2/29/04 logxy:qmle turb 174480.0 12 0.965  0.241 9.275

turb3 02/25/04 01:20 2/29/04  sqrt:duan turb 172221.0 12 0.954  1.041    NA

turb4 02/25/04 01:20 2/29/04      loess turb 181152.1 12 0.933 23.719    NA

# The estimates are remarkably similar consider the variability

ttsplot("ftr",04,turb1)

start     end   type surr     yhat  n   r2      s    cv

turb1 02/25/04 01:20 2/29/04 linear turb 173728.6 12 0.91 25.979 7.627

Total load = 173729 kg,  CV = 7.63 %  Quality good