HY08 RATING EQUATION UPDATES:

Added 4 measurements for FTR, 3 for HHB, 2 for SFM, and 3 for KRW

(To get water year, add 2000 to the symbols in the plot)

# Updated FTR equations are

Stage < 1.9609:  Q = 3.2437*s^4.4734

Stage > 1.9609:  Q = -238.66 + 132.23*s + 11.79*s^2

# HY03-HY05 have been calculated with old equation

# Apply new equations for HY06 and subsequent years

Added 0.045 ft to all stage measurements through CY2005.

The data included in the plots and calculations are

FTR:

hy         id       date   stg       q qual

1  2003  11/9/2002  11/9/2002 0.735    5.35 good

2  2003 12/14/2002 12/14/2002 2.455  166.72 poor

3  2003 12/15/2002 12/15/2002 2.085  110.89 good

4  2003 12-28-02 a 12/28/2002 5.285  814.52 fair

5  2003 12-28-02 b 12/28/2002 6.435 1089.22 fair

6  2003   1/5/2003   1/5/2003 1.645   36.81 good

7  2003  4/13/2003  4/13/2003 2.255  119.17 good

8  2004 10/12/2003 10/12/2003 1.015    0.29 fair

9  2004   1/4/2004   1/4/2004 2.045   69.67 good

10 2004  2/29/2004  2/29/2004 1.935   63.89 good

11 2005  9/19/2004  9/19/2004 1.015    0.33 good

12 2005  11/6/2004  11/6/2004 1.155    3.81 fair

13 2005   1/3/2005   1/3/2005 2.005   71.93 fair

14 2005   1/4/2005   1/4/2005 1.845   50.59 fair

15 2005   1-5-05 a   1/5/2005 1.685   29.36 good

16 2005   1-5-05 b   1/5/2005 1.685   34.26 fair

17 2005   1/6/2005   1/6/2005 1.615   24.06 fair

18 2005  3/21/2005  3/21/2005 2.295  125.47 fair

19 2005  3/22/2005  3/22/2005 2.115   93.99 good

21 2006  9/18/2005  9/18/2005 0.715    0.58 fair

22 2006 12/11/2005 12/11/2005 1.205   15.31 good

23 2006  3/12/2006  3/12/2006 1.830   50.26 good

24 2007  9/10/2006  9/10/2006 0.540    0.16 fair

25 2007 11/12/2006 11/12/2006 0.790    2.35 fair

26 2007 11/18/2006 11/18/2006 1.140    6.65 good

27 2007 12/29/2006 12/29/2006 1.820   40.67 good

28 2007  2/25/2007  2/25/2007 3.490  389.37 good

29 2008  1/10/2008  1/10/2008 2.170  108.66

30 2008  1/31/2008  1/31/2008 3.680  375.37

31 2008   2/1/2008   2/1/2008 2.710  179.13

32 2008   2/5/2008   2/5/2008 1.920   61.25

Excluded from FTR

20 2005  3/29/2005  3/29/2005 3.765  194.04 fair

# Updated HHB equations for HY05 to HY08 are

Stage < 2.89403:  Q =  12.606 - 36.016s + 23.568s^2

Stage > 2.89403:  Q = -80.000 + 31.458s + 11.310s^2

# HY05 loads were calculated with old HY03-07 rating equation

# It’s not obvious which rating curve is more applicable for HY05

# But this curve looks more realistic.  Old curve was linear for

# most of the range.  Recalculate HY05 loads with new rating eqn.

Data prior to HY04 are omitted. The data included in the plots and calculations are

HHB:

17 2004 10/25/2003 10/25/2003  0.97    0.72

18 2004  11/8/2003  11/8/2003  1.11    2.41

19 2004  1/11/2004  1/11/2004  1.90   40.65

20 2005  9/19/2004  9/19/2004  0.83    0.54

21 2005 12-15-04 A 12/15/2004  1.51   14.57

22 2005 12-15-04 B 12/15/2004  1.51   12.68

23 2005   1/4/2005   1/4/2005  2.65   79.55

24 2005   1/5/2005   1/5/2005  2.23   54.16

25 2005   1/6/2005   1/6/2005  2.15   40.93

26 2005  3-20-05 A  3/20/2005  2.73   80.92

27 2005  3-20-05 B  3/20/2005  7.37  705.81

28 2005  3/21/2005  3/21/2005  3.99  283.14

30 2005 3-29-05 ov  3/29/2005  9.57 1265.65

32 2006 10/29/2005 10/29/2005  1.32    2.14

33 2007  9/10/2006  9/10/2006  1.21    0.51

34 2007 11/18/2006 11/18/2006  1.60    8.78

35 2007 12/29/2006 12/29/2006  2.66   70.24

36 2007  2/22/2007  2/22/2007  6.16  656.78

37 2008  1/31/2008  1/31/2008 11.33 1796.00

38 2008   1/4/2008   1/4/2008  8.64  883.07

39 2008  2/26/2008  2/26/2008  2.78  114.22

Excluded from HHB:

40 2005   3-29-05 a  3/29/2005  9.57 1159.57 no overbank

41 2005   3-29-05 b  3/29/2005  6.65  350.75

Measurement of 1/31/2008 includes an estimated 353 cfs overbank flow

Two new points were added (shown in red) from 4/1/07, and 2/25/08.

The old line (black) and the new line with all the data (dashed red)

are indistinguishable.  Hence HY2008 loads were not recalculated.

They are still based on the HY03-07 equation (black line).

HY03-07: q = 2.5828*s^2+19.266*s-32  was used for HY06-08

HY03-08: q = 2.6237*s^2+18.692*s-32  start using for HY09

The data included in the plots are

date   hy   stg       q

1  16-Nov-02 2003  1.21    0.92

2  15-Dec-02 2003  4.89   93.51

3  16-Dec-02 2003 12.41  544.72

4  31-Dec-02 2003 16.85 1064.72

5   1-Jan-03 2003  6.73  233.23

6  19-Jan-03 2003  2.80   34.71

7   3-Jan-04 2004  5.33  145.92

8  12-Dec-04 2005  2.88   32.21

9  20-Mar-05 2005  5.61  154.76

10 20-Mar-05 2005  8.14  396.93

11 21-Mar-05 2005  6.45  239.79

12 29-Mar-05 2005 10.07  398.81

13  9-Apr-05 2005  9.18  425.16

14 20-Nov-05 2006  1.90    6.38

15  9-Dec-05 2006  2.65   19.82

19  2-Feb-06 2006 10.87  459.99

20  4-Feb-06 2006  6.27  201.09

21 11-Feb-06 2006  2.86   25.66

22  5-Mar-06 2006 15.71  898.23

23 27-Aug-06 2007  1.50    0.31

24 19-Nov-06 2007  1.89    5.29

25 13-Feb-07 2007  4.60   97.60

26 25-Feb-07 2007 10.63  398.65

27 10-Apr-07 2007  2.78   10.95 new point (red)

28 25-Feb-08 2008  6.30  181.24 new point (red)

Excluded data

16 20-Dec-05 2006  8.48  163.55

17 24-Dec-05 2006  4.68   56.68

18 28-Dec-05 2006 16.87  606.48

# Updated KRW equations based on HY00 to HY08 are

Stage < 1.3255:  Q = -1.304s + 7.883s^2

Stage > 1.3255:  Q = -27.321 + 19.577s + 2.394s^2

Although it’s possible that the rating curve shifted, these data don’t make a strong case. HY07 and HY08 stages shifted in opposite directions. Old rating (based on just 8 HY03-05 points) is a poor fit below about 4 ft. I think it’s best to assume no shift in the rating occurred, but recalculate all loads using the new equation.

The data included in the plots and calculations are:

date   hy   stg      q qual

1  10/24/1999 2000  0.43   0.28 fair

2   12/5/1999 2000  1.86  22.86 fair

3   12/2/2000 2001  1.16   7.67 fair

4   1/20/2001 2001  0.98   5.68 poor

5   3/11/2001 2001  1.47  15.84 good

6    4/2/2001 2001  1.36  15.23 fair

7   1/19/2002 2002  1.49  17.35 fair

8   1/22/2002 2002  4.43 141.00 fair

9   2/19/2002 2002  8.95 391.61 poor

10  2/20/2002 2002 12.81 687.56 poor

11  2/20/2002 2002 12.58 644.07 poor

12  2/21/2002 2002  5.13 168.01 fair

13 11/16/2002 2003  0.65   1.23 fair

14 12/15/2002 2003  6.16 205.00 poor

15 12/16/2002 2003 14.37 811.70 poor

16   1/1/2003 2003  6.49 229.70 fair

17  3/16/2003 2003  4.62 127.70 fair

18  3/21/2005 2005  6.69 264.24 poor

19  3/28/2005 2005  8.94 389.91 fair

20  3/29/2005 2005 12.62 608.28 fair

22  4/10/2007 2007  1.88   9.21

23  4/18/2007 2007  3.63  72.54

24   1/7/2008 2008  6.82 296.27

Excluded:

21   4/8/2005 2005 15.08 417.54 fair (but major outlier)

Note: First 12 measurements are before KRW was installed.  The stage at KRW for these pairs is based on Randy Klein’s correlation between manual stages at KRW and the NF bridge.  For the remaining pairs, a lag time was computed between equivalent stages at the two stations based on the mean velocity from the discharge measurement.  The corrected KRW stages for the calculated times were read from TTS adjuster charts.