Data corrections SFM08 - Jack Lewis

Dump 2 data was lost.

Observer record corrections – Jack Lewis

Changed

2007,11,10,0930,3,1.23  (cannot be right)

to

2007,11,10,0930,3,2.23  (guessing what was intended)

Rating equation

Used the equation based on HY03 to HY07 data

Stage corrections - Jack Lewis

Reconstructed dump 2 from KRW using time-linear ratio.  This got it

pretty close to the OR on Oct 19.

Reconstructed weird dip in hydrograph on Nov 21 and Nov 22, from KRW.

Field notes:

01Feb2008:1810 stage peaked earlier at 18.4 according to Stephanie bennett

03Feb2008:1058 stage peaked earlier at 12.2 with question mark

24Feb2008:2126 stage roughly 8.1 flashlight too dim to read, estage=8.254

13Mar2008:0900 time of staff plate reading uncertain

A series of time-variable shifts should take care of things.

1. One big shift from 0 on Oct 1:0000 to -0.047 on Mar 25:1700.

2. A variable shift from Mar 25 to Apr 22, matching both.

3. A couple of y-proportional shifts to match the Oct 19th OR

(during the reconstruction of dump 2)

4. A variable shift from 01Feb2008:1740 to 03Feb2008:1100 takes care of the

early Feb outlier and takes care of the weird stage jump at 1100.

That leaves this

plot.or("sfm",08,cor=T)

The remaining errors over 0.02 appear random or isolated and could

plausibly be time or staff plate reading errors, for example the

reading on 04Jan2008:0952 is correct for 0955. So I'll stop here.

Turbidity corrections - Jack Lewis

A great deal of bad turbidity data was coded as missing.

03Jan to 07Mar was generally fixable by interpolating the recessions.

The larger storm peaks generally had good data.

Need to get a DTS-12 at this site!

Samples - quite a few mismatches

No flow data to match the 21 of 24 samples that were processed from dump 2

There is also a fairly long list of bottles for which there is no lab data,

but they might have been intentionally discarded.