FTR DISCHARGE ERROR ANALYSIS HY03-05

# The following function was applied for 03-05

# It’s based on data from 03-07

qfunftr05 <- function(s) ifelse(s < 1.8365, 5.013*s^3.770, 6.911*s^2 + 172.61*s -290.70)

avg.err <- function(e) (exp(e) - exp(-e))/2

qftr <- read.csv("ftr/Q and rating curves/Qmeas0307.csv")

attach(qftr[-20,])

plot(stg,q)

x0 <- seq(0,6.6,0.2)

y0 <- qfunftr05(x0)

lines(x0,y0,col=1)

# Develop model for log(q)

fit1 <- lm(log(q) ~ log(stg))

fit2 <- lm(log(q) ~ log(stg) + I((log(stg))^2))

fit3 <- lm(log(q) ~ log(stg) + I((log(stg))^2) + I((log(stg))^3))

fit4 <- lm(log(q) ~ log(stg) + I((log(stg))^2) + I((log(stg))^3) + I((log(stg))^4) + I((log(stg))^4))

y1 <- predict(fit1,newdata=data.frame(stg=x0))

y2 <- predict(fit2,newdata=data.frame(stg=x0))

y3 <- predict(fit3,newdata=data.frame(stg=x0))

y4 <- predict(fit4,newdata=data.frame(stg=x0))

lines(x0,exp(y1),col=2)

lines(x0,exp(y2),col=3)

lines(x0,exp(y3),col=4)

lines(x0,exp(y4),col=5)

legend(locator(1),c(”Rating equation”,”log(q) ~ poly(1)”,”log(q) ~ poly(2)”,”log(q)  ~ poly(3)”,”log(q)  ~ poly(4)”),lty=c(1,1,1,1,1),col=1:5)

Unable to find a polynomial in log(stg) close to the original rating.  How does fit3 compare in terms of RMSE

q1 <- qfunftr05(stg)

q2 <- exp(predict(fit3,newdata=data.frame(stg)))

sqrt(mean((q-q1)^2))

[1] 6.511185

sqrt(mean((q-q2)^2))

[1] 73.21752

# Nowhere close

# One thing to note is the RMSE of 6.51 is very low

# The KRW03-08 curve had RMSE = 16.46

# The SFM06-08 curve had RMSE = 34.70

# On this basis I’d say the FTR 03-05 error is no greater than the SFM06-08 error of 5%

# Nevertheless, maybe we can get an upper bound error by using fit4

# I wouldn’t trust the usual adjustment method though (based on RMSE)

m1 <- cbind(log(stg),log(stg)^2,log(stg)^3, log(stg)^4)

HY03

m2 <- cbind(log(ftr03.flo$stg), log(ftr03.flo$stg)^2, log(ftr03.flo$stg)^3, log(ftr03.flo$stg)^4)

weighthalfwidth(fit4, m1, m2, ftr03.flo$q)

[1] 0.448743

avg.err(0.448743)

[1] 0.4639560

ABORT

TRY USING A TWO-PART MODEL FOR LOG(Q)

plot(stg,q)

x0 <- seq(0,6.6,0.2)

y0 <- qfunftr05(x0)

lines(x0,y0,col=1)

# Develop model for log(q)

split=1.5

overlap=0.25

fit1 <- lm(log(q) ~ log(stg), subset=stg < split+overlap)

fit2 <- lm(log(q) ~ log(stg) + I(log(stg)^2) + I(log(stg)^3) , subset=stg >= split-overlap)

x1 <- x0[x0<=split+overlap]

x2 <- x0[x0>split-overlap]

y1 <- predict(fit1,newdata=data.frame(stg=x1))

y2 <- predict(fit2,newdata=data.frame(stg=x2))

lines(x1,exp(y1),col=2)

lines(x2,exp(y2),col=3)

# Now the log model is reasonably similar to the original

# Find the intersection of the two log models

> fit1

Call:

lm(formula = log(q) ~ log(stg), subset = stg < split + overlap)

Coefficients:

(Intercept)     log(stg)

1.144        4.261

> fit2

Call:

lm(formula = log(q) ~ log(stg) + I(log(stg)^2) + I(log(stg)^3),     subset = stg >= split - overlap)

Coefficients:

(Intercept)       log(stg)  I(log(stg)^2)  I(log(stg)^3)

0.1861         8.4008        -3.8997         0.7264

uniroot(

f = function(x) 1.144 + 4.261*log(x)

– (0.1861 + 8.4008*log(x) – 3.8997*log(x)^2 + 0.7264*log(x)^3),

interval = c(1.25,1.75))$root

[1] 1.383634

# RMSE

q1 <- qfunftr05(stg)

q2 <- numeric(length(q1))

q2[stg < 1.3836] <- exp(predict(fit1,newdata=data.frame(stg=stg[stg < 1.3836])))

q2[stg > 1.3836] <- exp(predict(fit2,newdata=data.frame(stg=stg[stg > 1.3836])))

sqrt(mean((q-q1)^2))

[1] 6.511185

sqrt(mean((q-q2)^2))

[1] 7.771463

x1 <- stg[stg <= split+overlap]

x2 <- stg[stg >  split-overlap]

m11 <- log(x1)

m12 <- cbind(log(x2),log(x2)^2, log(x2)^3)

# HY03

stg1 <- ftr03.flo$stg[ftr03.flo$stg < 1.3836]

q1 <- ftr03.flo$q[ftr03.flo$stg < 1.3836]

m21 <- log(stg1)

weighthalfwidth(fit1, m11, m21, q1)

[1] 0.3739777

stg2 <- ftr03.flo$stg[ftr03.flo$stg > 1.3836]

q2 <- ftr03.flo$q[ftr03.flo$stg > 1.3836]

m22 <- cbind(log(stg2),log(stg2)^2,log(stg2)^3)

weighthalfwidth(fit2, m12, m22, q2)

[1] 0.08465424

# Now find the weighted average of these two estimates

(0.3739777*sum(q1) + 0.08465424*sum(q2)) / (sum(q1)+sum(q2))

[1] 0.10383

avg.err(0.10383)

[1] 0.1040167

# Adjust for RMSE

0.1040167 * 6.51/7.77

[1] 0.08714913

# HY04

stg1 <- ftr04.flo$stg[ftr04.flo$stg < 1.3836]

q1 <- ftr04.flo$q[ftr04.flo$stg < 1.3836]

m21 <- log(stg1)

weighthalfwidth(fit1, m11, m21, q1)

[1] 0.3652471

stg2 <- ftr04.flo$stg[ftr04.flo$stg > 1.3836]

q2 <- ftr04.flo$q[ftr04.flo$stg > 1.3836]

m22 <- cbind(log(stg2),log(stg2)^2,log(stg2)^3)

weighthalfwidth(fit2, m12, m22, q2)

[1] 0.07331297

# Now find the weighted average of these two estimates

(0.3652471*sum(q1) + 0.07331297*sum(q2)) / (sum(q1)+sum(q2))

[1] 0.1191033

avg.err(0.1191033)

[1] 0.1193851

# Adjust for RMSE

0.1193851 * 6.51/7.77

[1] 0.1000254

# HY05

stg1 <- ftr05.flo$stg[ftr05.flo$stg < 1.3836]

q1 <- ftr05.flo$q[ftr05.flo$stg < 1.3836]

m21 <- log(stg1)

weighthalfwidth(fit1, m11, m21, q1)

[1] 0.3669315

stg2 <- ftr05.flo$stg[ftr05.flo$stg > 1.3836]

q2 <- ftr05.flo$q[ftr05.flo$stg > 1.3836]

m22 <- cbind(log(stg2),log(stg2)^2,log(stg2)^3)

weighthalfwidth(fit2, m12, m22, q2)

[1] 0.0681908

# Now find the weighted average of these two estimates

(0.3669315*sum(q1) + 0.0681908*sum(q2)) / (sum(q1)+sum(q2))

[1] 0.1141240

avg.err(0.1141240)

[1] 0.1143719

# Adjust for RMSE

0.1143719 * 6.51/7.77

[1] 0.0958251