SFM 2003 Annual load estimation

# These calcs are based on the following rating equation built from HY03-05 measurements:

q <- ifelse(s<2.4302, 1.2326*s^3.1572, 2.1831*s^2+28.325*s-61.386)

# Read in the data

sfm03.flo <- read.flo("sfm",03)

sfm03.lab <- read.lab("sfm",03)

sfm03.sed <- merge.flo("sfm",03)

# Eliminate the storm event data

# This requires .../work/hy03/storms03.csv

nonstorms <- drop.storms("sfm",03)

dim(nonstorms)

[1] 31416     4

head(nonstorms,5)

chr     q turb turbcode

1 (11/16/02 12:40:00) 2.244   NA        2

2 (11/16/02 12:50:00) 2.238   NA        2

3 (11/16/02 13:00:00) 2.227   NA        2

4 (11/16/02 13:10:00) 2.209   NA        2

5 (11/16/02 13:20:00) 2.203   NA        2

table(nonstorms$turbcode)

0     1     2     5     7

7486    78 18862  4573   417

# 18862 values of bad turbidity out of 31416 intervals to be estimated

# Extract rows with turbidity present, to be estimated using turb

trb03.flo <- nonstorms[!is.na(nonstorms$turb), ]

# Extract rows with turbidity missing, to be estimated using flow

flo03.flo <- nonstorms[is.na(nonstorms$turb), ]

# Note we are tricking R into thinking “trb” and “flo” are gaging stations

dim(trb03.flo)

[1] 12554     3

dim(flo03.flo)

[1] 18862     3

# Verify that we got all the data in nonstorms

12554+18862

[1] 31416

# Develop model for good turb data

range(trb03.flo$turb)

[1]   2 720

table(sfm03.sed$labcode2)

0   1   3   5   6   8

280  52   1  16   1   2

# Eliminate lab codes 5-8, missing turbidity, and turbidity > 750

trb03.sed <- sfm03.sed[sfm03.sed$labcode2 < 4 & !is.na(sfm03.sed$turb) & sfm03.sed$turb < 750, ]

table(trb03.sed$turbcode)

0  1  5  7

67  2 25 65

turbsscplot("trb",03)

Coefficients:

(Intercept)         turb

-47.761        2.546

# Trouble is this will predict negative SSC for turb < 19

sum(trb03.flo$turb < 19)

[1] 6121

# There are 6121 values of turbidity < 19

# Here's another way to see this

tmp <- turbsrc("trb",03,020801,0000,030731,2400,var=F)

sum(tmp$predssc < 0)

[1] 6121

turbsscplot("trb",03,type="power")

# Apply the model.  Have to turn variance off for this much data.

turb1 <- turbsrc("trb",03,020801,0000,030731,2400,type="power",var=F)

total(turb1)

Warning: Objects are non-sequential: check for gaps and overlaps

start            end  type surr    yhat   n    r2       s cv

turb1 01/27/03 12:50 06/18/03 16:40 power turb 1248293 159 0.887 128.201 NA

Total load = 1248293 kg

# Note ttsplot() does not make sense for non-continuous data

# Compare with some other models that won't give negative SSC

turb2 <- turbsrc("trb",03,020801,0000,030731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",03,020801,0000,030731,2400,type="sqrt",var=F)

total(turb1,turb2,turb3)

start            end       type surr    yhat   n    r2       s cv

turb1 01/27/03 12:50 06/18/03 16:40      power turb 1248293 159 0.887 128.201 NA

turb2 01/27/03 12:50 06/18/03 16:40 logxy:mvue turb 1289447 159 0.850   0.421 NA

turb3 01/27/03 12:50 06/18/03 16:40  sqrt:duan turb 1305462 159 0.882   2.980 NA

# Only the r squares are comparable, since they each use different transformations

=======================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo03.flo$q)

[1]   1.733 297.400

> range(sfm03.sed$q)

[1]   11.28 1445.00

# Eliminate lab codes > 4 and high discharges that could tweak the regression

flo03.sed <- sfm03.sed[sfm03.sed$labcode2 < 4 & sfm03.sed$q <= 300, ]

qsscplot("flo",03)

flow1 <- flowsrc("flo",03,020801,0000,030731,2400,var=F)

total(flow1)

Warning: Objects are non-sequential: check for gaps and overlaps

start            end       type surr    yhat   n    r2     s cv

flow1 11/16/02 12:40 07/31/03 23:50 logxy:mvue flow 1382726 235 0.488 0.734 NA

Total load = 1382726 kg

# Linear model for comparison

flow2 <- flowsrc("flo",03,020801,0000,030731,2400,type="linear",var=F)

total(flow2)

Warning: Objects are non-sequential: check for gaps and overlaps

start            end   type surr    yhat   n    r2       s cv

flow2 11/16/02 12:40 07/31/03 23:50 linear flow 1385391 235 0.189 334.391 NA

Total load = 1385391 kg

sum(flow2$predssc < 0)

[1] 11059	# Oops 11059 SSC predictions are negative; don't use this model

# Compare with some other models that won't give negative SSC

flow3 <- flowsrc("flo",03,020801,0000,030731,2400,type="power",var=F)

flow4 <- flowsrc("flo",03,020801,0000,030731,2400,type="sqrt",var=F)

total(flow1,flow3,flow4)

start            end       type surr    yhat   n    r2       s cv

flow1 11/16/02 12:40 07/31/03 23:50 logxy:mvue flow 1382726 235 0.488   0.734 NA

flow3 11/16/02 12:40 07/31/03 23:50      power flow 1387286 235 0.190 334.152 NA

flow4 11/16/02 12:40 07/31/03 23:50  sqrt:duan flow 1532351 235 0.353   6.333 NA

# Only the r squares are comparable, since they each use different transformations

=======================================================================

# Total non-storm load = 1248293 by turbidity + 1382726 by flow

1248293 + 1382726 = 2631019 kg = 2631 metric tons

# Total storm load = 25533 metric tons

25533 + 2631 = 28164 metric tons

# Storm loads are 25533/28164 = 90.7% of annual load