FTR 2003 Annual load estimation

# Read in the data

ftr03.flo <- read.flo("ftr",03)

ftr03.lab <- read.lab("ftr",03)

ftr03.sed <- merge.flo("ftr",03)

# Eliminate the storm event data

# This requires .../work/hy03/storms03.csv

nonstorms <- drop.storms("ftr",03)

dim(nonstorms)

[1] 30703     4

head(nonstorms,5)

chr      q turb turbcode

1 (11/20/02 20:10:00) 0.1440    0        0

2 (11/20/02 20:20:00) 0.1440    0        5

3 (11/20/02 20:30:00) 0.1440    0        5

4 (11/20/02 20:40:00) 0.1454    0        5

5 (11/20/02 20:50:00) 0.1440    0        5

table(nonstorms$turbcode)

0     2     4     5     7

28428    84    39  2145     7

# 84 values of bad turbidity out of 31124 intervals to be estimated

# Extract rows with turbidity present, to be estimated using turb

trb03.flo <- nonstorms[!is.na(nonstorms$turb), ]

# Extract rows with turbidity missing, to be estimated using flow

flo03.flo <- nonstorms[is.na(nonstorms$turb), ]

# Note we are tricking R into thinking “trb” and “flo” are gaging stations

dim(trb03.flo)

[1] 30619     4

dim(flo03.flo)

[1] 84  4

# Verify that we got all the data in nonstorms

30619+84

[1] 30703

# Develop model for good turb data

range(trb03.flo$turb)

[1]   0 342

range(ftr03.sed$turb,na.rm=T)

[1]   1 819

table(ftr03.sed$labcode2)

0   1   8

269  16   1

# Eliminate samples beyond range of prediction

trb03.sed <- ftr03.sed[!is.na(ftr03.sed$turb) & ftr03.sed$turb < 350, ]

table(trb03.sed$turbcode)

0   4   5   6

206   2  15   1

turbsscplot("trb",03)

# Trouble is this will predict negative SSC for turb less than about 25

# Need to also eliminate outlier at bottom

trb03.sed <- trb03.sed[!(trb03.sed$turb<20 & trb03.sed$ssc > 100), ]

turbsscplot("trb",03,type="power")

turbsscplot("trb",03,type="logxy")

# The log-log model is a poor fit

turb1 <- turbsrc("trb",03,020801,0000,030731,2400,type="power",var=F)

total(turb1)

start            end  type surr     yhat   n    r2      s cv

turb1 11/20/02 20:10 07/31/03 10:20 power turb 454036.5 223 0.936 41.345 NA

Total load = 454037 kg  Good

=======================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo03.flo$q)

[1] 18.37 41.61      # 41.61 is 1.18 cms

> range(ftr03.sed$q)

[1]   1.484 995.800

qsscplot("ftr",03)

# Eliminate high and low discharges that could tweak the regression

flo03.sed <- ftr03.sed[ftr03.sed$q > 8.7, ]

qsscplot("flo",03)

# Next eliminate the high end

flo03.sed <- ftr03.sed[ftr03.sed$q > 8.7 & ftr03.sed$q < 100, ]

qsscplot("flo",03)

flow1 <- flowsrc("flo",03,020801,0000,030731,2400)

flow2 <- flowsrc("flo",03,020801,0000,030731,2400,type="power")

flow3 <- flowsrc("flo",03,020801,0000,030731,2400,type="linear")

flow4 <- flowsrc("flo",03,020801,0000,030731,2400,type="loess")

total(flow1,flow2,flow3,flow4)

start            end       type surr     yhat  n    r2      s     cv

flow1 12/18/02 19:50 12/19/02 09:40 logxy:mvue flow 1239.102 79 0.178  0.633  8.061

flow2 12/18/02 19:50 12/19/02 09:40      power flow 1234.032 79 0.162 28.471 11.010

flow3 12/18/02 19:50 12/19/02 09:40     linear flow 1204.151 79 0.158 28.550 11.030

flow4 12/18/02 19:50 12/19/02 09:40      loess flow 1203.730 79 0.164 28.578     NA

# Very little difference between these estimates and the load is miniscule

total(flow1)

start            end       type surr     yhat  n    r2     s    cv

flow1 12/18/02 19:50 12/19/02 09:40 logxy:mvue flow 1239.102 79 0.178 0.633 8.061

Total load = 1239 kg,  CV = 8.06 %  Good

=======================================================================

# Total non-storm load = 454037 by turbidity + 1111 by flow

454037 + 1239 = 455276 kg = 455 metric tons

# Total storm load = 10296399 kg

10296 + 455 = 10751 metric tons

# Storm loads are 10296 / 10751 = 95.8% of annual load