Data corrections KRW07 - Jack Lewis

Lab data corrections

To match electronic data:

Dump 10, bottle 1 was renumbered as dump 9, bottle 20

Dump 10, bottle numbers 2-17 were decremented by one

Observer record corrections – Jack Lewis

I double-checked all ORs indicating errors of 0.04 or more.

All had electronic stage written down so I could detect time errors.

Only the following OR had a time error:

2007,03,16,1420,15,2.17

was changed to

2007,03,15,1420,15,2.17

estage on FF was 2.1826, matching the same time on Mar 15.

Rating equation –

Stage corrections - Jack Lewis

Start of year until index change at 21Oct2006:1210 required constant shift of +0.04.

There were no other index changes.

Interpolated 3 small gaps in Oct and Nov

plot.or("krw",07,sdate=060916,edate=070330,linear=T,cor=T)  # after constant shift

plot.or("krw",07,sdate=060916,edate=070330,x="stage",linear=T,cor=T)  # after the shift

Either a weak time or stage trend is plausible. Both are significant at p=0.008, but not convincing enough of a pattern to apply any blanket corrections.

The Dec 26 OR indicated an error of -0.046 in the dark and stage was changing rapidly. I'm choosing to ignore it.

The OR of 23Feb07:1720 shows an error or -0.078.  No explanation for this anomaly unless the stage was simply written down wrong (e.g. 6.87 instead of 6.97).

I'm not sure what to do with this data.  Most (13 of 17) of the ORs indicate small errors (less than 0.02) within the measurement error of a staff plate reading.  There is not much of a pattern in the 4 larger errors.  The 2 largest suggest that there may be a strong negative bias at high stages, but with only 2 points I wouldn't want to apply a large shift.

No further corrections made.

Later: dump 17 found and added to data set.  Now last 4 ORs have diffs of -.013,-.029,-.015, and -.027.  Average is -.021.  Shifted all stage from 15Mar to 22Apr by the average, -.021.

Turbidity corrections - Jack Lewis

At the time of these corrections I still don't have any SSC data,

so I am unable to use scatterplots as a correction tool.  However,

there aren't really any situations in this data set where these are needed.

Except for sunspikes, the turbidity was a fairly constant 5 FNU from the start of the year until 13Nov2006:1700 when it dropped to 0 where it remained until 21Nov2006:1110. I'm assuming there was an offset change at that time, and the correct turbidity was 0 from 16Sep2006 to 21Nov2006. Made it so.

21Nov2006:1120 to 22Nov2006:1440 coded bad turb as missing

Removed many sunspikes wherever they occurred by interpolating lengthy sections, generally from midnight to midnight.

Also removed obvious fouling spikes as usual.