SFM04 Event Loads

These loads were calc’ed using the following rating equation, built from HY03-05 measurements:

q <- ifelse(s<2.4302, 1.2326*s^3.1572, 2.1831*s^2+28.325*s-61.386)

SFM,401,031212,1600,031213,2110   65409 kg; very good

SFM,402,031213,2120,031217,0000  614703 kg; poor/fair

SFM,403,031228,2010,031231,1430 1024124 kg; excellent

SFM,404,031231,1440,040105,0000 1001197 kg; excellent

SFM,405,040216,1010,040217,2320 1897091 kg; good

SFM,406,040217,2330,040220,1200 1079483 kg; good

SFM,407,040225,0220,040229,0000  730552 kg; excellent

-----------------------------------------------------

SUM of events                   6412559 kg; very good

-----------------------------------------------------

sfm04.flo <- read.flo("sfm",04)

sfm04.lab <- read.lab("sfm",04)

mismatches("sfm",04)

# Many unmatched samples in flo file; none in lab file

sfm04.sed <- merge.flo("sfm",04)

Storm 401: dump 5:  SFM,401,031212,1600,031213,2110

turbsscplot("sfm",04,031212,1600,031213,2110)

Only 4 samples with good turbidity - not very useful

Actually, upon comparison of turbidity with SSC,

I'd have to conclude the turbidity is NOT good.

qsscplot("sfm",04,031212,1600,031213,2110)

But this is a good candidate for linear time. Use the SSC from sample 26 at 031212,1520 for starting value.

We don't have a good ending value, but can use a rating curve after bottle 48, based on 42:48?

qsscplot("sfm",04,dump=5,bot=42:48)

time1 <- lineartime("sfm",04,031212,1600,031213,0750,ssc1=178)

flow1 <- flowsrc("sfm",04,031213,0800,031213,2110,dump=5,bot=42:48)

ttsplot("sfm",04,time1,flow1)

It's odd that the SSC starts so high.  The pattern in bots 25:28

suggests there was a significant erosion event late on the recession

of the previous small event.

total(time1,flow1)

start            end       type surr      yhat  n    r2   s     cv

time1 12/12/03 16:00 12/13/03 07:50     linear time 62420.586 22    NA  NA     NA

flow1 12/13/03 08:00 12/13/03 21:10 logxy:mvue flow  2988.689  7 0.846 0.1 23.757

Total load = 65409 kg: quality very good

Storm 402: SFM,402,031213,2120,031217,0000

Unfortunately we have no samples whatsover and the turbidity record was very bad and had to be reconstructed

speculatively from sporadic data points that looked reasonable.  The only dump in HY04 that includes high enough

turbidity is dump 12, so try that first.  Fortunately there doesn't seem to be a great deal of variation from dump

to dump this year.

During dump 5 the turbidity looked much like it did during this event and the scatterplot below indicates that the

reconstructed turbidity in dump 5 may still be too high.  Indeed it doesn't make sense to have turbidity above 300

when the SSC is less than 50.

turbsscplot("sfm",04,type="power",dump=3:12,txt="dump")

turb1 <- turbsrc("sfm",04,031213,2120,031217,0000,dump=3:12,type="power")

ttsplot("sfm",04,turb1)

The total of 614703 compares to 498972 if just dump 12 is used and 623759 if all dumps are used. So the range of answers using a turbidity rating curve is about 20% of the estimate. The situation could be worse.

total(turb1)

start     end  type surr     yhat   n    r2       s    cv

turb1 12/13/03 21:20 2/17/03 power turb 566145.7 136 0.942 124.714 2.355

Total load = 614703 kg,  CV = 2.38 %  poor to fair (no samples, poor turbidity, cannot trust CV)

I would have called the quality poor, but for the following alternate estimate based on a discharge/sediment rating curve from the next event (dump 7)

The two estimates are only 15% apart, and the turbidity estimate is unexpectedly the lower of the two.

flow1 <- flowsrc("sfm",04,031213,2120,031217,0000,dump=7,bias="qmle")

start     end       type surr     yhat  n    r2    s     cv

flow1 12/13/03 21:20 2/17/03 logxy:qmle flow 708449.9 24 0.625 0.69 18.249

Total load = 708450 kg,  CV = 18.25 %

Storm 403:  dump 7:  SFM,403,031228,2010,031231,1430

Very nice turbidity data and good sample coverage.

four()

turbsscplot("sfm",04,031228,2010,031231,1430)

turbsscplot("sfm",04,031228,2010,031231,1430,type="power")

turbsscplot("sfm",04,031228,2010,031231,1430,type="sqrt")

turbsscplot("sfm",04,031228,2010,031231,1430,type="logxy")

Linear relation looks OK if it doesn't predict negative SSC

turb1 <- turbsrc("sfm",04,031228,2010,031231,1430)

head(turb1$predssc,20)

[1] -7.0512822 -8.6304097 -7.0512822 -5.4721547 -5.4721547 -5.4721547 -5.4721547

[8] -2.3138997 -5.4721547 -2.3138997 -2.3138997  0.8443554  2.4234829  5.5817379

[15]  8.7399929  8.7399929 11.8982479 16.6356304 19.7938854 26.1103954

# The first 11 points are negative.

turb2 <- turbsrc("sfm",04,031228,2010,031231,1430,type="sqrt")

turb3 <- turbsrc("sfm",04,031228,2010,031231,1430,type="power")

turb4 <- turbsrc("sfm",04,031228,2010,031231,1430,type="logxy",bias="qmle")

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2      s    cv

turb1 12/28/03 20:10 12/31/03 14:30     linear turb 1005669 18 0.931 79.073 3.879

turb2 12/28/03 20:10 12/31/03 14:30  sqrt:duan turb 1006952 18 0.948  1.725    NA

turb3 12/28/03 20:10 12/31/03 14:30      power turb 1013158 18 0.930 79.852 3.667

turb4 12/28/03 20:10 12/31/03 14:30 logxy:qmle turb 1024124 18 0.962  0.173 4.719

# All give similar results; Looking at the ttsplots, the logxy fits the recession best.

ttsplot("sfm",04,turb4)

total(turb4)

start            end       type surr    yhat  n    r2     s    cv

turb4 12/28/03 20:10 12/31/03 14:30 logxy:qmle turb 1024124 18 0.962 0.173 4.719

Total load = 1024124 kg,  CV = 4.72 %  Quality Exellent

Storm 404: dumps 7-9: SFM,404,031231,1440,040105,0000

Excellent turbidity data and sample coverage

par(mfrow=c(2,1))

turbsscplot("sfm",04,031231,1440,040105,0000,type="power")

turbsscplot("sfm",04,031231,1440,040105,0000,type="logxy")

turb1 <- turbsrc("sfm",04,031231,1440,040105,0000,type="power")

turb2 <- turbsrc("sfm",04,031231,1440,040105,0000,type="logxy",bias="qmle")

total(turb1,turb2)

start     end       type surr    yhat  n    r2      s    cv

turb1 12/31/03 14:40 1/05/04      power turb 1001197 15 0.986 51.397 3.229

turb2 12/31/03 14:40 1/05/04 logxy:qmle turb 1031079 15 0.992  0.112 3.646

# The power model nails the peak better

ttsplot(“sfm”,04,turb1)

total(turb1)

start     end  type surr    yhat  n    r2      s    cv

turb1 12/31/03 14:40 1/05/04 power turb 1001197 15 0.986 51.397 3.229

Total load = 1001197 kg,  CV = 3.23 %  Quality excellent

Storm 405: dump 12 - SFM,405,040216,1010,040217,2320

Excellent turbidity data but we missed a few samples on and after the peak

turbsscplot("sfm",04,040216,1010,040217,2320)

turbsscplot("sfm",04,040216,1010,040217,2320,type="power")

turbsscplot("sfm",04,040216,1010,040217,2320,type="logxy")

turbsscplot("sfm",04,040216,1010,040217,2320,type="sqrt")

# None fit the upper 4 points well

turbsscplot("sfm",04,040216,1010,040217,2320,type="loess")

turbsscplot("sfm",04,040216,1010,040217,2320,type="loess",degree=2)

# The second degree loess does it best

turb1 <- turbsrc("sfm",04,040216,1010,040217,2320,type="loess",degree=2)

ttsplot("sfm",04,turb1)

total(turb1)

start            end  type surr    yhat  n    r2       s cv

turb1 02/16/04 10:10 02/17/04 23:20 loess turb 1897091 19 0.949 187.724 NA

# By comparison a power model gives 1723851 with CV=5.38%

Total load = 1897091 kg  Good (would be better if the last 18 hrs had samples)

Storm 406: dump 13: SFM,406,040217,2330,040220,1200

No samples on first turbidity pulse (was out of bottles)

The first couple hours and 16 hrs of the recession were reconstructed.

turbsscplot("sfm",04,040217,2330,040220,1200)

They don't get any better than that!  We have to assume the relationship also holds for the

first turbidity pulse

turb1 <- turbsrc("sfm",04,040217,2330,040220,1200)

ttsplot("sfm",04,turb1)

total(turb1)

start            end   type surr    yhat n    r2   s    cv

turb1 02/17/04 23:30 02/20/04 12:00 linear turb 1079483 7 0.999 8.3 0.673

Total load = 1079483 kg,  CV = 0.67 %   Quality good

Storm 407 - dump 14: SFM,407,040225,0220,040229,0000

Excellent turbidity and good sample coverage

turbsscplot("sfm",04,040225,0220,040229,0000,type="power")

This is a great relation, but could be improved by splitting out

samples 1-5, which are all on the first rising limb

turbsscplot("sfm",04,dump=14,bot=1:5,type="loess")

turbsscplot("sfm",04,dump=14,bot=6:16,type="power")

turb0 <- turbsrc("sfm",04,040225,0220,040229,0000,type="power")

turb1 <- turbsrc("sfm",04,040225,0220,040225,2100,type="loess")

turb2 <- turbsrc("sfm",04,040225,2110,040229,0000,type="power")

total(turb0)

start     end  type surr     yhat  n    r2      s    cv

turb0 02/25/04 02:20 2/29/04 power turb 709134.3 16 0.979 29.993 2.337

Total load = 709134 kg,  CV = 2.34 %

total(turb1,turb2)

start            end  type surr      yhat  n    r2      s    cv

turb1 02/25/04 02:20 02/25/04 21:00 loess turb  22740.47  5 0.997  8.608    NA

turb2 02/25/04 21:10 02/29/04 00:00 power turb 712539.42 11 0.994 14.885 1.332

Total load = 735280 kg

# These two estimates only disagree by 3.7%

# Substituting a power model for the loess model yields 734793 (CV=1.30%)

ttsplot("sfm",04,turb1,turb2)

There is a discontinuity because the regression lines diverge.  We can get a better

transition if include bots 1-7 in the first model

par(mfrow=c(1,2))

turbsscplot("sfm",04,dump=14,bot=1:7,type="loess",degree=2)

turbsscplot("sfm",04,dump=14,bot=7:16,type="power")

# Make the transition at bottle 7 040226,0940

turb1 <- turbsrc("sfm",04,040225,0220,040226,0940,dump=14,bot=1:7,type="loess",degree=2)

turb2 <- turbsrc("sfm",04,040226,0950,040229,0000,dump=14,bot=7:16,type="power")

ttsplot("sfm",04,turb1,turb2)

total(turb1,turb2)

start            end  type surr     yhat  n    r2      s    cv

turb1 02/25/04 02:20 02/26/04 09:40 loess turb 174827.9  7 0.970 35.519    NA

turb2 02/26/04 09:50 02/29/04 00:00 power turb 555723.8 10 0.998  8.528 0.859

Total load = 730552 kg  Quality excellent